Agenda item: 	10.6

Source: 	Lenovo

Title: 	[FS_5G_RTP_Ph2] Views on AL-FEC support in Rel-19

Document for	Discussion and Agreement

# Introduction

KI#3 (Enhancements for application-layer FEC support) and KI#4 (AL-FEC awareness for PDU Set handling) have been dedicated for the study of 5GS support of AL-FEC in Rel-19 for RTP-based media delivery (FS_5G_RTP_Ph2), [1]. The primary focus was on low-latency, interactive and immersive RTC, such as split-rendering and/or mobile gaming.

# Background and Analysis

SA2 decided in TR 23.700-70 [2] to support AL-FEC optimized handling in Rel-19 for XRM_Ph2 based on existent PDU Set concept and static content ratio. The following conclusions are part of TR 23.700-70:

### TR 23.700-70: 8.1.4	Conclusion for KI#1_AL-FEC

Normative work for AL-FEC awareness at NG-RAN will be based on the following principles:

-	When requesting or updating QoS for a DL flow, an AF may provide to PCF/NEF - together with PDU Set QoS parameters - the content ratio for the flow. An AF may provide either PSIHI or content ratio for a flow.

-	If a PCF receives the content ratio from an AF, then the PCF may include the content ratio in PCC rules that it provides to the SMF.

-	The SMF provides content ratio to NG-RAN when establishing/modifying a QoS flow.

-	If NG-RAN supports PDU Set content ratio awareness and has received content ratio information for a QoS flow, then NG-RAN may consider content ratio information as follows:

-	If NG-RAN needs to discard PDUs of the QoS flow during congestion, then NG-RAN may use the content ratio information for discarding.

NOTE 1:	The principles concluded for AL-FEC awareness at RAN (as above) is subject to RAN-level enhancements of RLC Acknowledged Mode (RLC AM) for XR as per RP-240791 and further feasibility analyses and feedback from RAN WG2. RLC UM is not intended to be supported in this release due to concerns shared so far from RAN WG2 (see R2-2405781), pending further status update from RAN WG2.

NOTE 2:	RAN can, e.g. primarily discard obsolete AL-FEC PDUs of each PDU Set.

NOTE 3:	There is no difference in terms of how many PDUs NG-RAN will be discarded for a given QoS flow. In other words, NG-RAN will not discard more PDUs if content ratio information is provided for a QoS flow compared to when no content ratio information is provided for the same QoS flow. This scheme merely allows NG-RAN to select the PDUs to be discarded to ensure that the original content can still be recovered at the receiver.

NOTE 4:	Since there is no difference how many PDUs will be discarded, there are no impacts to existing congestion control algorithms.

Solutions to KI#3 and KI#4 (i.e., Sol#5, Sol#8, Sol#10 Sol#17, Sol#18, Sol#19) provided inputs and feedback to SA2 on the topic of AL-FEC. SA4 feedback seems to have been considered to some extent in Stage-2 design decisions. SA2 design principle tries to account to issues raised by SA4 regarding AL-FEC discarding impact on applications and congestion control mechanisms. To this extent, PDU discarding of AL-FEC protected PDU Sets is limited only to DL congestion when discarding would be anyway inevitable.

Based on the above architecture SA2 aims for RAN to simply use PDU Set content ratio (including the awareness that a PDU Set is protected by AL-FEC) not to increase/decrease net PDU discard volume during congestion. Instead, the AL-FEC content ratio may be used by RAN implementations to “smartly” discard PDUs such that AL-FEC PDU Sets are still recoverable at the receiver.

A formal quantitative analysis of net E2E benefits for support of AL-FEC based on the above principles has not been captured so far in 3GPP. Sol#17 discusses however some limited qualitative aspects and provides recommendations regarding operation points and preferred content ratio dynamicity for optimized available bandwidth utilization with AL-FEC enabled.

SA2 architectural design assumptions on RAN operation are yet to be checked and confirmed by RAN2 based on LS (to: RAN2, cc: SA4), [3], for both RLC AM and RLC UM modes.

The content ratio design would maximally benefit from MDS AL-FEC codes, such as those described for (S)RTP media delivery in Sol#5. MDS codes guarantee recoverability (at least asymptotically) as long as at least any number of PDUs of a PDU Set larger or equal than the content ratio of the PDU Set is successfully delivered. Given the content ratio is static or semi-statically controlled via control plane signalling, the PDU Set enhancements to AL-FEC support seem to be reduced from the normative perspective of 5G_RTP_Ph2, since the RAN only needs be aware which PDU Sets are protected by AL-FEC on a QoS flow. However, other SA4 RTC related specifications may be impacted (e.g., TS 26.510, TS 26.113).

# Way forward and conclusions

Based on the above information, a recommended way forward is to:

- Agree on supporting PDU Set handling with AL-FEC awareness in Rel-19 5G_RTP_Ph2 normative work.

NOTE 1: The agreement is conditioned by RAN confirmation to feasibility of using content ratio information for discarding DL PDUs during congestion for RLC AM mode based on the above SA2 principles in Rel-19.

- Specify any necessary (S)RTP enhancements for PDU Set marking with AL-FEC awareness.

NOTE 2: To realize Stage-3 aspects of the agreed SA2 design over 5G-RTC other impacted technical specifications are not precluded (e.g., TS 26.510, TS 26.113).

- Specify requirements and guidelines for MDS AL-FEC coding schemes necessary for PDU Set handling with AL-FEC awareness by the 5GS.

# 4	 Proposal

Discuss and agree on the way forward and conclusions as of Clause 3. If agreeable, capture conclusions in the corresponding TR 26.822 Clause 8.

# References

[1] TR 26.822 v1.0.0, Study of 5G Real-time Transport Protocol Configurations, Phase 2.

[2] TR 23.700-70 v19.0.0, Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2.

[3] LS in S4-241811/ S2-2410999, Reply LS on Application-Layer FEC Awareness at RAN (response to: Reply LS on Application-Layer FEC Awareness at RAN (R2-2405781/S2-2409567)), TSG SA4#130, Orlando, Status: Not yet treated.