3GPP TSG-SA WG4 Meeting #130	S4-242100

# USA, Orlando, 18 – 22 November 2024 								          Revision of S4-241853 and of S4aR240094

Source:	InterDigital Europe

Title:	Haptics signals and media formats

Spec:	3GPP TR 26.854

Agenda item: 	10.8

Document for:	Agreement

1. Introduction

As per the minutes of the sept. 25th call, and further discussion on the mailing list this revision of S4aR240094 proposes a first text and information on Haptics signals and media formats in more details and take in account the adoption of pCR 77.

A JSON file is attached which is proposed to be included in annex A of the TR.

Note: for traceability, the multiples changes to the pCR since its first presentation have been kept.

2. Reason for Change

Provide initial text for Haptics signals and media formats to populate section 6, some references for section 2, acronyms for section 3 of TR 26.854.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[SMPTE]	SMPTE ST 2100-1:2017 “Definition and Representation of Haptic-Tactile Essence for Broadcast Production Applications”.

[AHAP]	Apple Haptic and Audio Pattern JSON format for Haptics https://developer.apple.com/documentation/corehaptics/representing_haptic_patterns_in_ahap_files, retrieved Sept. 2024.

[23090-31]	ISO/IEC 23090-31 Information technology — Coded representation of immersive media Part 31: Haptics coding

[HAPT]	xxx

[IVS]	https://www.immersion.com/technology/ retrieved Sept. 15th. 2024.

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

AI	Artificial Intelligence

AHAP	Apple Haptic and Audio Pattern

ERM	Eccentric Rotating Mass

HJIF	Haptic JSON Interchanged Format

HMD	Head Mounted Display

JSON	JavaScript Object Notation

LRA	Linear Resonant Actuators

MPEG	Motion Picture Expert Group

PCM	Pulse-Code Modulation

SMPTE	Society of Motion Picture Television Experts

RIFF	Ressource Interchange File Format

WAV	Waveform Audio File format

XML	Extensible Markup Language

XR	Extended Reality

* * * Next Change * * * *

# 6	Haptics signals, media formats and device types

## 6.1	Introduction

Haptics refers to the sense of touch, and encompasses the generation, manipulation, and perception of tactile sensations, forces, and motions. In the media domain it relates to the capture and rendering of physical information about objects and environments due to interactions with the user. The capture is generally performed with sensors and the rendering with hardware actuators allowing to render different modalities.

In general, three types of modalities are considered, tactile (vibration, temperature, pression), kinaesthetic (force) and proprioception (motion, acceleration). Those modalities are rendered in the human body through different mechanoreceptors. The density and properties of those receptors are different depending on the modality and the body parts. In addition, each individual has different sensibility, which can be managed through users’ profiles and adaptation.

Finally, the rendering is performed with specific hardware actuators embedded into user devices. The different Haptic modalities are rendered with different actuators. Typically, tactile sensations can be rendered with vibrotactile (e.g. linear resonant actuators (LRA), eccentric rotating mass (ERM), temperature, wind (fan), force … devices.

Haptics signals correspond to the raw haptics data at the output of the sensors or authoring tools, described in section 6.2

Haptics media formats are the formats representing the haptics signals once processed for use by a production framework, as described in section 6.3. For example, the raw synthetic signal described in 6.2.3 can be transformed in any of the haptics media parametric representation formats described in 6.3.4

## 6.2	Haptics signals

### 6.2.1	Introduction

Haptic  signals are essential in capturing and transmitting tactile sensations in a variety of applications, ranging from simple vibrations in mobile devices to complex force feedback in virtual reality systems. Two types of signals exist, time-sampled signals typically captured by sensors, and synthetic signals of haptic effects, typically created from authoring tools.

### 6.2.2		Time-sampled signals

In Figure 6.2-1, two examples of time-sampled signals are provided. The left side shows the samples of 3D angular orientations and linear accelerations of a horse captured with an accelerometer positioned on the horseman. The right side illustrates the haptic signal of a texture (here a carpet) captured from the sound of a pen moving on the surface of the carpet in one direction.

Those haptics time sampled  signals are generally used for signals captured from sensors.

Figure 6.2-1 - Example of time sampled signals (left: Horse riding 3D angular orientations and linear accelerations captured with an accelerometer, right: texture captured from sound on one direction).

Generally, the sampling frequency is set between 2kHz and 16kHz (8kHz in Figure 6.2-1) with the haptic perceptual bandwidth being around [5Hz-1kHz], with some variations depending on the haptic modality, rendering device and human haptic receptor. 5Hz and below is particularly significant for force feedback. This leads to a raw data-rate average of 128 kbps per channel.

### 6.2.3		Synthetic signals

Synthetic signals of haptic effects are  mostly generated with design tools and thus correspond to synthetic data. In Figure 6.2-2, two examples of synthetic signals are provided, on the left a heartbeat synthetic signal (thus the regularity of the signal), and on the right a synthetic rain effect (quasi random design) with 4 channels corresponding to 4 different target locations on the user.

Figure 6.2-2 - Example of parametric rendered haptic (left: heart beat synthetic signal, right: synthetic rain effect with 4 channels for 4 different locations on the user).

The synthetic aspect of those signals compared to time-sampled signals captured from sensor is obvious. In general, a synthetic signal is used to synthesize a short duration effect and is used only at some events or at dedicated point in time during a gaming or movie experience, by opposition to time-sampled signals that represent a continuous signal sampled at a constant frequency

Synthetic signals need a synthesizer to be converted to a timed signal compatible with the actuators used. The raw bit-rate dependents on the number of effects and their distribution in time.

### 6.2.4		Comparison and applications

The choice between time-sampled signals and synthetic signals depends on the application and the desired level of control. Time-sampled signals are more suitable for applications requiring high fidelity and real-time capture of physical interactions, such as in medical simulations, playback of recorded signals (motion) or fine-grained tactile feedback systems.

Synthetic signals, on the other hand, are ideal for applications where flexibility and ease of use are paramount, such as in mobile notifications, communication, movies or gaming, where the haptic feedback needs to be easily adjustable and consistent across a variety of devices. Synthetic signals are also well suited for XR applications, where haptics effect may be attached to events or objects.

* * * Next Change * * * * All new

# 6.3 	Haptic media formats

## 6.3.1 	Introduction

In this clause, haptics media formats are the formats representing the raw data (or haptics signals described in 6.2) and used at the production side (e.g. for encoding, packaging and delivery). Most commonmedia formats for haptics are proprietary. However standardised interchangeable media formats (or mezzanine formats) have now been defined.

## 6.3.2		PCM format

PCM or Pulse Code Modulation is a digital format that represents analog signals in digital format as a series of samples at a regular sampling frequency. It has been defined for audio but can be used to store haptic signals from sensors.

Various file formats support PCM such as the audio WAV format. ISO/IEC MPEG Haptic [23090-31] is using WAV as one of the input file format for time-sampled haptic signals. This type of file format uses different bit depth and data types to store the signal usually interpreted on a [-1 ; 1] range. For this reason, additional headers and metadata is required to specify the actual value ranges, gain or other characteristics of the signal essential for rendering. There are currently no standard defining this type of metadata and therefore the interpretation of the signal is often application dependent.

This PCM format is suitable for storing or distributing sensors data and its implementability has been demonstrated for audio signals. The drawback of the PCM format is its overhead as it is sampled continuously at constant frequency in time as opposed to parametric formats that are much sparser.

## 6.3.4		Proprietary parametric formats

Descriptive haptic is represented through parametric primitives. It can be described by an amplitude, a frequency, a duration, and a waveform shape such as sinusoid, or square. Two types of parametric effects are usually defined: transient and continuous. A transient effect is a short pulse with an amplitude, a position in time or space, and a reference frequency. A continuous effect is represented by a frequency and an amplitude information modulated on the timeline.

Existing proprietary descriptive formats use JSON or XML to describe haptic effects. Two of those proprietary formats are commercially deployed: AHAP from Apple [AHAP] and IVS [IVS] from immersion corp.

AHAP has been mainly developed for vibrotactile signals and implementation into iOS Core Haptic framework.

IVS was targeting media applications (movies, gaming) and communication (smartphone) using mainly vibrotactile signals described in XML format.  A binary version is available for networks transmission.

HAPT is a vendor-specific haptic coding format based on the RIFF format. It is designed to enable efficient coding of a device-specific haptic effects. [HAPT]

Editor’s note: need to find proper reference for HAPT

These formats may further be encoded and compressed using haptic codecs. The ISO/IEC MPEG Haptic [23090-31] codec supports both the AHAP and IVS formats, while the IEEE codec [IEEE] only supports PCM input signals.

## 6.3.5	Standardized and Interchangeable parametric formats

SMPTE defined the Haptic-Tactile Essence for Broadcast Production Applications [SMPTE ST 2100-1:2017]. However, it was mainly dedicated to position, orientation, velocity and acceleration.

[ Editor’s note: For information, not to be included into the Draft TR due to trademark usage: The Guitammer Company developed and implemented this format for broadcasting sports physical events. Proof of concepts have been done with NHRA and NHL sports.]

MPEG HJIF: MPEG defined in section 6 of specification [23090-31] a standardized and interchanged parametric JSON based format (or mezzanine format) for describing haptics effects called HJIF

This format is compatible with the proprietary AHAP format. Transcoding from and to the other is possible. Similarly, the IVS format can be transcoded from XML to JSON, for most of its functionalities, since similar parametric patterns/functions are used. Transcoding  of the primitives is lossless.

In addition, MPEG HJIF can integrate a binary encoded version of a PCM signal.

The main advantage of MPEG HJIF is its flexibility to support various haptic modalities, various parametric formats and its flexibility for heterogeneous end-user devices and experiences. Thanks to its JSON formatting, it is also easy to edit and modify the file.

MPEG HJIF can further be encoded and compressed using MPEG haptic codecs, in a binary format for networks transmission.

* * * End of Changes * * * *