3GPP TSG-SA WG4 Meeting #130	S4-242104 is revision of S4-242034

# USA, Orlando, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR | 002 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of reference to device capabilities in TS 26.565. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 2024/11/24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Under Table 8.4.2.2-1, device capabilities point to wrong reference clause of 6.2.5 of TS 26.119. TS 26.119 does not contain the clause 6.2.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected reference clause is proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misinterpretation of the reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.4.2.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

Table 8.4.2.2-1 Split Rendering Configuration Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| renderingFlags | Array(SR_CONFIG_FLAGS) | 0..1 | Provides a set of flags to activate/deactivate selected rendering functions. The defined SR_CONFIG_FLAGS are: - FLAG_ALPHA_BLENDING - FLAG_DEPTH_COMPOSITION - FLAG_EYE_GAZE_TRACKING |
| splitRenderingProfile | array(URI) | 0..1 | A list of supported split-rendering profile identifiers on the UE. The profile identifiers are listed in Annex C for each profile. |
| deviceCapabilities | Object | 0..1 | Device capabilities as defined in TS 26.119 [4], clause 6.1. |
| spaceConfiguration | Object | 0..1 | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |
| referenceSpaces | Array | 0..1 | An array of reference spaces and their identifiers. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| refSpace | enum | 1..1 | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE. |
| actionSpaces | Array | 0..1 | An array of action spaces that need to be defined by the split rendering client in the XR session. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| actionId | number | 1..1 | Provides the unique identifier of the action. |
| subactionPath | string | 1..1 | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |
| initialPose | Pose | 0..1 | Provides the initial pose of the new XR space’s origin. |
| viewConfiguration | Object | 0..1 | Conveys the view configuration that is configured for the XR session. |
| type | Enum | 1..1 | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |
| width | number | 1..1 | The recommended width of the swapchain image. |
| height | number | 1..1 | The recommended height of the swapchain image. |
| compositionLayer | string | 1..1 | An identifier of the selected composition layer. |
| minPoseInterval | number | 0..1 | The minimum time interval between two consecutive pose information instances sent to the network, in milliseconds. |
| fovs | Array | 0..1 | An array that provides a list of the field of views (FoV) associated with each view. |
| fov | Object | 1..n | Indicates the four sides of the field of view used for the projection of the corresponding XR view. The number of views n is determined by the type enum of the viewConfiguration. Both the viewPoses in the Pose Format and the fovs arrays shall be ordered in a consistent way (i.e., a same index can be used to retrieve the view pose and the related FoV information). |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |
| environmentBlendMode | enum | 1..1 | The type indicates the environment blend mode configuration. Defined values are OPAQUE, ADDITIVE and ALPHA_BLEND. Other values may be added. |
| actionConfiguration | Array | 0..1 | This contains a list of the actions that are to be defined by the SR client. |
| action | Object | 1..n | A definition of a single action object. |
| id | number | 1..1 | A unique identifier of the action. |
| actionType | enum | 1..1 | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |
| subactionPaths | string | 1..n | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |
| extraConfigurations | Object | 0..1 | A placeholder for addition configuration information. |

## End of change