3GPP TSG-SA WG4 Meeting #130						S4-242178

USA, Orlando, 18 – 22 November 2024

Source:	Fraunhofer HHI

Title:	[FS_AI4Media] pCR on formats and compression tools for AI/ML models

Spec:	3GPP TR 26.927

Agenda item:	9.6

Document for:	Agreement

1. Introduction

TR 26.927 documents MPEG’s Neural Network Coding (NNC) standard in clause 6.2.5.3 and reports on compression results published by MPEG in clause 6.7.2. Both, the description of NNC and the related coding results, are based on the first edition of NNC, which targeted compression of entire neural networks.

Meanwhile, MPEG has issued a second edition of NNC, which additionally supports the compression of incremental neural network updates, as e.g. required for federated learning or for federated transfer learning. To demonstrate the compression performance in such scenarios, MPEG has furthermore issued a white paper. Since federated learning is within the scope of TR 26.927, we propose to also document NNC’s capabilities for such scenarios.

2. Proposal

We propose

- to add a brief technical description of the new edition 2 features of NNC to the existing description of NNC in clause 6.2.5;

- to extend clause 6.7.2, which already comprises MPEG’s coding results, by the new results for federated learning from the MPEG white paper;

- and to update the reference to NNC to the latest edition and to add a reference to the new MPEG whitepaper.

The proposed changes are show below as revision marks.

* * * First Change * * * *

<Proposed changes in revision marks>

#### 6.2.5.3 Neural Network Coding (NNC) format

…

NNC also supports compression of incremental updates of neural networks, which can e.g. be applied for updating neural networks or in federated learning scenarios. For such distributed AI scenario, NNC also comprises dedicated high-level syntax that allows to identify updates, their order, and their dependencies.

* * * Second Change * * * *

<Additions to the end of the clause in revision marks>

### 6.7.2	MPEG NNC

### …

Table 6.7.2-2 shows MPEG’s coding results for the distribution of incremental AI/ML model updates that are reported in MPEG’s whitepaper [xx].  MPEG’s results are related to different models for image recognition. In the federated learning use cases, the model is trained from scratch. In the federated transfer learning use cases the model is re-trained for a new dataset.

In the MPEG framework, training is performed in multiple rounds using a server and 16 clients. Each training round consist of the following steps:

- Each client performs a local training step based on its local data and its local model. It computes a local model update, which is the difference of the local model before and after the local training step. Then, it sends the local model update to the server.

- The server averages all received local model updates and sends the averaged update to the clients.

- Each client adds the received averaged update to its local model as it was before the local training step.

This way, all clients start the next training round with the same local model.

The table shows MPEG’s results in an uncompressed communication scenario and a compressed communication scenario.

The uncompressed communication scenario is given for reference. The table shows the top-1 accuracy that is achieved after training with the number of training rounds that is also given in the table. Moreover, the table shows the accumulated number of GB that are transmitted for sending the local and averaged model updates without any kind of compression.

In the compressed communication scenario, local and averaged model updates are lossy compressed with NNC. The number of training rounds has been selected to approximately match the accuracy achieved in the uncompressed communication scenario. The table shows that MPEG reports a reduction of the transmitted data volume to 3% to 0.1% (of the uncompressed data volume) when using compression with NNC, while the accuracy is not reduced by more than 1%.

Table 6.7.2-2: Application of NNC in different federated learning use cases as reported by MPEG.

|  |  | Uncompressed communication |  |  | Compressed communication |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Datasets | Training Rounds | Top-1 accuracy in % | Transmitted data in GB | Training Rounds | Top-1 accuracy in % | Transmitted data in % of uncompressed |
| Federated Learning |  |  |  |  |  |  |  |
| ResNet56 | CIFAR-100 | 99 | 57.79 | 7.86 | 103 | 57.49 | 2.87 |
| ResNet50 | ImageNet-200 | 75 | 53.00 | 229.61 | 89 | 52.58 | 2.08 |
| MobileNetV2 | ImageNet-200 | 97 | 45.44 | 30.79 | 94 | 45.33 | 3.07 |
| Transfer-Federated Learning |  |  |  |  |  |  |  |
| ResNet18 | ImageNet-1k to Pascal VOC | 49 | 72.61 | 70.16 | 46 | 72.13 | 0.64 |
| ViT-B/16 | ImageNet-1k to Pascal VOC | 20 | 78.54 | 219.68 | 46 | 77.98 | 0.11 |

More detailed information on the different scenarios can be found in the documents referenced in [xx].

* * * Third Change * * * *

<Proposed changes in revision marks>

### 2. References

[b3]	"ISO/IEC 15938-17:2024 Information technology - Multimedia content description interface - Part 17: Compression of neural networks for multimedia content description and analysis”, Edition 2, ISO/IEC, January 2024.

[xx]	"White Paper on Neural Network Coding", MPEG document N23564, ISO/IEC JTC 1/SC 29/AG 03, January 2024.