3GPP TSG SA WG4 128	S4-240XXX

Jeju, South Korea, 20 - 24 May, 2024

Title:	[Draft] LS Reply to SA2 LS on FS_XRM Ph2

Response to:	LS (S2-2405625/S4-240874) from SA2

Release:	Release 19

Work Item:	FS_XRM Ph2, FS_5G_RTP_Ph2

Source:	SA WG4

To:	SA WG2,

Cc:	RAN WG2, RAN WG3

Contact Person:

Name:	Qi Pan

E-mail Address:	panqi8 DOT AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1. Overall Description:

SA4 thanks SA2 for the LS on FS_XRM Ph2 (S2-2405625/S4-240874) and would like to provide the feedback as following:

- Question1 [for SA4, RAN2 and RAN3]: PDU Set correlation information (Sol#23) provides the dependency relationship among PDU Sets. Does SA4, RAN2 and RAN3 see any improvement with adding inter-PDU set correlation information to assist RAN making PDU set discarding decision as comparing to the existing (R18) PDU Set information that is already provided by the AS?

- Reply 1: Release 18 TS 26.522 contains guidelines that assign PDU Set Importance implicitly taking the dependencies between PDU Sets into account. There is no consensus to add another solution besides the PDU Set importance in SA4. However, SA4 may further study this.

- Question2 [for SA4]: In Sol#29, PDU Set QoS or ordinary per packet based QoS (e.g. PER, PDB) can be applied for different media streams multiplexed in an IP flow, SA2 would like to ask SA4 whether a media stream (e.g. a video RTP stream) can include packet which is not related to PDU Set?

Reply 2: Yes, it may be possible. However, for a single media stream, i.e., a video stream, the common understanding from SA4 is each PDU should be marked if PDU Set marking is enabled for this video stream. SA4 is also further studying whether and how to handle the potential lonely PDU issue from media application perspective in FS_5G_RTP_Ph2.

- Question4 [for SA4 and RAN2]: In Sol#30, the PSA UPF may identify the size of incoming burst based on N6 protocol, and send it to NG-RAN to assist RAN scheduling.

  - To SA4: is it possible that the application server provides the burst size in the first packet of the burst via N6?

  - Reply 4: Yes, but not always. It is possible since there are cases where the application server can become aware of the data burst size before sending it out. However, it depends on the application implementations and there are cases where it may introduce additional latency. Therefore, it needs to be confirmed by the application layer that such feature is available, or even feasible given the application requirements. SA4 is studying the enhancement of data burst related traffic characteristics in FS_5G_RTP_Ph2.

- Question5 [for SA4]: Some of the solutions support only QUIC-based media delivery. Can SA4 provide feedback on choosing only solutions for PDU Set identification for encrypted traffic that only support QUIC as transport protocol?

Reply 5: It could be fine to use QUIC for a start as baseline. However, other protocols can be for further study. SA4 is studying QUIC as transport protocol for segmented media delivery. However, SA4 hasn't studied QUIC for Real Time Communication services yet. SA4 may provide additional information on QUIC if there is further study or normative work.

- 2. Actions:

- To SA2:

ACTION: SA4 kindly asks SA2 to take the above into account and provide feedback if any.

- 3. Date of Next SA4 Meetings:

<add telco dates that have power to send/receive LSs>

SA4#129-e	19th August – 23rd August 2024			Online

SA4#130	18th Nov – 22nd Nov 2024				Orlando, US