3GPP TSG-SA4 Meeting #128	S4-241270

# Jeju, Korean,  20. - 24. May, 2024											revision of S4-240920

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | - | rev | - | Current version: | 1.1.4 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS_Pro_Ph2]: Extending 5GMS with Oauth 2.0 support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 03.11.2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The OpenAPI definitions within TS 26.510 are extended for the usage of Oauth 2.0 (according to the SA3 guidelines) for 5GMS protocols based on the conclusions in TR 26.804, clause 6.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.3, Annex C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 01: - Add OAuth Client Credential Scopes on M1 and M5 APIs (for each HTTP method). - Add OAuth Authorization Code flow and scopes to M5 APIs. |  |  |  |  |  |  |  |  |

**** First Change for TS 26.510 ****

## 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

…

[RFC5246]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[RFC8446]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[RFC6749]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework", October 2012.

[29222]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

**** Next Change ****

## 7.4	Security

### 7.4.1	General

The Media AF shall enable secure provision of information in the Media Delivery System by authenticated and authorised Media-aware Applications or Media Application Providers.

### 7.4.2	Authorising Media Application Provider access to the Media AF at reference point M1

When a Media Application Provider deployed outside the Trusted DN attempts to access a Media AF deployed inside the Trusted DN, the Media Delivery System shall authenticate and authorise the Media Application Provider.

Access to the Maf_Provisioning API of the Media AF by the Media Application Provider at reference point M1 may be authorised by means of the OAuth 2.0 protocol specified in RFC 6749 [RFC6749]), using the client credentials authorization grant.

NOTE:	The provisioning and the negotiation of the security method is not specified in this release.

When CAPIF (see TS 29.222 [29222]) is used for external API exposure:

-	The CAPIF core function shall play the role of authorization server, the Media AF shall play the role of resource server and the Media Application Provider shall play the role of client.

-	Before invoking any service operation exposed by the Media AF, the Media Application Provider shall negotiate the security method (PKI, TLS-PSK or OAuth 2.0) with the CAPIF core function and shall ensure that the Media AF has the required credentials to authenticate access tokens subsequently presented by the Media Application Provider (see clauses 5.6.2.2 and 6.2.2.2 of TS 29.222 [29222]).

-	If PKI or TLS-PSK is the selected security method between the Media Application Provider and the Media AFthe Media AF shall, upon invocation of a Maf_Provisioning service operation by the Media Application Provider at reference point M1, retrieve the authorisation information from the CAPIF core function as described in clause 5.6.2.4 of TS 29.222 [29222].

-	If OAuth 2.0 [RFC6749] is the selected security method between the Media Application Provider and the Media AF	, the Media Application Provider shall, prior to invoking Maf_Provisioning service operations on the Media AF at reference point M1, obtain an access token from the authorization server (CAPIF core function) by invoking the Obtain_Authorization service operation specified in clause 5.6.2.3.2 of TS 29.222 [29222].

Otherwise:

-	The Media AF shall play the role of both authorization server and resource server, and the Media Application Provider shall play the role of client.

-	The Media Application Provider shall obtain an access token from the authorization server (Media AF) using the client credentials authorization grant specified in section 4.4 of RFC 6749 [RFC6749] prior to invoking Maf_Provisioning service operations on the resource server (Media AF) at reference point M1.

### 7.4.3	Authorising Media Session Handler access to the Media AF at reference point M5

When a Media Session Handler deployed in a Media Client attempts to access a Media AF deployed inside the Trusted DN, the Media Delivery System shall authenticate and authorise the Media Session Handler.

Access to the Maf_SessionHandling API of the Media AF by the Media Session Handler at reference point M5 shall be authorised by means of the OAuth 2.0 protocol specified in RFC 6749 [RFC6749], using the "Client Credentials" or "Authorization Code" flow grant types.

NOTE:	The provisioning and the negotiation of the security method is not specified in this release.

When CAPIF (see TS 29.222 [29222]) is used for external API exposure:

-	The CAPIF core function shall play the role of authorization server, the Media AF shall play the role of resource server and the Media Session Handler shall play the role of client.

-	Before invoking any service operation exposed by the Media AF, the Media Application Provider shall negotiate the security method (PKI, TLS-PSK or OAuth 2.0) with the CAPIF core function and shall ensure that the Media AF has the required credentials to authenticate access tokens subsequently presented by the Media Application Provider (see clauses 5.6.2.2 and 6.2.2.2 of TS 29.222 [29222]).

-	If PKI or TLS-PSK is the selected security method between the Media Session Handler and the Media AFthe Media AF shall, upon invocation of a Maf_SessionHandling service operation by the Media Session Handler at reference point M1, retrieve the authorisation information from the CAPIF core function as described in clause 5.6.2.4 of TS 29.222 [29222].

-	If OAuth 2.0 [RFC6749] is the selected security method between the Media Session Handler and the Media AF	, the Media Session Handler shall, prior to invoking Maf_SessionHandling service operations on the Media AF at reference point M5, obtain an access token from the authorization server (CAPIF core function) by invoking the Obtain_Authorization service operation specified in clause 5.6.2.3.2 of TS 29.222 [29222].

Otherwise:

-	Either the Media AF shall play the role of both authorization server and resource server, or the Media AF shall play the role of the resource server and the Media Application Provider plays the role of the authorization server. The Media Session Handler shall play the role of client.

-	The Media Session Handler shall obtain an access token from the authorization server using either the client credentials grant specified in section 4.4 of RFC 6749 [RFC6749] or the authorization code grant specified in section 4.1 of RFC 6749 [RFC6749] prior to invoking Maf_Provisioning service operations on the resource server (Media AF) at reference point M5.

**** Open API ****

# 3GPP Forge merge request

The code changes associated with this Change Request are available for review at the following URL:

https://forge.3gpp.org/rep/sa4/5gms_pro_ph2/-/merge_requests/12/diffs

**** Last Change ****