3GPP TSG-SA5 Meeting #158	S5-246372

Orlando, USA, 18 - 22 November 2024

# Source:	Huawei

# Title:	DP on observations and proposals for aIMLInferenceName

# Document for:	Endorsement

Agenda Item:	6.4.3

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP TS 28.105 "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management"".

[2]	3GPP TS 28.104 "Management and orchestration; Management Data Analytics (MDA) ".

[3]	3GPP TS 29.520 "5G System; Network Data Analytics Services; Stage 3"

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[6]	3GPP TS 38.401: "NG-RAN; Architecture description".

3	Rationale

3.1 Background and observations

3.1.1 Stage 2 and Stage 3 definitioin for attribute “aIMLInferenceName”

Following is the Stage 2 definitioin for attribute “aIMLInferenceName” in TS 28.105

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aIMLInferenceName | It indicates the type of inference that the ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for NG-RAN (see TS 38.300 [16] and TS 38.401 [17]), and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

The attribute “aIMLInferenceName” is defined as string with following allowed values:

- the values of the MDA type (see 3GPP TS 28.104 [2])

- Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3])

- types of inference for NG-RAN (see TS 38.300 [16] and TS 38.401 [17])

- vendor's specific extensions

Following is the stage 3 definition for the attribute “aIMLInferenceName” in TS28105_AiMlNrm.yaml

aIMLInferenceName:

type: string

Observation#1: An attribute defined as String, especially for the important attribute name “aIMLInferenceName” which is used for multiple IOCs throughout the whole ML model LCM management, is not a good design pattern to support interoperability in multi-vendor scenarios. Also, the allowed values for string are not described in stage 3 openAPI definition.

Proposal#1: Suggest to change the type for the attribute“aIMLInferenceName” to <<Choice>> as follows, each Choice is defined as Enumeration. SchedulingTime <<choice>> Attribute name; S; isReadable; isWritable; isInvariant; isNotifyable. CHOICE_1.1 mDAType; CM; T; T; F; T. CHOICE_2.1 nwdafAnalyticsType; CM; T; T; F; T. CHOICE_3.1 ngRanInferenceType; CM; T; T; F; T. CHOICE_4.1 vSExtensionType; CM; T; T; F; T. Name; Definition. CHOICE_1.1 mDAType; This attribute shall be supported, when the MnS producer supports management of AI/ML inference for MDA. CHOICE_2.1 nwdafAnalyticsType; This attribute shall be supported, when the MnS producer supports management of AI/ML inference for NWDAF. CHOICE_3.1 ngRanInferenceType; This attribute shall be supported, when the MnS producer supports management of AIML inference for NG-RAN. CHOICE_4.1 vSExtensionType; This attribute shall be supported, when the MnS producer supports a management activity for an ML model with inference name is vendor's specific extensions.

3.1.2 Stage 2 and Stage 3 definitioin for MDAType

Following is the Stage 2 definitioin for attribute “aIMLInferenceName” in TS 28.104

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mDAType | It indicates the type of MDA type (corresponding to the MDA capability). AllowedValues: the value of MDA type defined for each MDA capability in clause 8. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

8.4.1.1	Coverage problem analysis

8.4.1.1.1	MDA type

The MDA type for coverage problem analysis is: CoverageAnalytics.CoverageProblemAnalysis.

8.4.3.1	MDA assisted failure prediction

8.4.3.1.1	MDA type

The MDA type for failure prediction analysis is: MDAAssistedFaultManagement.FailurePrediction.

…..

Following is the stage 3 definition for the attribute “mDAType” in TS28104_MdaNrm.yaml

MDATypes:

type: array

items:

type: string

Observation#2: For MDA type, the potential values (e.g. CoverageAnalytics.CoverageProblemAnalysis, MDAAssistedFaultManagement.FailurePrediction) are specified for each MDA capability in clause 8.4, however, the Enumeration for the attribute “mDAType” is missing, especially for stage3 definition.

Proposal#2: It proposes to add Enumerations for the attribute “mDAType” for both stage 2 and stage 3 according to the value of MDA type defined for each MDA capability in clause 8 in TS 28.104.

3.1.3 Stage 2 and Stage 3 definitioin for Analytics ID(s) of NWDAF

Observation#3: The Analytics ID is used and described for each NWDAF analytic use case in TS 23.288, however, corresponding stage 2 and stage3 definition for Analytics ID(s) of NWDAF is NwdafEvent with Enumerations specified in TS 29.520.

Table 5.1.6.3.4-1: Enumeration NwdafEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NF_LOAD | Indicates that the event subscribed is NF Load. | NfLoad |
| QOS_SUSTAINABILITY | Indicates that the event subscribed is QoS sustainability. | QoSSustainability |
| SLICE_LOAD_LEVEL | Indicates that the event subscribed is load level information of Network Slice |  |
| SERVICE_EXPERIENCE | Indicates that the event subscribed is service experience. | ServiceExperience |
| UE_MOBILITY | Indicates that the event subscribed is UE mobility information. | UeMobility |
| UE_COMM | Indicates that the event subscribed is UE communication information. | UeCommunication |
| ABNORMAL_BEHAVIOUR | Indicates that the event subscribed is abnormal behaviour information. | AbnormalBehaviour |
| USER_DATA_CONGESTION | Indicates that the event subscribed is user data congestion information | UserDataCongestion |
| NETWORK_PERFORMANCE | Indicates that the event subscribed is network performance information | NetworkPerformance |
| NSI_LOAD_LEVEL | Indicates that the event subscribed is load level information of Network Slice and the optionally associated Network Slice Instance | NsiLoad |
| DISPERSION | Indicates that the event subscribed is dispersion information. | Dispersion |
| RED_TRANS_EXP | Indicates that the event subscribed is redundant transmission experience. | RedundantTransmissionExp |
| WLAN_PERFORMANCE | Indicates that the event subscribed is WLAN performance. | WlanPerformance |
| DN_PERFORMANCE | Indicates that the event subscribed is DN performance information. | DnPerformance |
| E2E_DATA_VOL_TRANS_TIME | Indicates that the event subscribed is E2E data volume transfer time | E2eDataVolTransTime |
| SM_CONGESTION | Indicates that the event subscribed is the Session Management Congestion Control Experience information for specific DNN and/or S-NSSAI. | SMCCE |
| PFD_DETERMINATION | Indicates that the event subscribed is the PFD Determination information for known application identifier(s). | PfdDetermination |
| PDU_SESSION_TRAFFIC | Indicates that the event subscribed is the PDU Session traffic information. | PduSesTraffic |
| MOVEMENT_BEHAVIOUR | Indicates that the event subscribed is the Movement Behaviour information. | MovementBehaviour |
| LOC_ACCURACY | Indicates that the event subscribed is the Location Accuracy information. | LocAccuracy |
| RELATIVE_PROXIMITY | Indicates that the event subscribed is the Relative Proximity information. | RelativeProximity |

Proposal#3: It proposes reuse the attribute “NwdafEvent” and corresponding Enumerations for Analytics ID(s) of NWDAF

3.1.4 Stage 2 and Stage 3 definitioin for types of inference for NG-RAN

Observation#4: For types of inference for NG-RAN, TS 28.105 described to see TS 38.300 and TS 38.401, however, it is impossible to find inference type in TS 38.300 and TS 38.401. Currently only three AIML for NG-RAN use cases (Network Energy Saving, Load Balancing, Mobility Optimization) described in TS 38.300.

Proposal#4: It proposes to add attribute “ngRanInferenceType” and corresponding Enumerations according to the three AIML for NG-RAN use cases (Network Energy Saving, Load Balancing, Mobility Optimization) described in TS 38.300 to support interoperability for ML model management for NG-RAN.

4	Detailed proposal

It proposes to discuss and endorse following proposals:

Proposal#1: Suggest to change the type for the attribute“aIMLInferenceName” to <<Choice>> as follows, each Choice is defined as Enumeration.

SchedulingTime <<choice>>

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 mDAType | CM | T | T | F | T |
| CHOICE_2.1 nwdafAnalyticsType | CM | T | T | F | T |
| CHOICE_3.1 ngRanInferenceType | CM | T | T | F | T |
| CHOICE_4.1 vSExtensionType | CM | T | T | F | T |

Proposal#2: It proposes to add Enumerations for the attribute “mDAType” for both stage 2 and stage 3 according to the value of MDA type defined for each MDA capability in clause 8 in TS 28.104.

Proposal#3: It proposes reuse the attribute “NwdafEvent” and corresponding Enumerations for Analytics ID(s) of NWDAF.

Proposal#4: It proposes to add attribute “ngRanInferenceType” and corresponding Enumerations according to the three AIML for NG-RAN use cases (Network Energy Saving, Load Balancing, Mobility Optimization) described in TS 38.300 to support interoperability for ML model management for NG-RAN.