3GPP TSG-SA5 Meeting #158	S5-246467

Orlando, USA, 18 - 22 November 2024

# Source:	China Telecom

# Title:	Rel-19 DP on Charging Aspects of 5GS LCS

# Document for:	Discussion

Agenda Item:	7.4.2

1	Decision/action requested

The group is asked to discuss and endorse the proposal.

2	References

[1]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[2]	3GPP WID S5-241827: “Rel-19 New WID on Charging Aspects of Ranging and Sidelink Positioning”.

[3]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

3	Rationale

3.1 Background

5G System (5GS) Location Services (LCS) is defined in 3GPP TS 23.273 [1], with the architecture illustrated in Figure 1. However, there are currently no corresponding charging solutions for 5GS LCS. Related work has been conducted on the normative work of 5GS charging for Ranging and Sidelink Positioning [2], which shares similarities with 5GS LCS. However, Ranging and Sidelink Positioning represents only one scenario within 5GS LCS. It is necessary to specify charging solutions specifically for 5GS LCS and the converged charging architecture may be reusable for 5GS LCS with further investigation.

Besides, TS 23.273 [1] has introduced LCS user plane positioning, where LPP messages between UE and LMF can be transferred via user plane after UE and LMF established a user plane connection, which means if UE initiated an LCS session via user plane connection (e.g., request GNSS assistance data from LMF via the use plane), AMF and GMLC is not aware of this LCS session. How to perform charging in this case needs further investigation.

Figure 1: Non-roaming reference architecture for Location Services in reference point representation [1]

Figure 2: Ranging and Sidelink Positioning Converged charging architecture [3]

3.2 Objective

The objective of this work is to specify charging mechanisms for the 5GS LCS:

- WT-1: Support of architecture for converged charging for 5GS LCS

- WT-2: Support of converged charging for 5GC-MT-LR Procedure

- WT-3: Support of converged charging for 5GC-MO-LR Procedure

- WT-4: Support of converged charging for 5GS LCS user plane positioning

3.3 TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) | Non-3GPP Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- | --- | --- |
| WT-1 | 0 | 0.5 | No | No | No |
| WT-2 | 0 | 1 | No | No | No |
| WT-3 | 0 | 1 | No | No | No |
| WT-4 | 0 | 1 | No | No | No |

Total TU estimates for the study phase: 0

Total TU estimates for the normative phase: 3.5

Total TU estimates: 3.5

3.4 Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 32.271 | Enhancement to support the 5GS LCS | TSG SA#108 (Jun 2024) |  |
| TS 32.290 | Update common converged charging service description | TSG SA#108 (Jun 2025) |  |
| TS 32.291 | Update Nchf_ConvergedCharging service API with definitions that are specific to 5GS LCS | TSG SA#109 (Sep 2025) |  |
| TS 32.298 | 5GS LCS charging related Charging Data Record (CDR) description | TSG SA#109 (Sep 2025) |  |

4	Detailed proposal

The group is asked to endorse the proposal in the rationale.