3GPP TSG-SA5 Meeting #158	S5-246468

# Orlando, US, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0609 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 New measurements to monitor the DL packet delay for XR PDU sets |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  | XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements as defined in TS 22.261. The below table (table 7.6.1-1 from 3GPP TS 22.261) provides information different KPIs and the characteristics of those KPIs for different use cases. 3GPP TS 22.261 Table 7.6.1-1 KPI Table for additional high data rate and low latency service Use Cases; Characteristic parameter (KPI); Influence quantity. Max allowed end-to-end latency; Service bit rate: user-experienced data rate; Reliability; # of UEs; UE Speed; Service Area (note 2). Cloud/Edge/Split Rendering (note 1); 5 ms (i.e. UL+DL between UE and the interface to data network) (note 4); 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content.; 99,99 % in uplink and 99,9 % in downlink (note 4); -; Stationary or Pedestrian (note 7); Countrywide. Gaming or Interactive Data Exchanging (note 3); 10ms (note 4); 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content.; 99,99 % (note 4); ≤ [10]; Stationary or Pedestrian (note 7); 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h). Consumption of VR content via tethered VR headset (note 6); [5 to 10] ms (note 5); 0,1 to [10] Gbit/s (note 5); [99,99 %]; -; Stationary or Pedestrian; -. NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and can be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. NOTE 7: Similar user-experienced data rates may be achievable also at higher UE speeds. [50]. Based on these requirements for the KPIs, different working groups in 3GPP have derived the WG specific requirements and worked on those. The maximum allowed latency for DL and UL has been specified for different use cases and the 3GPP system (Core and RAN) need to make sure that the actual latency is less than the specified latency in the table above. Hence it is essential for the mobile networks operator to measure the latency of the XR data packets in the 3GPP networks. |
| Summary of change: |  | 4 new measurements are proposed - Average delay for GTP T-PDU of PDU Set in UPF - Average delay for GTP T-PDU of PDU set in gNB-CU-UP for split scenario - Average delay for RLC PDUs of the PDU set in gNB-DU for split scenario - Average delay for PDCP PDUs of the PDU set in gNB for non-split scenario |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no standardized means of monitoring and measuring the delay of the PDU set for XR data transmitted over 3GPP network and hence there will be no means for the operator to know about the existing delay in PDU set transmission in the 3GPP network. If the delay budget requirement is not met for XR use cases, the delay would be visible to the end user and will lead to poor user experience. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Below new clauses are added 5.1.1.27.a, 5.1.1.27.b, A.x |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-246468 is a revision of S5-245522 |  |  |  |  |  |  |  |  |  |

Start of First change

#### 5.1.2.x	PDCP PDU Set Delay

##### 5.1.2.x.1	Average DL PDCP PDU Set delay

a)	This measurement provides the average (arithmetic mean) of the delay seen in DL PDCP PDUs corresponding to PDUs in the PDU Set between gNB and the UE. The measurement is split into subcounters per QFI and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: the difference between the time at which the first PDCP PDU of the first PDU of the PDU set is transmitted from the gNB to UE and the time at which the acknowledgement of the last PDCP PDU of the last PDU of the same PDU set is received at the gNB from UE.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	DRB.PDUSetPDCPPacketDelayDl.QFI, where QFI identifies the QFI;

DRB.PDUSetPDCPPacketDelayDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality for XR services)

Note: This measurement is applicable for RLC AM mode only.

Start of next change

##### 5.1.3.1.x	Average DL GTP T-PDU delay for PDU Set in gNB-CU-UP

a)	This measurement provides the average (arithmetic mean) of the delay seen in DL GTP T-PDUs corresponding to PDUs in the PDU Set between gNB-CU-UP and gNB-DU. The measurement is split into subcounters per QFI and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: the difference between the time at which the first GTP T-PDU corresponding to the first PDU of the PDU set is transmitted from the gNB-CU-UP to gNB-DU and the time at which the acknowledgement of the last GTP T-PDU of the last PDU of the same PDU set is received at the gNB-CU-UP from gNB-DU.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	DRB.PDUSetGTPPacketDelayDl.QFI, where QFI identifies the QFI;

DRB.PDUSetGTPPacketDelayDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

##### 5.1.3.1.y	Average DL RLC PDU Set delay in gNB-DU

a)	This measurement provides the average (arithmetic mean) of the delay seen in DL RLC PDUs corresponding to PDUs in the PDU Set between gNB-DU and the UE. The measurement is split into subcounters per DRB-ID and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: the difference between the time at which the first RLC PDU of the first PDU of the PDU set is transmitted from the gNB-DU to the UE and the time at which the acknowledgement of the last RLC PDU of the last PDU of the same PDU set is received at the gNB-DU from the UE.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	DRB.PDUSetPDCPPacketDelayDl.DRBID, where DRBID identifies the DRB;

DRB.PDUSetPDCPPacketDelayDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

Note: This measurement is applicable for RLC AM mode only.

Start of next change

##### 5.4.5.1.x	Average DL PDU Set GTP packet delay in PSA UPF

a)	This measurement provides the average (arithmetic mean) DL PDU Set GTP packet delay between PSA UPF and gNB. The measurement is split into subcounters per QFI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained as: the difference between the time at which the first GTP T-PDU corresponding to the first PDU of the PDU set is transmitted from the PSA UPF to gNB and the time at which the acknowledgement of the last GTP T-PDU of the last PDU of the same PDU set is received at the UPF from the gNB. In case of split gNB, it is defined as the difference between the time at which the first PDU of the PDU set is transmitted from the PSA UPF to gNB-CU-UP and the time at which the acknowledgement of the last PDU of same PDU set is received at the UPF from the gNB-CU-UP.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	GTP.DelayDlPduSetUpfMean.QFI, where QFI identifies the QFI;

GTP.DelayDlPduSetUpfMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

Start of next change

# A.x	XR PDU Set Packet Delay

To provide immersive experience for users, XR and Media services (XRM) are usually characterized by high data rate and low latency. Different work groups in 3GPP worked on different aspects of the XRM services like multi-modality transmission, 5G system (5GS) information exposure, PDU set-based QoS handling, uplink-downlink transmission coordination, Packet Delay Variation monitoring and reporting, and Power Saving enhancements. The AF provides RT latency requirements together with a RT latency indication to the PCF, which indicates the service data flow needs to meet that the RT delay overhead doesn't exceed the RT latency requirement. The PCF is responsible to split the RT latency into an UL PDB and a DL PDB. The restriction is that the UL PDB and DL PDB can be unequal but their sum shall not exceed the RT latency. The PCF generates two PCC rules for UL stream and DL stream and assigns the 5QIs according to PDBs respectively. Similarly, the AF provides Packet Delay Variation requirement for specific QoS flow. The PDV can show the network transmission stability. Once the PCF accept the PDV requirement, the corresponding PCC rules for QoS monitoring can be generated and sent to SMF to trigger the QoS monitoring for packet delay. When the PCF receives the packet delay values, the PCF itself can calculate the PDV based on IETF RFC 1889 or 3393.

The DL and the UL PDU Set Delay Budget is configured in UPF and gNB from the SMF separately. The delay budget values are to be considered by the UPF and the gNB to make sure the end user experience for immersive XRM use cases are satisfied. The following table from SA1 indicates the delay requirements for different use cases of ZRM services.

3GPP TS 22.261 Table 7.6.1-1 KPI Table for additional high data rate and low latency service

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5 ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % in uplink and 99,9 % in downlink (note 4) | - | Stationary or Pedestrian (note 7) | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % (note 4) | ≤ [10] | Stationary or Pedestrian (note 7) | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Consumption of VR content via tethered VR headset (note 6) | [5 to 10] ms (note 5) | 0,1 to [10] Gbit/s (note 5) | [99,99 %] | - | Stationary or Pedestrian | - |
| NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and can be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. NOTE 7: Similar user-experienced data rates may be achievable also at higher UE speeds. [50] |  |  |  |  |  |  |

The below diagram depicts the scenario of a non-split gNB. The overall packet delay in the DL is the sum of delays of the individual hops. In this case, there are 2 hops – the NG-U interface between the UPF and gNB-CU-UP and the DRB over air interface

Figure A.x-1: PDU Set Delay – Non-Split gNB – Delay Calculation

The below diagram depics the scenario of split-gNB. The overall packet delay is the sum of three individual hops. In this case, there are 3 hops, NGU interface between the UPF and the gNB-CU-UP, the F1U interface between the gNB-CU-UP and gNB-DU, and the NR-Uu interface between the gNB-DU and the UE.

Figure A.x-2: PDU Set Delay – Split gNB – Delay Calculation

End of change