3GPP TSG-SA5 Meeting #158	S5-246511

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0054 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.254 Correction on Attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Amdocs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Misalignment on the attributes defined for API Charging Information |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description and presence of certain attributes for API Charging Information is inconsistent between different clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There could be confusion on the implementation for Northbound API Charging between different vendors |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a.3.2, 6.3.1.4, 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 6.2a.3.2	NEF charging CHF CDR data

If enabled, CHF CDRs for NEF charging shall be produced for NEF chargeable events.

The fields of NEF charging CHF CDR are specified in table 6.2a.3.2.1.

Table 6.2a.3.2.1: NEF charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | Described in TS 32.298 [3] |
| Recording Network Function ID | OM | Described in TS 32.298 [3] |
| Subscriber Identifier | M | This field contains the identification of the API user (e.g. SCS Identifier), or the identifier of the AF as an alternative to tenant identifier |
| Tenant Identifier | OC | This field contains the identification of the Tenant, and may hold the Application Identifier |
| NF Consumer Information | M | This field holds the information of the NEF that used the charging service |
| List of Multiple Unit Usage | OM | Described in TS 32.298 [3] |
| Record Opening Time | OC | Described in TS 32.298 [3] |
| Duration | M | Described in TS 32.298 [3] |
| Record Sequence Number | OC | Described in TS 32.298 [3] |
| Cause for Record Closing | M | Described in TS 32.298 [3] |
| Local Record Sequence Number | OM | Described in TS 32.298 [3] |
| Record Extensions | OC | Described in TS 32.298 [3] |
| NEF API Charging Information | OM | This field holds the NEF specific information defined in clause 6.3.1.4 |

Second Change

#### 6.3.1.4	Definition of the NEF API Information

##### 6.3.1.4.1	Definition of the NEF API Charging Information

Network Exposure Function API specific charging information is provided within the NEF API Charging Information. The detailed structure of the NEF API Charging Information can be found in table 6.3.1.4.1.

Table 6.3.1.4.1: Structure of the NEF API Charging Information

| Information Element |  | Category |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| External Individual Identifier |  | OC |  | This parameter holds the external Identifier of the individual UE, e.g., the GPSI. |  |
| External Individual Id List |  | OC |  | This parameter holds a list of external identifiers, used instead of External Individual Identifier if there is more than one. |  |
| Internal Individual Identifier |  | OC |  | This parameter holds the internal Identifier of the individual UE e.g., the SUPI. |  |
| Internal Individual Id List |  | OC |  | This parameter holds a list of internal identifiers, used instead of Internal Individual Identifier if there is more than one. |  |
| External Group Identifier |  | OC |  | This parameter holds the external identifier for a group of individual UE(s). |  |
| Internal Group Identifier |  | OC |  | The internal Identifier identifying a group of individual UE(s). |  |
| API Direction |  | OC |  | This field holds the direction to indicate if it is an API invocation from an AF or notification to an AF. |  |
| API Target Network Function |  | OC |  | This field holds the identifier of the network function that either is the destination of the API invocation or triggers the notification: e.g. PCF, UDM, AMF, SMF, UPF. |  |
| API Result Code |  | OC |  | This parameter holds the result of API Invocation. |  |
| API Name |  | M |  | This field holds the name of the API invoked. |  |
| API Operation |  | OC |  | This field holds the operation of the API invoked, together with detailed description. |  |
| API Reference |  | OC |  | This field holds the reference to the definition of the format of the API invocation, this can be a URI or refence to the standard where it's specified |  |
| API Content |  | OC |  | This field holds the actual content of the API invocation, in the format described by the API Reference |  |

Third Change

### 6.3.4	Detailed message format for converged charging

The operation types are listed in the following order: I  (Initial) / U (Update)/T  (Termination)/E  (Event). Therefore, when all Operation types are possible it is marked as IUTE. If only some operation types are allowed for a node, only the appropriate letters are used ( e.g., IUT or E) as indicated in the table heading. The omission of an operation type for a particular field is marked with "-" ( e.g., I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.4.1 illustrates the basic structure of the supported fields in the Charging Data Request for exposure function API online charging.

Table 6.3.4.1: Supported fields in Charging Data Request message

| Information Element |  | Node Type | NEF |  |
| --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | IUTE |  |
| Session Identifier |  |  | T |  |
| Subscriber Identifier |  |  | I-TE |  |
| Tenant Identifier |  |  | I-TE |  |
| NF Consumer Identification |  |  | I-TE |  |
| Charging Identifier |  |  | I-TE |  |
| Invocation Timestamp |  |  | I-TE |  |
| Invocation Sequence Number |  |  | I-TE |  |
| One-time Event |  |  | ---E |  |
| One-time Event Type |  |  | ---E |  |
| Notify URI |  |  | I--- |  |
| Supported Features |  |  | I--E |  |
| Triggers |  |  | I-TE |  |
| Multiple Unit Usage |  |  | I-TE |  |
| Rating Group |  |  | I--E |  |
| Requested Unit |  |  | I--E |  |
| Used Unit Container |  |  | --TE |  |
| NEF API Charging Information |  |  |  |  |
| External Individual Identifier |  |  | I-TE |  |
| External Individual Id List |  |  | I-TE |  |
| Internal Individual Identifier |  |  | I-TE |  |
| Internal Individual Id List |  |  | I-TE |  |
| External Group Identifier |  |  | I-TE |  |
| Internal Group Identifier |  |  | I-TE |  |
| API Direction |  |  | I-TE |  |
| API Target Network Function |  |  | I-TE |  |
| API Result Code |  |  | I-TE |  |
| API Name |  |  | I-TE |  |
| API Operation |  |  | I-TE |  |
| API Reference |  |  | I-TE |  |
| API Content |  |  | I-TE |  |

Table 6.3.4.2 illustrates the basic structure of the supported fields in the Charging Data Response for exposure function API converged charging.

Table 6.3.4.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | IUTE |
| Session Identifier |  | I-TE |
| Invocation Timestamp |  | I-TE |
| Invocation Result |  | I-TE |
| Invocation Sequence Number |  | I-TE |
| Session Failover |  | I-TE |
| Supported Features |  | I--E |
| Multiple Unit Information |  | I--E |
| Result Code |  | I--E |
| Rating Group |  | I--E |
| Granted Unit |  | I--E |
| Validity Time |  | I--E |

End of changes