# 3GPP TSG SA WG5 Meeting #158	S5-246580

# Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei

# Title:	Evaluation for IMS DC charging topic 2

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to approve the following update in TR 28.851 [1].

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR provides evaluation for topic 2 on IMS DC download and usage charging in TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851 [1].

First change

5.2.5	Evaluation

Solution #1 proposed a new charging unit, i.e. the number of DC application download. This requires DCSF as a new CTF to collect and report DC application download information to CHF.

Solution #x proposed to charge based on the DC application usage duration. The charging information can reuse the existing IE in IMS charging. The charging principle may be extended to describe the support for duration based charging per DC application.

Solution #y proposed to charge based on the data volume consumed during DC application usage on per IMS DC session basis. The SMF charging can already support this soluion. However, this solution does not distinguish the volume consumed for each application, when the IMS DC session involved more than one application DC.

Solution #z proposed to charge based on the data volume consumed during DC application usage on per application DC basis. Compared to solution #y, this solution proposes a more specific granularity to count the volume, i.e. on a per application DC basis. However, the extension of charging information to identify each application depends on the support from N7 and N5/Rx interface, which is in the scope of SA2/CT work groups.

End of changes