3GPP TSG SA WG5 Meeting #158	S5-246584

Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei

# Title:	Correction on roaming A2X communication solution

# Document for:	Approval

Agenda Item:	7.5.4

1 	Discussion

This is a pCR to provide the correction to solution for UAS charging in TR 28.853.

2	References

[1]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems".

[2]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

3	Rationale

This pCR proposes to correct the solution #4.3 on roaming A2X communication solution. According to TS 23.247 [2] clause 4.1, Multicast/Broadcast Service for roaming is not supported in this release, which refers to release 18 (the TS has not upgrade to release 19 yet). Therefore, the charging solution should remove MBS session from the roaming A2X communication charging solution.

A minor correction: to correct the number for key issue #4a, aligning with the style in other topics within this TR.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

5.4.3.1	Key issue #4a: Charging information required

This key issue is for investigating how to support the A2X services for UAS charging considering REQ- 3GPPCH-A2X-01, REQ- 3GPPCH-A2X-02 and REQ- 3GPPCH-A2X-03. This investigation covers the following:

-	identification of the charging architecture for the A2X service for UAS Charging;

-	identification of triggers and charging information required to A2X service for UAS Charging.

Next change

5.4.4.1	Solution #4.1: Provisioning for A2X communication via UAS NF/NEF

5.4.4.1.1	General description

This solution#4.1 which relying on CHF/5G Converged Charging System for UAS NF/NEF Charging, addresses the Key Issue#4a.

Next change

5.4.4.2	Solution #4.2: Non-roaming A2X Communication over Uu reference point

5.4.4.2.1	General description

This solution#4.2 which relying on CHF/5G Converged Charging System for Non-roaming A2X Communication Charging, addresses the Key Issue#4a.

A2X communication over PC5 reference point or over Uu reference point are supported, and this solution only considers the A2X communication over Uu reference point.

-	the mechanisms defined in TS 23.501 [4] and TS 23.502 [5] can be used to establish the suitable PDU Sessions, and V2X messages are routed towards V2X Application Server or towards UEs with existing unicast routing, as specified in the clause 5.2.2.1 TS 23.287[8];

-	The mechanisms defined in TS 23.247 [9] can be used to establish the suitable MBS sessions, and V2X messages are routed from the V2X Application Server towards UEs via broadcast MBS sessions or multicast MBS sessions, as specified in the clause 5.2.2.2 TS 23.287[8].

Next change

5.4.4.3	Solution #4.3: Roaming A2X Communication over Uu reference point

5.4.4.3.1	General description

This solution#4.3 which relying on CHF/5G Converged Charging System for Roaming A2X Communication, addresses the Key Issue#4a.

The charging for A2X communication over Uu reference point using the PDU session  in roaming case, including HR and LBO scenarios, refers to TS 23.501 [4] and TS 23.502 [5]. As defined in TS 23.247 [9], multicast/broadcast service for roaming is not supported in this release.

5.4.4.3.2	Architecture description

The 5G System high level charging architecture for SMF charging in roaming cases to support the A2X communication roaming scenarios.

5.4.4.3.3	Procedures description

The high-level charging procedure for A2X communication over Uu reference point in roaming case can refer to the PDU session charging as specified in the TS 32.255[10].

End of change