3GPP TSG-SA5 Meeting #158	S5-24xxxx

Orlando, USA, 18 - 22 November 2024

# Source:	Samsung

# Title:	Evaluations enhancements

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

None

# 3	Rationale

This provides enhancements for existing evaluations

# 4	Detailed proposal

Next Change

### 5.1.x	Exposure of management services through the CAPIF framework

To expose management services through the CAPIF framework, the MnS Producer, in addition to producing the MnS, shall also provide the API provider domain functions (i.e., API Exposing Function (AEF), API management function (AMF), API publishing function (APF)) as defined by the CAPIF framework in clause 6.3 of TS 23.222[5]. The MnS Producer can consume CCF APIs (i.e., CAPIF-3, CAPIF-4 and CAPIF-5 interfaces) and allow API invokers to access management services through service APIs (using CAPIF-2/2e interface).

Figure 5.1.0-1: MSEF implementing the API provider domain functions as defined by the CAPIF framework.

The MnS Producer implements the CAPIF API provider domain functions. The MnS Producer publishes the management service APIs for discovery through the CAPIF core function (CCF) and exposes the management services for invocation by external MnS consumers through the AEF.

EGMF implements CCF functionalities in addition to functionalites related with managing authorization of external MnS consumer including what consumer is authorized to discover and consume.

Next Change

### 5.1.1	Use case #1: MnS producer registration into CAPIF

#### 5.1.1.1	Description

For a MnS to be made available for consumption through an exposure framework, it is needed that this MnS producer registers into that exposure framework. Once registered, the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS from external MnS consumers. These procedures include publication, discovery and access control, among others.

In the CAPIF, the scope of registration is limited to API provider domain functions, which include API Exposing Function (AEF), API Publishing Function (APF) and API Management Function (AMF). The registration is a procedure whereby these functions become recognized users of the CCF. This procedure is described in clause 8.28 of 3GPP TS 23.222 [5], with stage 3 solution set detailed in clause 8.9 of 3GPP TS 29.222 [13].

To register (and/or update the registration information of) an API provider domain on the CCF, the AMF from this API provider domain communicates with the CCF over CAPIF-5 interface. This means that:

-	This API provider domain should have an AMF.

-	The AMF of this API provider domain should be an authorized user of CCF.

-	The AMF security information for CCF to validate the registration request is provisioned by the CAPIF administrator.

To register a MnS producer on the CCF, it is needed for the MnS producer to support some API provider domain functionality, as detailed below:

-	AEF. If supported, this means that the MnS producer will need to support:

-	CAPIF-2/2e interface, so that the API invokers acting as external MnS consumers can access service APIs, when required. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clause 9.1) can be configured later, depending on the intended usage of MnS producer.

NOTE:	The service APIs can carry different MnS information for different API invokers. This issue is discussed in use case #3 (clause 5.1.3).

-	CAPIF-3 interface, so that the MnS producer can communicate with CCF to exercise access and policy related control for service API invocations initiated by the API invoker. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clauses 8.3, 8.5, 8.7 and 8.6) can be configured later, depending on the intended usage of MnS producer.

-	APF. If supported, this means that the MnS producer will need to support CAPIF-4 interface, so that it can communicate with CCF to publish (and manage the published) MnS information. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clause 8.2) can be configured later, depending on the intended usage of MnS producer.

-	AMF. If supported, this means that MnS producer will need to support CAPIF-5 interface. The API operations that will be implemented in this interface (see 3GPP TS 29.222 [13], clauses 8.3 and 8.9) can be configured later, depending on the intended usage of MnS producer.

The MnS producer could support one or more API provider domain functions, with at least one of them being AEF; otherwise, the MnS producer is not eligible for integration into the CAPIF.

The AMF communicates the MnS producer details (including supported API provider domain functions) to the CCF using CAPIF_API_Provider_Management_API (see 3GPP TS 29.222 [13], clause 8.9). Upon reception of these details, the CCF registers the MnS producer with the supported API provider domain function(s).

#### 5.1.1.2	Potential requirements

PREQ-FS_MExpo-Reg-01: The 3GPP management system shall provide the capability to register a MnS producer on the CCF.

PREQ-FS_MExpo-Reg-02: The 3GPP management system shall provide the capability to deregister a MnS producer on the CCF.

PREQ-FS_MExpo-Reg-03: The 3GPP management system shall provide the capability to update the registration details of a MnS producer on the CCF.

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	Potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails

5.1.1.3.1.1	Introduction

This solution focuses on meeting the requirements PREQ-FS_MExpo-Reg-01, PREQ-FS_MExpo-Reg-02 and PREQ-FS_MExpo-Reg-03.

In the CAPIF, the registration procedure is executed with the CAPIF_API_Provider_Management API (see 3GPP TS 29.222 [13], clause 8.3), initiated by the AMF over the CAPIF-5 interface. The AMF sends a HTTP POST message to the CCF with a request body containing the following dataType: "APIProviderEnrolmentDetails" (see 3GPP TS 29.222 [13], clause 8.9.4.2.2).

The solution proposes to capture MnS producer information with "APIProviderEnrolmentDetails" and reuse the CAPIF_API_Provider_Management API to register, de-register and update registration information related to MnS producer.

5.1.1.3.1.2	Description

How the MnS producer information relevant for registration can be captured with "APIProviderEnrolmentDetails" is described in tables 5.1.1.3.1.2-1, 5.1.1.3.1.2-2 and 5.1.1.3.1.2-3.

Table 5.1.1.3.1.2-1 lists the attributes of type APIProviderEnrolmentDetails (see clause 8.9.4.2.2 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.9.4.2.2-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type APIProviderEnrolmentDetails.

Table 5.1.1.3.1.2-1: Representing MnS producer registration information with
APIProviderEnrolmentDetails attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiProvDomId | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). |  |
| regSec | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | It can be used to store the credentials of the MnS producer. |
| apiProvFuncs | The data type of this attribute is defined as "array(APIProviderFunctionDetails)" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | See Table 5.1.1.3.1.2-2. |
| apiProvDomInfo | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). |  |
| suppFeat | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "C" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | Only applicable when API provider is SCEF/NEF, and thus not applicable when API provider is MnS producer. |
| failReason | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.9.4.2.2-1 of TS 29.222 [13]). |  |
| apiProvName | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.2-1 of TS 29.222 [13]). | RNAA. |

Table 5.1.1.3.1.2-2 lists the attributes of type APIProviderFunctionDetails (see clause 8.9.4.2.3 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.9.4.2.3-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type APIProviderFunctionDetails.

Table 5.1.1.3.1.2-2: Representing MnS producer registration information with
APIProviderFunctionDetails attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiProvFuncId | The data type of this attribute is defined as "string" and presence qualifier is defined as "C" (see table 8.9.4.2.3-1 of TS 29.222 [13]). |  |
| regInfo | The data type of this attribute is defined as "RegistrationInformation" and presence qualifier is defined as "M" (see table 8.9.4.2.3-1 of TS 29.222 [13]). | See Table 5.1.1.3.1.2-3. |
| apiProvFuncRole | The data type of this attribute is defined as "ApiProviderFuncRole" and presence qualifier is defined as "M" (see table 8.9.4.2.3-1 of TS 29.222 [13]). | This data type serves to specify, for the MnS producer to be registered, which CAPIF API provider domain function(s) will be supported. |
| apiProvFuncInfo | See clause 8.9.4.3.3 of TS 29.222 [13] for the enumeration values and description of type ApiProviderFuncRole. |  |

Table 5.1.1.3.1.2-3 lists the attributes of type RegistrationInformation (see clause 8.9.4.2.4 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.9.4.2.4-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type RegistrationInformation.

Table 5.1.1.3.1.2-3: Representing MnS producer registration information with
RegistrationInformation attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiProvPubKey | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.9.4.2.4-1 of TS 29.222 [13]). | It can be used to store the public key of the MnS producer. |
| apiProvCert | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.9.4.2.4-1 of TS 29.222 [13]). | It can be used to store the client certificate of the MnS producer, if existing. |

#### 5.1.1.4	Evaluation of potential solutions

##### 5.1.1.4.1	Evaluation of potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails

To manage the registration of an MnS producer on the CCF using CAPIF_API_Provider_Management API, it is needed to populate the "APIProviderEnrolmentDetails" datatype with the registration information of the MnS producer. Detailed in clause 5.1.1.3.1.2, the mapping solution is summarized below:

-	The credentials of the MnS producer can be mapped to "APIProviderEnrolmentDetails /regSec".

-	The different roles played for the MnS producer can be mapped to "APIProviderEnrolmentDetails /apiProvFuncs". Note that the MnS producer will always play AEF, with the option to also play APF and/or AMF, if needed.

Note: This imply that MnS Producer will implement AEF functionality. MnS Producer can optionally implement ARF and AMF functionalities.

-	The public key of the MnS producer can be mapped to "APIProviderEnrolmentDetails/ apiProvFuncs/regInfo/apiProvPubKey".

The solution is feasible.

Next Change

### 5.1.5	Use case #5: Logging the management service API invocations to the CCF

#### 5.1.5.1	Description

When exposing management services for consumption by the external MnS consumers (referred to as the API invoker in CAPIF terms), it is crucial to monitor, for example, what management service API was invoked (i.e. the MOIs, attributes or operations), who invoked the API (i.e. the ID of the external MnS consumer), the result of the invocation (e.g. success, or failure) and at what time it was invoked.

Accordingly, the MnS producer (which acts as the CAPIF AEF) should be able to create the management service API (now service API) invocation log(s) (see clause 8.7 of 3GPP TS 29.222 [13]) with the desired information. Subsequently, the MnS producer should be able to send the invocation log(s) to the CCF via the CAPIF-3 interface. The stored logs of the service API invocations can be consumed by authorized consumers (e.g. CAPIF API AMF for auditing purposes and the charging functions). However, currently, the MnS producers do not support these functionalities.

#### 5.1.5.2	Potential requirements

PREQ-FS_MExpo-Log-01: The 3GPP management system should support the capability to create logs based on the management service API invocations by external MnS consumers.

PREQ-FS_MExpo-Log-02: The 3GPP management system should support the capability to log the management service API invocations to the CCF.

#### 5.1.5.3	Potential solutions

##### 5.1.5.3.1	Potential solution #1: Creation and logging of the management service API invocations

5.1.5.3.1.1	Introduction

The potential solution assumes that the MnS producer is the AEF and interacts with the CCF via the CAPIF-3 interface. Accordingly, this potential solution describes how the MnS producer can create the management service API invocation log. Secondly, the solution describes how the MnS producer can send these logging data to the CCF for authorized consumers to consume (e.g. AMF and charging functions).

5.1.5.3.1.2	Description

To log the management service API invocations to the CCF, the MnS producer should create an invocation log (see clause 8.7 in 3GPP TS 29.222 [13]). Table 5.1.5.3.1.2-1 maps the CAPIF InvocationLog data type and the MnSInfo IOC (see clause 4.3.42 of 3GPP TS 28.622 [3]) attributes.

Table 5.1.5.3.1.2-1 lists the attributes of type InvocationLog (see clause 8.7.4.2.2 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.7.4.2.2-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type InvocationLog.

Table 5.1.5.3.1.2-1: Mapping between CAPIF InvocationLog data type and the MnSInfo IOC attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| aefId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.2-1 of TS 29.222 [13]). | MnS producer obtains this information from the CCF after MnS producer registration |
| apiInvokerId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.2-1 of TS 29.222 [13]). | Provided by the external MnS consumer when invoking the management service API |
| logs | The data type of this attribute is defined as "array(Log)" and presence qualifier is defined as "M" (see table 8.7.4.2.2-1 of TS 29.222 [13]). | See Table 5.1.5.3.1.2-2 |
| supportedFeatures | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "O" (see table 8.7.4.2.2-1 of TS 29.222 [13]). |  |

Table 5.1.5.3.1.2-2 lists the attributes of type Log (see clause 8.7.4.2.3 of TS 29.222 [13]) and how the MnS producer information can be mapped. See table 8.7.4.2.3-1 of TS 29.222 [13] for the data type, presence indicator, cardinality, description and applicability information for the attributes of type Log.

Table 5.1.5.3.1.2-2: Mapping between CAPIF Log data type and the MnSInfo IOC attributes

| Attribute name | Attribute additional information | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- |
| apiId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | The MnS producer receives this attribute from the CCF after publishing the MnS information. |
| apiName | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Same as mnsLabel. |
| apiVersion | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Same as mnsVersion. |
| resourceName | The data type of this attribute is defined as "String" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | No possible mapping to MnSInfo attributes (could be the IOC name of the MOI). |
| uri | The data type of this attribute is defined as "Uri" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Can be the DN that uniquely identifies the MOI. The DN-to-URI mapping mechanism is defined in clause 4.2.3, 3GPP TS 32.158 [30]. |
| protocol | The data type of this attribute is defined as "Protocol" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | HTTPS?? |
| operation | The data type of this attribute is defined as "Operation" and presence qualifier is defined as "C" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | E.g. PUT, POST, GET, DELETE, PATCH, etc. |
| result | The data type of this attribute is defined as "string" and presence qualifier is defined as "M" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | HTTP status codes. |
| invocationTime | The data type of this attribute is defined as "DateTime" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Is this time when the MnS producer receives the service API invocation request from the external MnS consumer? Needs further clarification. |
| invocationLatency | The data type of this attribute is defined as "DurationMs" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). | Is this the duration of the service API invocation (i.e. from the time the external MnS consumer sends the invocation request to the MnS producer to the time when the MnS producer sends the response back to the external MnS consumer)? Needs further clarification. |
| inputParameters | The data type of this attribute is defined as "ANY TYPE" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| OutputParameters | The data type of this attribute is defined as "ANY TYPE" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| srcInterface | The data type of this attribute is defined as "InterfaceDescription" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| destInterface | The data type of this attribute is defined as "InterfaceDescription" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |
| fwdInterface | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.7.4.2.3-1 of TS 29.222 [13]). |  |

After creating the invocation log, the MnS producer can log these data to the CCF via the CAPIF-3 interface.

#### 5.1.5.4	Evaluation of potential solutions

The proposed solution satisfies the requirement PREQ-FS_MExpo-Log-01. To fulfil the use case requirement PREQ-FS_MExpo-Log-02, the proposed solution requires that the 3GPP management entity providing the AEF role implements the CAPIF-3 interface and the associated functionalities.

The solution proposed in clause 5.1.5.3.1 states “The potential solution assumes that the MnS producer is the AEF and interacts with the CCF via the CAPIF-3 interface” and “After creating the invocation log, the MnS producer can log these data to the CCF via the CAPIF-3 interface”. This imply that MnS Producer implements AEF functionality and should be considered in the normative work.

End of Changes