3GPP TSG-SA5 Meeting #158	S5-246660

Orlando, USA, 18 - 22 November 2024

# Source:	China Mobile, ZTE Corporation

# Title:	Rel-19 pCR TR 28.858 Add Evaluation and Conclusion for Fine-tuning

# Document for:	Approval

Agenda Item:	6.19.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.858: " Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2"

3	Rationale

In last SA5 meeting, the use case and potential solution for fine-tuning is agreed. Therefore, this contribution proposes to add evaluation and conclusion for fine-tuning.

4	Detailed proposal

Start of First change

5.1.3	ML Fine-tuning

5.1.3.1	Description

ML fine-tuning refers to training an already trained ML model (e.g. a pre-trained ML model) with a changed scope.

Re-training of an ML Model is not based on a specific type of inference, while fine-tuning of a pre-trained model means that the ML model will be trained with focus on some specific types of inference.

ML Model training process defined in TS 28.105 covers ML fine-tuning.

5.1.3.2	Use cases

5.1.3.2.1		ML fine-tuning for a pre-trained ML model

An ML model can be trained by dataset that covers more than one type of inference, which the pre-trained model could cover. The pre-trained model can be fine-tuned for specific types of inference by the producer.

NOTE: It is for FFS whether fine-tuning process could be applied for the same producer who pre-trained the model or for another producer.

5.1.3.3	Potential requirements

REQ-ML_ TRAIN-MLFT-1: The ML training MnS producer shall have a capability to enable an authorized consumer to request the fine-tuning of a pre-trained ML model.

Editor’s Note: It is for further discussion if the same requirements in clause 6.2b.3 "Requirements for ML model training" in TS 28.105 [2] related to training function also applies for fine-tuning.

5.1.3.4	Possible solutions

The exsting MLModel IOC and MLTrainingRequest IOC can be enhanced to support fine-tuning.

Enhancement Aspects on MLTrainingRequest IOC:  Since the exsting MLTrainingRequest IOC cannot support consumer requested ML fine-tuning, both following enhancements are possible.

- FineTuningIndication, indicates the consumer requested ML fine-tuning. The support qualifier should be CM, which means this attribute should be present when the MLTrainingRequest MOI represents the request for ML Model Fine-tuning. Besides, change the support qualifier of “aIMLInferenceName” to CM, which means that this attribute should be present when the MLTrainingRequest MOI does not represent the request for ML Model Fine-tuning.

- PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

Enhancement Aspects on MLModel IOC:  Since fine-tuning is aiming at a specific inference type for a pre-trained model, the AIML capabilities would be narrowed which has the best performance. It is proposed to introduce following attributes:

- FinetuningModelIndication, indicates that the ML Model is a Finetuning Model. Besides, change the support qualifier of “aIMLInferenceName” to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

- PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

5.1.3.5	Evaluation

The solution described in clause 5.1.3.4 proposes to add “FineTuningIndication” and “PotentialInferenceScope” to the MLTrainingRequest IOC, and add “FinetuningModelIndication” and “PotentialInferenceScope” to the MLModel IOC. Only simple enhancement is introduced to support fine-tuning, which is feasible to be developed further in the normative specifications.

End of First change

Start of Second change

6	Conclusions and recommendations

For the development of Rel-19 normative specifications, it is recommended to

- specify information models for enabling knowledge-based transfer learning according to the solution in clause 5.1.1.4.

- specify information models for coordination of AI/ML Inference as described in the solution in clause 5.5.1.4.

- specify information models for orchestrating AI/ML Inference as described in the solution in clause 5.5.1.4

- specify information models for managing the progression of inference emulation according to the solution in clause 5.3.1.4.

6.1	ML model training

6.1.X	ML Fine-tuning

For the development of Rel-19 normative specifications, it is recommended to enhance the existing MLTrainingRequest IOC and MLModel IOC to support fine-tuning as the solution proposed in 5.1.3.4.

6.4	AI/ML deployment

6.4.X	Managing ML Model Transfer in RAN

End of Second change