3GPP TSG-SA5 Meeting #158	S5-246667

Orlando, USA, 18 - 22 November 2024

# Source:		Nokia

# Title:	Rel-19 pCR TR 28.880 Add use case on cell proximity-based energy saving

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TR 28.880: " Study on energy efficiency and energy saving aspects of 5G networks and services"

# 3	Rationale

This contribution proposes to add a new use case on cell proximity based energy saving.

The inference function has been introduced in TS28.105 and it is agreed that Energy Saving is one use case for which for AI/ML inference functions may be instantiated. This pCR adds the configurable features and requirements for AI/ML-based Energy Saving Functions.

This is key issue related to the following objective that is described in FS_Energy_OAM_Ph3.

- WT-1.3 Study new use cases, requirements and solutions for energy efficiency and energy saving, applying to NG-RAN and/or 5GC and/or network slicing, (including but not limited to intent based, analytics based and AI/ML assisted energy saving)

# 4	Detailed proposal

The following changes are proposed for TR 28.880[1].

1st Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CE	Carbon Emission

CEE	Carbon Emission Efficiency

CEF	Carbon Emission Factor

CPC	Cell Proximity Coupling

GST 	Generic Slice Template

NEST	Network Slice Type

P-NEST 	Private NEST

REF	Renewable Energy Factor

S-NEST 	Standardized NEST

Next Change

## 5.A	Use case #<A>: Cell proximity-based energy saving

### 5.A.1	Description

#### 5.A.1.1	Overview

TS 28.310 defines a capacity booster cell which can be deactivated when the load is low. For a capacity booster cell to be deactivated, a coverage cell should have adequate overlap with the booster cell to make sure that deactivation of the booster cell does not lead to a coverage hole. A coverage cell may have several capacity booster cells that can be deactivated. The energy saving function needs to select the best among the capacity booster cells to be deactivated at a given time. The energy saving function should be enabled to support the selection for and sequencing of deactivation of the capacity booster cells.

#### 5.A.1.2	Cell proximity

To activate or deactivate cells, an energy saving function (be it the Domain-centralized ES or the distributed ES function, see TS 28.310 [12]) computes the degree to which the coverage of cells overlap among each other to ensure that the coverage of a deactivated cell (the capacity booster cell) is compensated by another cell (called the coverage cell). The degree of to which the cells overlap with each other can be termed as the Cell Proximity Coupling (CPC) and measures the degree to which a capacity booster cell overlaps with the coverage cell. The energy saving function can compute or consumes services of an MnS producer which computes the CPC among the cells. Alternatively, the CPC can be obtained by another the Mns consumer, e.g., the operator and configured onto the energy saving function. As input, the entity computing the CPC takes 1) the cell configuration data including transmit powers, applied antennas characteristics and tilts; 2) the propagation characteristics like building information; and 3) performance information like cell handover statistics.

#### 5.A.1.3	Cell grouping and categorization

To optimize the selection of cells to be deactivated or reactivated, the energy saving function needs to group the cells that need to be treated togther for energy saving into a group, say called the Energy Saving Cell Group. Within the Energy Saving Cell Group, the cells are categorized into two sets - the coverage cells which are not deactivated and provide primary coverage and capacity booster cells which only enhance capacity and can thus be deactivated when the load is low. The MnS producer can inform the MnS consumer (e.g., the operator or an energy saving function) about the group of cells that need to be treated together, i.e., the the Energy Saving Cell Group and the categorization of cells either as a coverage or a capacity booster cell., Where the energy saving function does not have the capability to compute the Energy Saving Cell Group, the MnS consumer is enabled to configure the Energy Saving Cell Group and cell categorisations to the energy saving function.

#### 5.A.1.4	Cell deactivation sequence

When selecting cells to deactivate, the energy saving function considers the load in a set of cells to then decides the most optimal cell(s) to be deactivated and the order in which they can be deactivated. Based on the degree of neighborliness among cells as well as the cell grouping and categorization as coverage or capacity booster cells, the MnS producer can compute the order in which the cells can be deactivated and reactivated. The energy saving function can be the MnS producer that informs the MnS consumer (e.g., the operator) of the cell deactivation order, e.g., by indicating each capacity booster cell’s rank in the deactivation and reativation process. The MnS consumer (e.g. an Operator) can be enabled to request for such a deactivation order. Where the energy saving function does not have the capability to compute the cell deactivation order, the MnS consumer (e.g. an AIML inference function) can be enabled to configure the cell deactivation order onto the energy saving function.

Given the cell deactivation order, the energy saving function routinely compares the load in the set of cells (the Cell Group load) to the low and high load thresholds to decide if activation or deactivation can be executed. The the Cell Group load needs to be computed for the set of active cells Moreover, the MnS consumer can be enabled to configure the activation or deactivation load thresholds.

Note: the term deactivation is used to represent a total switching off of the cell or any disabling of a part of the cell, e.g., a disabling of the cell's power amplifier.

### 5.A.2	Potential requirements

REQ-NES-01: The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with the Cell Proximity Coupling (CPC), describing the degree the coverage areas of any candidate neighbour cells overlap with each other.

REQ-NES-02:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the group of cells that need to be treated together (called the Energy Saving Cell Group) when selecting cell to be activated or deactivated.

NOTE 1: Configuration is needed only if the energy saving function cannot compute the Energy Saving Cell Groups.

REQ-NES-03:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell categories indicating coverage cells and capacity booster cells.

NOTE 2: Coverage cells are those that provide primary coverage and are not deactivated while capacity booster cells are those which only enhance capacity and can thus be deactivated when the load is low.

REQ-NES-04:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell deactivation order which is the capacity booster cell’s rank in the deactivation and reactivation process indicating the order in which the cells in a given area should be deactivated.

NOTE 3: Configuration is needed only if the energy saving function does not compute the cell deactivation order.

REQ-NES-05:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with the deactivation and reactivation load thresholds.

REQ-AIML-NES-01: The 3GPP management system should have a capability enabling an MnS producer (the MnS producer for AIML inference) to provide to an authorized MnS consumer a set of Cell Proximity Couplings (CPC) expressing the degree to which the coverage areas of any candidate neighbour cells overlap with each other.

REQ-AIML-NES-02: The 3GPP management system should have a capability enabling an MnS producer (the MnS producer for AIML inference) to provide to an authorized MnS consumer information on the group of cells that need to be treated together (called the Energy Saving Cell Group) when selecting cell to be activated or deactivated.

REQ-AIML-NES-03: The 3GPP management system should have a capability enabling an MnS producer (the MnS producer for AIML inference) to provide to an authorized MnS consumer information on the categorisation of cells indicating which among them are coverage cells or capacity booster cells.

REQ-AIML-NES-04: The 3GPP management system should have a capability enabling an MnS producer (the MnS producer for AIML inference) to provide to an authorized MnS consumer information on the cell deactivation order which is the capacity booster cell’s rank in the switch off/on process indicating the order in which the cells in a given area should be deactivated.

### 5.A.3	Potential solutions

#### 5.A.3.1	Potential solution #A: AI/ML-assisted Network Energy Saving

##### 5.A.3.1.1	Introduction

In this potential solution, the energy saving function takes 4 pieces of information to derive the point at which specific cells are deactivated for energy saving:

-	the degree of neighborliness among cells called the CPC,

-	the group of cells that should be considered together for energy saving called the Energy Saving Cell Group

-	categorisation of cells within the the Energy Saving Cell Group as either coverage cells or capacity booster cells

-	the cell deactivation order which indicates the order in which the cells in a given area should be deactivated, e.g., as a rank of each capacity booster cell in the deactivation and reactivationprocess.

An energy saving function can be configured by the MnS consumer with any of the four pieces of information while the rest may be derived internally by the energy saving function. The energy saving function may also consumer the services of one or more AI/ML inference function(s) for energy saving optimization which derives any of the four input information.

The energy saving function may employ an AIML inference function to support the selection for and sequencing of deactivation of the capacity booster cells. In particular,

- The AIML inference MnS producer can computes the CPC among the cells, allowing any MnS consumer to obtain the CPC, e.g., the operator and configure it onto the energy saving function. As input, the AIML inference function takes 1) the cell configuration data including transmit powers, applied antennas characteristics and tilts; 2) the propagation characteristics like building information; and 3) performance information like cell handover statistics.

- The AIML inference MnS producer that takes as input the CPC of the cells to compute the Energy Saving Cell Groups and categorization of cells as coverage or a capacity booster cells.

- Based on the degree of neighborliness among cells as well as the cell grouping and categorization as coverage or capacity booster cells, the energy saving function can employ an AIML inference function or consumes services of an AIML inference MnS producer to compute the order in which the cells can be deactivated and reactivated.

##### 5.A.3.1.2	Description

This potential solution proposes the following:

1)	Introduce an IOC representing a group of cells that should be managed together for energy saving purposes. It can be a generic grouping IOC (e.g., GenericCollection) or a specific IOC for an energy saving purpose called EnergySavingCellGroup (derived from GenericCollection IOC).

-	The group can be configured by the operator. Alternatively the group can be learned by an AI/ML function utilizing the Cell Proximity Coupling among the cells

-	Introduce attributes on the EnergySavingCellGroup IOC representing the set of coverage and capacity booster cells. They can respectively be called coverageCells and capacityBoosterCells. The cells belong to each of the two sets can be learned by an AI/ML function utilizing the Cell Proximity Coupling among the cells and the configuration and coverage information of the cells.

-	Introduce an attribute on the EnergySavingCellGroup representing the recommended cell deactivation order.

3)	Enhance the energySavingFunction IOC to add the following attributes:

- the Cell Proximity Coupling for any pair of cells

- the EnergySavingCellGroup

3)	Introduce an IOC for an AI/ML inference function for energy saving optimization

- Introduce in the inference report of the energy saving optimization AI/ML inference function:

-	an attribute for the Cell Proximity Coupling (CPC) for each gNB,

-	a list indicating the group of cells that should be considered together for energy saving

-	for each entry in the list a flag indicating the categorization of the cell as either a coverage or a capacity booster cell within that group

-	for each capacity booster cell in the list an integer indicating the rank of that cell in the cell deactivation order

- all the attributes are conditional manadatory, the AIML inference function could implement only a subset of them.

### 5.A.4	Evaluation of potential solutions

Only one solution in clause 5.A.3.1 is provided for this use case. The solution enables an AIML inference function to generate any of the four input information for energ saving while also enabling the same to be configured onto the energy saving function. It is thus a feasibe solution and normative work should be pursued following this solution.

Next change

## 6.A	Cell proximity-based energy saving

The use case, requirement and solution for Cell proximity-based energy saving is described in clause 5.A. In this use case, four input information for energ saving is described including how the energ saving may be configured with that information. it is also described how an energy saving function may rely on an AIML inference function to obtain the four input information

It is recommended to add this use case and requirement in TS 28.310 [12] and take the solution as baseline for normative work. It is also recommended to add the new attribute proposed in the solution to TS 28.541 [16].

End of change