3GPP TSG-SA5 Meeting #158	S5-246735

Orlando, USA, 18 - 22 November 2024

# Source:	Ericsson, Deutsche Telekom

# Title:	pCR TR 28.867 Clarify evaluation 5.9.4

# Document for:	Approval, Information, Discussion

Agenda Item:	6.19.4

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.867  Study on closed control loop management

[2]	3GPP TS 28.535	Management and orchestration; Management services for communication service assurance; Requirements

[3]	3GPP TS 28.536 Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3

# 3	Rationale

In the use case description for Consumers feedback on CCL Actions (section 5.9 [1]) , the producer provides the consumer with a list of actions to which the consumer may give a rating to the result of the execution of the action. The producer receives the rating and may decide to change the closed loop operation in order to improve the rating.

The study concluded with “Only one potential solution is proposed. The proposed solution satisfies all the requirements and is considered feasible.”

What is not discussed in the evaluation is the fact that proposed solution does not meet automation requirements and technical competence requirements during network operations.

- A closed loop that is in operations has autonomy to decide how to handle degradation within the scope of the configuration the closed loop has been set-up for. Any mechanism that reduces the autonomy of a closed control loop is not meeting the operator goal for automated operations.

- A closed loop in operation does not require detailed knowledge from network operations point of view, the reporting of fulfilment and potentially other information is sufficient.

- Providing ways of manipulating loop internal operation requires network operations to have detailed technical knowledge to handle the manipulation according to the producer (vendor) specifications.

The study concluded with “Only one potential solution is proposed. The proposed solution satisfies all the requirements and is considered feasible.” However, the proposed solution does not satisfy the definition of a closed control loop as documented in TS 28.535 clause 4.2.4 which is as follows:

“In a closed control loop, there is no direct involvement of a human operator or other management entity in the control loop, the control loop is fully automated.”

As described this definition is not met, however when changing to open control loop as described in TS 28.535 clause 4.2.3 the definition is met.

Therefore, it is proposed to change CCL to OCL.

# 4	Detailed proposal

First change

## 5.9	Consumers feedback on OCL actions

### 5.9.1	Description

In fully automated control loops, the CCL re-configures a particular NF to meet its stated goals without the involvement of any other entity. The actions executed by a control loop have different levels of satisfaction for the different consumers. Without a reliable means to gauge consumer Execution Satisfaction, the CCL lacks the feedback to fine-tune and optimize functionality, and so is unable to improve the overall performance. To be able to provide gauge the satisfaction, the consumer should be able to receive information about the provisioning operations executed by the control loop. This information includes operation performed, MOIs updated, etc.

Based on some local policies, the consumer may prefer that a particular NF is not updated as part of the Execution step of CCL. The consumer should be enabled to revoke the changes made to a NF. Consumer may also update the CCL to ensure that a particular NF is never updated in future. The existing attribute aCCLDisallowedList can be used, as appropriate.

Alternatively, the consumer may want to provide feedback enabling the control loop to apply an alternative approach to achieve the objectives, i.e., creating an Open Control Loop (OCL). The consumer should be able to provide its feedback on the execution indicating how satisfied the consumer is with the OCL actions.

EXAMPLE:	The consumer feedback may grade the usefulness of the executed action on a fixed scale say from 0 (indicating a terrible and never to be re-used action) to 10 (indicating a very good action for the interests of the consumer).

NOTE:	The actions that need to be provided to the consumer are decided by the producer.

### 5.9.2	Potential Requirements

REQ-FED-FUN-01: The 3GPP management system should enable consumer to provide its feedback on the action(s) taken by OCL.

REQ-FED-FUN-02: The 3GPP management system should enable consumer to request for revocation of the action(s) taken by the OCL.

REQ-FED-FUN-03: The 3GPP management system should have a capability enabling consumer to receive information (e.g. operation performed, MOIs updated) about the action(s) taken by the OCL.

5.9.3			Potential Solution

The solution involves introducing a new set of information to be maintained for each OCL. This information includes the following:

- OCL identification (only when the IOC is name-contained in SubNetwork)

- Feedback Time stamp: Time at which the feedback was provided. This is decided by the producer.

- Satisfaction Score: It indicates the numeric value ranging from 1 to 10 (1 being the worse), providing the consumer’s satisfaction for the OCL. This is provided by the consumer. This will enable producer to better decide on the actions to be taken as part of execute step.

- Execution Revoke: The action taken by the OCL may be proved to be so bad that a consumer may, optionally, request to revoke them. This is provided by the consumer.

There are several alternatives to include this information as part of network resource model definition:

Alternative 1: This information can be defined as part of a new IOC. This IOC can be name contained in  Subnetwork IOC (3GPP TS 28.622).

Alternative 2: The information can be added as part of AssuranceReport IOC in form of a new <<datatype>

Procedure flow

The procedure allows consumer to provide its feedback for a particular OCL. This functionality will enable customer satisfaction and help with fine tuning the OCL algorithm thereby improving the overall performance of the automation technique.

Figure 5.9.3-1

- The OCL gets provisioned.

- The consumer sends createMOI for OCLFeedback IOC to provide feedback on an existing OCL.

- Producer sends a response.

- Alternatively, consumer may also send modifyMOIAttribute for AssuranceReport IOC to provide feedback on an existing OCL.

- Producer sends a response.

- The producer checks if the action performed need to be revoked. The value of attribute RevokeAction will indicate if the actions are to be revoked. If yes, then it will also identify the particular action(s) to be revoked.

- The producer revokes the actions as indicated by the consumer.

### 5.9.4	Evaluation of solutions

.

Editor’s Note: the text in the evaluation is FFS

End of change