3GPP TSG-SA5 Meeting #158 	S5-246781

Orlando, Florida, 18 - 22 Nov 2024																(rev of S5-245781)

# Source:	Ericsson Inc.

# Title:	Discussion paper on scope and cleanup of FNIM content

# Document for:	Endorsement

Agenda Item:	6.3.11

# 1	Decision/action requested

This discussion paper is to present issues with the current 5G specifications references to the TMF Federated Network Information Model (FNIM) support and to seek SA5 endorsement on a potential solution to address them.

# 2	References

[1]	3GPP TS 28.620: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) Umbrella Information Model (UIM)".

[2]       3GPP TR 32.833 "Study on Management of Converged Networks".

[3]       3GPP TS 32.107 “Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM)”

[4]       3GPP TS 28.622 “	Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”

# 3	Rationale

This discussion paper identifies issues with TS 28.620 [1] related to content and referenced materials which are outdated, incorrect and/or obsolete.

This discussion paper proposes options to update the scope and subsequently remove content no longer of relevance to SA5 specifications.

## 3.1	Background

The current scope statement of [1], clause 1, states:

“As a result of the analysis of the requirements for the harmonization of the 3GPP and TM Forum Information Models in the context of FMC basic use cases were developed [6], [14]. These use cases led to the recognition that it would be necessary to define common model elements applicable for wire-line and wireless networks. This document defines these common model elements.“

The TS aims to define a Federated Network Information Model (FNIM) in support of Fixed Mobile Convergence (FMC).  A requirement based on input from [2], which noted a complete solution was not defined at that time.

The content of [1] includes references to TMF specifications which were used as input to define the Federated Network Information Model (FNIM) across TMF and SA5 specs.  In the case of TMF specifications references are included for specific artefacts from which class definitions were sourced.  In the case of SA5 the abstract classes defined in [1] are referenced and form the basis of class definitions in other SA5 specifications.  For example, [1] clause 4.2 includes:

“-	Domain_ (similar to SubNetwork of 3GPP [20] and MultiLayerSubNetwork of SID/MTOSI [7])

-	Function_ (similar to ManagedFunction of 3GPP [20] and LogicalResource of SID/MTOSI [7])

-	LayerTermination_ (similar to a single layer in the layerParameterList_T structure of SID/MTOSI [7])”

## 3.2	Issues

Issue #1:  The scope of [1] indicates that at the time of original publication is was necessary to define common model elements for wire-line and wireless networks.  It is unclear whether such requirement is still valid for 3GPP SA5.

- TR 32.833 scope did not include 5G and was published in 2012

- TS 32.107 does not include 5G;  Rel 18 version is available

Issue #2:  Numerous references to specifications in [1] have issues, including:

- [7] refers to TMF GB222 Rel 9.5 whereas the current production release is 23.5

- [8] refers to TMF MTOSI Rel 2 from 2010.  Later releases appear to be classified as ‘archived’ by TM Forum

- [11], [16] refer to TMF artefacts in an archived MTOSI release

- [15] is assumed to refer to 3GPP TS 32.107 [3], but not clearly indicated

- [7], [8], [14] have invalid links

- [17] is not used in the document

Issue #3:  The TMF solutions referenced include XML (MTOSI) and CORBA (MTNM) implementations.  The relevance of these in context of the 3GPP SBMA architecture is unclear.

Issue #4: The format of [1] includes definitions which are inconsistent with those used in other 5G specifications.     For example, the class definitions contain extra rows (as indicated below by ) in the attribute definitions.  The meaning of these additional rows and the values contained in them (e.g. “-“) are not clearly specified:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnPrefix<_<- | M | M | - | - | M |
|  |  | T | F | F | T |
| userLabel | M | M | M | - | M |
|  |  | T | T | F | T |
| userDefinedNetworkType | M | M | M | - | M |
|  |  | T | T | F | T |

This leads to confusion for developers when trying to interpret the meaning of the content in [1] as it applies to 5G since the template above is not used in the other TSes which import these definitions.

# 4	Potential Solutions

Potential solutions should prioritize the following:

- Clear, and complete, definition of the 5G Generic RAN NRM.

Additional considerations should include the following, but can be addressed in a separate activity:

- Relevance of TS 28.620 to NR and SBMA specifications.

- Potential update of TS 28.620 to align to latest TM Forum specifications.

## 4.1	Potential Solution 1:  Remove FMC content from TS 28.620

This solution would update the scope and content of [1] to focus on its current relevance and usage in 3GPP specifications.

This solution assumes the requirement to map to TMF specs in Issue #1 is no longer applicable.

The following updates would be made to address each issue:

- Issue #1:  Remove the FMC content related to TMF models from 5G versions of the specification.  Update the scope to indicate the TS defines the base classes of the 3GPP information model.  Include 5G in scope of [1] and void Annex C.

- Issue #2:  Void the references to TM Forum specs ([7], [8], [11], [14], [15], [16]).

- Issue #3:  the XML and CORBA implementations would remain, with scope clarification.

The updates above should be made in [1] from Rel16 and later.  This would align with approach taken in TS where removal of IRP and XML/CORBA solution sets applied in later releases was indicated by the Scope statement.  No updates would be required in [3].

This would allow existing references in other 3GPP specifications to continue to reference [1].  This would require no ongoing maintenance of the TMF content as it would be removed.

## 4.2	Potential Solution 2:  Update FMC content in affected TSes

This solution would update the FMC related content of all impacted 3GPP TS to clarify the relevance to latest TMF specifications.

This solution assumes the requirement to map to TMF specs in Issue #1 is still applicable.

The following updates would be made to address each issue:

- Issue #1:  Update [1] and [3] scope and content to include 5G and clarify relationship to updated TMF specs.  Update Annex C.

- Issue #2:  Update the references to TM Forum specs ([7], [8], [11], [14], [15], [16])*

- Issue #3:  references to current TMF solutions appropriate to 5G SBMA deployments may be required

Initial investigation into locating latest versions of the references in Issue #2 has identified further investigation is required to resolve potential issues including:

- GB922 is now part of TM Forum Information Framework Suite whose structure and contents have changed considerably since the references in [1].  The most relevant document now appears to be GB922 Resource v23.0.0 however it is unclear how some parts of UIM defined in [1] would or could now map to this.  Older versions of GB922 up to R16.0.1 are more closely aligned to UIM defined in [1] however these are ‘archived’ according to TM Forum.

- Specific artefacts referred to in archived MTOSI release such as [11] ‘SD1-18_layers.pdf’ and [16] ‘SD1-25_objectNaming.pdf’ need further investigation to determine appropriate replacements.

- Some analysis of [2] and applicability to 5G may be required.

The updates above should be made in [1] and [3] from Rel16 onwards.  This aligns with the release timing where the most divergence between the TM Forum and 3GPP specifications seems to have begun.

This would allow existing references in other 3GPP specifications to continue to reference [1].  This would require ongoing maintenance of the updated TMF content.

## 4.3	Potential Solution 3:  Leave FMC content for 3G/4G as-is and create new specification for 5G

This solution would retain [1] for 4G and 3G only, and require a new TS be created for 5G dependencies.  I.e. a new TS defining the base NRM upon which the 5G NRM are defined, minus the FNIM and TMF content.

This solution assumes the requirement to map to TMF specs in Issue #1 remains applicable for 3G and 4G, but not applicable to 5G.

The following updates would be made to address each issue:

- Issue #1:  Current content would be retained in [1] and scope remains 3G and 4G.  New TS would be defined for the base classes and dependencies in current 3GPP NRMs.

- Issue #2:  The new TS would not include any FNIM/UIM related TMF content or references.

- Issue #3:  The XML and CORBA implementations would no longer be referenced from 5G and SBMA specs.

The new TS would be created in Rel19.

This would require update to existing references in other 3GPP specifications to reference the new TS.  There would be no related TMF content to maintain.

## 4.4	Potential Solution 4:  Leave FMC content for 3G/4G as-is and move 5G content to another specification

This solution would retain [1] for 4G and 3G only and move the relevant information model content from [1] to another 5G TS. E.g. NR NRM.

This solution assumes the requirement to map to TMF specs in Issue #1 remains applicable for 3G and 4G, but not applicable to 5G.

The following updates would be made to address each issue:

- Issue #1:  Current content would be retained in [1] and scope remains 3G and 4G.  Some content from [1], at minimum the base class definitions, would need to be moved to another TS.  Any dependencies in current 5G NRMs which refer to [1] would need to be updated to refer to the spec(s) into which the content was moved.

- Issue #2:  The updated TS would not include any FNIM/UIM related TMF content or references.

- Issue #3:  The XML and CORBA implementations would no longer be referenced from 5G and SBMA specs.

The updates above should be made in Rel16 onwards.

This would require update to existing references in other 3GPP specifications to reference the TS into which the content is moved.  There would be no related TMF content to maintain.

## 4.5	Potential Solution 5:  Move 5G content to TS 28.622

This solution would retain existing content in [1] for 4G and 3G and copy the relevant information model content from [1] to 5G TS Generic NR NRM [4]

This solution assumes the requirement to map SA5 NRM to the TMF specs in Issue #1 remains applicable for 3G and 4G only.

The following updates would be made to address each issue:

- Issue #1:  Current content would be retained in [1] and scope remains 3G and 4G.    The content required to fulfil the 5G and SBMA references would be moved to TS 28.622, Generic NR NRM [4].

- Issue #2:  The updated TS 28.622 [4] would not include any FNIM/UIM related TMF content or references.

- Issue #3:  The XML and CORBA implementations would no longer be referenced from 5G and SBMA specs.

- Issue #4:  The inconsistent content would no longer be imported from the non-5G specifications.  New content in 5G specs would follow the same template/content as current Generic NR NRM.

This would provide a complete, and consistent, definition of the Generic Network Resource Model [4] applicable to 5G development.

The updates above should be made in Rel16 onwards.

This would require update to existing references in other 3GPP specifications to reference [4], the TS into which the relevant content is moved.  There would be no related TMF content to maintain.

The decision on whether to update [1] to align to current TM Forum specifications would be handled separately.  This should include discussion with TM Forum to determine whether the requirement to apply this specification to NR is valid.

# 5	Detailed proposal

SA5 is asked to endorse Potential Solution #5 in clause 4.5.