3GPP TSG-SA5 Meeting #158	S5-246794

Orlando, FL, U.S.A., 18 - 22 November 2024

# Source:	Ericsson

# Title:	Withdraw incorrect contributions on RACH optimisation

# Document for:	Approval

Agenda Item:	6.3.14

# 1	Decision/action requested

Approve this contribution.

# 2	References

[1]	S5-245310, Rel-16 CR TS 28.541 Align allowed value for ueAccProbabilityDist

[2]	S5-245311, Rel-17 CR TS 28.541 Align allowed value for ueAccProbabilityDist

[3]	S5-235312, Rel-18 CR TS 28.541 Align allowed value for ueAccProbabilityDist

[4]	S5-235313, Rel-19 CR TS 28.541 Align allowed value for ueAccProbabilityDist

[5]	3GPP TS 28.628, Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[6]	3GPP TS 28.313, Self-Organizing Networks (SON) for 5G networks

[7]	3GPP TS 38.331, NR; Radio Resource Control (RRC); Protocol specification

[8]	3GPP TS 28.552, Management and orchestration; 5G performance measurements

[9]	3GPP TS 28.541, Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

## 3	Rationale

During SA5#157, Hyderabad, Contribution [1] with mirrors [2,3,4] were agreed. This was unfortunate, as they are incorrect and should not be implemented. This contribution attempts to explain why they are incorrect and asks for them to be withdrawn.

These contributions deal with RACH Optimization. This topic was pioneered by Ericsson and contributed to E-UTRAN for Rel-10. The solution was then carried over to NR with no functional change.

## 3.1	Background (can be skipped)

The Distributed RACH (DRACH) optimisation function was documented for E-UTRAN in TS 28.628 [5], clause 4.6. This specification explains that:

### 4.6.1 Objective and targets The objective of RACH Optimization is to automatically set several parameters related to the performance of RACH. One of the following targets shall be used. The specific target value shall be configured by operators. Target Name; Definition; Legal Values. Access Probability, AP; The probability that the UE has access after a certain random access attempt number. The target is met if the actual access probability is higher than the target probability value.; CDF of access attempts. See section 5.5.1. Access Delay Probability, ADP; The probability distribution of Access Delay expected to be experienced by UEs accessing the RACH Channel. The target is met if the actual access probability is higher than the target probability value.; CDF of delays. See section 5.5.1. ### 4.6.2 Parameters to be optimized To achieve RACH optimization target, RACH optimization function may optimize several parameters defined in TS 36.300 [14] section 22.4.3.

A similar but less thorough explanation exists for NR, in TS 28.313 [6] clause 7.1.1.

The TS 28.628 clause 4.6.1 above means that the target for optimisation is a distribution of the access probability, or the access delay probability. Graphically, the operator may ideally configure a wanted distribution in a Cumulative Distribution Function (CDF) like this:

200

1

100%

0%

probability

number of preambles

Figure 1 Continuous CDF

As a continuous CDF is difficult to implement, we chose a more implementation friendly discreet CDF, with four specified probability values, 25%, 50%, 75% and 90%. For NR, see TS 28.541, clause 4.4.1, ueAccProbabilityDist. The CDF would then look like this:

200

1

100%

0%

probability

number of preambles

25%

50%

75%

90%

Figure 2 Discreet CDF

This target was chosen to match corresponding measurements, RACH.PreambleDist.Bin, see TS 28.552 [8] clause 5.1.1.20.3. If the bins in this measurement are chosen wisely, it will be possible for a RACH optimisation function to compare the configured, wanted distribution with the measured distribution.

Furthermore, TS 28.628, clause 4.6.1 (see above) says “The objective of RACH Optimization is to automatically set several parameters related to the performance of RACH…” One of the RACH related parameters that the optimisation function may configure (by the gNB) to the UE is preambleTransMax, see TS 28.331, clause 6.3.2. Here, it is important to understand that preambleTransMax in TS 28.331 is configured as an output of the DRACH optimisation function, while the targetProbability and numberOfPreamblesSent in TS 28.541 [9] are input to the DRACH optimisation function.

An example implementation is depicted in Figure 3 below. The operator configures a wanted value in the NRM. The actual value is received from the UE and added to a PM counter. The DRACH optimisation function configures the UE with RRC Information Elements (IE) for the RACH procedure in an implementation dependant fashion. In this example, the RRC IE preambleTransMax is configured.

gNB

PM(RACH.PreambleDist.Bin)

RRC(preambleTransMax)

NRM(targetProbability,
numberOfPreamblesSent)

DRACH optimisation function

UE

Figure 3 Example implementation of DRACH optimisation function.

## 3.2	Why are these contributions wrong?

Contributions [1,2,3,4] are incorrect in that they assume that the attribute numberOfPreamblesSent in TS 28.541 [9] are used to configure attribute preambleTransMax in TS 28.331 [7]. Instead, numberOfPreamblesSent is used to configure a wanted distribution of RACH preambles in a vendor implemented DRACH optimisation function. This function in turn may configure preambleTransMax in TS 28.331 [7].

## 3.3	Further considerations

The CRs [1,2,3,4] implements a non-backwards compatible change.

The CRs [1,2,3,4] proposes a functional change, and does not correct any fault.

## 3.4	What should SA5 do?

For the reasons outlined in clauses 3.2 and 3.3, SA5 should withdraw contributions [1,2,3,4].

# 4	Detailed proposal

Withdraw contributions [1,2,3,4].