3GPP TSG-SA5 Meeting #158	S5-246801

Orlando, USA, 18 - 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0641 | rev | 0 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 modify Use cases of GTP capacity performance measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support OAM input data for QoS Sustainability Analytics, SA5 added the new performance measurements and KPIs in TS 28.552 related to “DL capacity GTP” for gNB in clause 5.1 and “UL/DL capacity GTP” for UPF in clause 5.4 In SA5 154 a LS (S5-241777) was sent to SA2, to explain that “available GTP capacity” could be simply obtained by substracting the actual value of network data transmission from the maximum value of the defined GTP capacity, and therefore not requiring any new performance measurement. In SA5 155 meeting, CR(S5-243355) added the use case of GTP capacity performance measurements to better explain the purpose of related performance measurements. Even though it is certainly more difficult to obtain the Available Capacity since it varies over time and depends on multiple factors including network congestion, we believe that such additional performance measurement could be beneficial to maintain. Furthemore, this information is not so straight forward as it might seem due to the fact to easily obtain the real usage value from all links of a given shared network. We also propose to extend to Rel-19 to introduce the “Available GTP Capacity” performance measurement descritption in TS 28.552 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explain the necessity to keep the “Available GTP Capacity” performance measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Don’t support the “Available GTP Capacity” as initially requested by SA2 to fully support OAM input data for QoS Sustainability Analytics |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS/TR 28.554 ... CR 0194 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# A.124	Use case for GTP capacity performance measurements

The performance measurements in clauses 5.1(GTP capacity) and 5.4 (GTP capacity) are needed for the NWDAF to produce QoS sustainability analytics.

As described in TS 23.288, the consumer of QoS Sustainability analytics may request NWDAF analytics information regarding the QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area. To improve QoS Sustainability analytics, the NWDAF may additionally collect GTP metrics.

Therefore, the GTP capacity performance measurements are defined to support NWDAF to produce QoS sustainability analytics according to the IP-layer clause capacity and IP-layer available clause capacity definition from ITU-T Y.1540 [40] between UE, NG-RAN, and UPF at the GTP level. The definitions are harmonized with the ones proposed by IETF and described in IETF-RFC 5136 “Defining Network Capacity”.

For available GTP capacity, we believe it can be obtained by subtracting the actual value of network data transmission if known, from the maximum value of the defined GTP capacity, . ,.

Additionally, this available value may be limited by various factors such as capabilities of network equipments, network congestion, and whether the network is overloaded, and needs to be determined based on the circumstances at the time.

However, ITU-T Y.1540 states that available capacity is not easily measureable, and ususually requires support from monitoring systems with access to utilization measurement for all links. Such access is normally limited to the corresponding network operators. Addidtionnaly RFC 5136 specifies within the IP Link Usage definition that the information transmitted across the link can be generated by any source, including those sources that may not be directly attached to either side of the link. According to ITU-T Y.1540: “knowing how much IP-layer capacity is available in real-time across an IP network (congested or not) is valuable information to the network operators and to the application users.”  The RFC 5136 acknowledges also that determining how much capacity is available for use on a congested link is potentially much more useful.

End of modification