3GPP TSG-SA5 Meeting #158	S5-246822

Orlando, USA, 18 - 22 November 2024

# Source:	Ericsson

# Title:	Start and termination of service data flow triggers

# Document for:	Discussion

Agenda Item:	7.3

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TS 32.255 5G data connectivity domain charging; Stage 2

[2]	3GPP TS 32.290 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)

[3]	3GPP TS 32.291 5G system, charging service; Stage 3

[4]	3GPP TS 32.298 Charging Data Record (CDR) parameter description3

# 3	Rationale

## 3.1	Background

SMF can perform converged charging for each of the following:

-	Charging data related to PDU session

-	Charging data related to service data flows within the PDU session

Converged charging includes quota management and usage reporting, and SMF can report charging events to CDF for CDR generation.

In general, the charging of a service data flow is linked to the PDU session under which the service data flow has been activated. When a service data flow is governed by a PCC Rule indicated with "Offline" charging method, quota management is not applicable. Usage reporting is required for the service data flow without affecting the delivery.

There are four triggers for service data flows defined in TS 32.255 [1] table 5.2.1.4.1:

-	Start of the service data flow and no charging session exists (triggers an initial charging data request)

- 	Start of service data flow, in case no valid quota for this rating group (triggers an update charging data request)

-	Start of SDF additional access, in case no valid quota for this access rating group (triggers an initial charging data request)

-	Termination of service data flow - last service data flow under a given rating group (triggers an update charging data request)

There are three chargeable events for service data flows defined in TS 32.255 [1] table 5.2.1.4.2:

-	Start of the service data flow

- 	Start of SDF additional access

-	Termination of service data flow

The conditions for the “Start of the service data flow” defined in TS 32.255 [1] table 5.2.1.4.2 are:

-	If quota management is required, and valid quota for this rating group does not exist (triggers an update charging data request)

-	If service identifier level reporting is required by the PCC rule

-	If rating group level reporting is required by the PCC rule

-	If sponsored connectivity level reporting is required by the PCC rule

-	If charging resource, i.e. charging session, for the PDU session does not exist (triggers an initial charging data request)

The conditions for the “Start of SDF additional access” defined in TS 32.255 [1] table 5.2.1.4.2 are:

-	If ATSSS is supported with access differentiated rating groups, quota management is required, and valid quota for this access rating group does not exist (triggers an update charging data request)

-	If ATSSS is supported with access differentiated rating groups, service identifier level reporting is required by the PCC rule

-	If ATSSS is supported with access differentiated rating groups, rating group level reporting is required by the PCC rule

-	If ATSSS is supported with access differentiated rating groups, sponsored connectivity level reporting is required by the PCC rule

The conditions for the “Termination of service data flow” defined in TS 32.255 [1] table 5.2.1.4.2 are:

-	If service identifier level reporting is required by the PCC rule and this is the last service data flow for this combination of the rating group and service id (triggers an update charging data request)

-	If rating group level reporting is required by the PCC rule and this is the last service data flow utilizing that specific rating group (triggers an update charging data request)

-	If sponsored connectivity level reporting is required by the PCC rule and this was the last active service data flow for this combination of rating group, sponsor identity and application service provider identity (triggers an update charging data request)

There are two triggers for service data flows defined in TS 32.291 [3] table 6.1.6.3.6-1:

-	START_OF_SERVICE_DATA_FLOW

-	START_OF_SDF_ADDITIONAL_ACCESS

There are four triggers for service data flows defined in TS 32.298 [4] clause 5.2.5.2:

-	startOfServiceDataFlowNoSession

-	startOfServiceDataFlowNoValidQuota

-	startOfSDFAdditionalAccessNoValidQuota

-	terminationOfServiceDataFlow

## 3.1	Evaluation

### 3.1.1	Start of the Service data flow

The chargeable event “Start of the service data flow” correspond to “Start of the service data flow and no charging session exists” and “Start of service data flow, in case no valid quota for this rating group” triggers in TS 32.255 [1], and “START_OF_SERVICE_DATA_FLOW” trigger in TS 32.291 [3], and “startOfServiceDataFlowNoSession” and “startOfServiceDataFlowNoValidQuota” trigger in TS 32.298 [4].

The mapping of both the “Start of the service data flow and no charging session exists” and “Start of service data flow, in case no valid quota for this rating group” to “START_OF_SERVICE_DATA_FLOW” shouldn’t cause any issues, however mapping this back again to “startOfServiceDataFlowNoSession” and “startOfServiceDataFlowNoValidQuota” will be an issue since the CHF will have to infer based on other information how to do the mapping.

The fact that there are only two triggers in TS 32.298 [4] either related to no session or no valid quota means that there are no triggers defined for the conditions when new counts are started for start of a new service data flow within an existing rating group, e.g. a new service identifier, addition of sponsor, in TS 32.291 [3] the “START_OF_SERVICE_DATA_FLOW” could be reused, but this cannot be mapped to any trigger in TS 32.298 [4]. The result of this is that the usedUnitContainer for these new counts cannot include any triggers. If there will be a triggerTimestamp will be interpretation dependent since TS 32.290 [2] clause 7 states “This field holds the timestamp of the trigger.”, i.e. referring to the time of the non-existent trigger, while TS 32.255 [1] table 5.2.1.4.2 states “Start new counts with time stamps for the rating group” without stating how this is supposed to be reported.

### 3.1.2	Start of SDF additional access

The chargeable event “Start of SDF additional access” correspond to “Start of SDF additional access, in case no valid quota for this access rating group” trigger in TS 32.255 [1], “START_OF_SDF_ADDITIONAL_ACCESS” trigger in TS 32.291 [3], and “startOfSDFAdditionalAccessNoValidQuota” trigger in TS 32.298 [4].

The mapping of the “Start of SDF additional access, in case no valid quota for this access rating group” to “START_OF_SDF_ADDITIONAL_ACCESS” and back again to “startOfSDFAdditionalAccessNoValidQuota” is not an issue for the CHF.

The fact that there are only one trigger in TS 32.298 [4] related to no valid quota means that there are no triggers defined for the conditions when new counts are started for start of a new service data flow within an existing rating group, e.g. a new service identifier, addition of sponsor, in TS 32.291 [3] the “START_OF_SDF_ADDITIONAL_ACCESS” could be reused, but this cannot be mapped to any trigger in TS 32.298 [4]. The result of this is that the usedUnitContainer for these new counts cannot include any triggers. If there will be a triggerTimestamp will be interpretation dependent since TS 32.290 [2] clause 7 states “This field holds the timestamp of the trigger.”, i.e. referring to the time of the non-existent trigger, while TS 32.255 [1] table 5.2.1.4.2 states “Start new counts with time stamps for the rating group” without stating how this is supposed to be reported.

### 3.1.2	Termination of service data flow

The chargeable event “Termination of service data flow” correspond to one table 5.2.1.4.1: “Termination of service data flow - last service data flow under a given Rating Group” trigger in TS 32.255 [1], none in TS 32.291 [3], and “terminationOfServiceDataFlow” trigger in TS 32.298 [4].

The mapping of “Termination of service data flow - last service data flow under a given Rating Group” to nothing shouldn’t cause any issues, however mapping this back again to “terminationOfServiceDataFlow” will be an issue since the CHF will have to infer based on other information how to do the mapping. The “terminationOfServiceDataFlow” trigger in TS 32.298 [4] can for quota managed be inferred by the fact that there is a usedUnitContainer without any requestedUnit in a multipleUnitUsage, for non-quota managed reporting there won’t be any possibility to infer why the usage of the rating group was reported.

This means that there are no triggers for the conditions when counts are closed for “Termination of service data flow” within for an existing rating group, the result of this is that the usedUnitContainer for the closed counts cannot include any triggers. If there will be a triggerTimestamp will be interpretation dependent since TS 32.290 [2] clause 7 states “This field holds the timestamp of the trigger.”, i.e. referring to the time of the non-existent trigger, while TS 32.255 [1] table 5.2.1.4.2 states “Close the counts with time stamps” without stating how this is supposed to be reported.

# 4	Detailed proposal

The mapping between chargeable events and triggers as well as between triggers on Nchf and CDR for the SMF need to be further specified to not have interoperability issues.

One possible solution could be to:

-	add “TERMINATION_OF_SERVICE_DATA_FLOW” trigger in TS 32.291 [3]

-	add “startOfServiceDataFlow” and “startOfSDFAdditionalAccess” triggers in TS 32.298 [4]

-	deprecate “startOfServiceDataFlowNoSession”, “startOfServiceDataFlowNoValidQuota”, and “startOfSDFAdditionalAccessNoValidQuota” triggers in TS 32.298 [4]

-	describe the relationship between chargeable events and triggers in TS 32.255 [1]