3GPP TSG-SA5 Meeting #158	S5-246896

Orlando, USA, 18 - 22 November 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add conclusions and recommendations

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

The pCR proposes to add conclusions and recommendations for the exposure of management services.

# 4	Detailed proposal

It is proposed that the following changes be made to clause 6 of TR 28.879 [1].

Begin Change

# 6	Conclusions and recommendations

## 6.1 Conclusions

The study has explored existing telco exposure initiatives and decided to focus on the CAPIF exposure framework for exposing management services to external consumers.

The study identified the following use cases and derived the corresponding requirements for exposing management services through the CAPIF framework:

- Use case #1: MSEF registration into CAPIF

- Use case #2: Publishing of management services to the CCF

- Use case #3: Configuring discovery information of an external MnS consumer

- Use case #4: Authorization of the external MnS consumer to access the management service API

- Use case #5: Logging the management service API invocations to the CCF

Accordingly, the study has come to the following conclusions:

- There is a need to define the API provider domain entity to enable the exposure of management services through the CAPIF framework. In the present document, this entity is referred to as i.e., the management services exposure function (MSEF).

- All the identified use cases in clause 5.1 have impact on the 3GPP management system except for use case #3 defined in clause 5.1.3.

- The need to map management service information into CAPIF information elements.

- There is a relation between MSAC and the authorization information of the external MnS consumer at the CCF.

## 6.2 Recommendations

It is recommended to start normative work on the following:

- Add new capabilities for the 3GPP management system to support the use cases defined in clauses 5.1.1, 5.1.2, 5.1.4 and 5.1.5.

- Specification of MSEF and the supported capabilities.

End Change