3GPP TSG-SA5 Meeting #158	S5-246919

Orlando, FL, U.S.A., 18 - 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0355 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.532 correction of duplicated clauses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LG Co |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-11-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Same clause numbers are re-used between releases. In release 16 11.3.2.1.4 XML file format definition is voided but clause number is reused again with another title which means TS there are two clause 11.3.2.1.4, one is valid, and one is void. [x] and [y] in text but a relevant TS not in refernece list. Attribure definition point out wrong TS. Several not used references are listed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the voided clause, correct references. Add correct reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Two subcluases one valid and one void remains in the document which is misleading. Wrong references remain in the document. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 11.1.1.1.1, 11.1.1.2.3, 12.1.1.4.1a.4, 11.1.1.4.2, 11.3.1.3.2, 11.3.1.3.2, 11.3.2.1.2, 11.3.2.1.3.1, 11.3.2.1.3.2, 11.3.2.1.4, 12.3.2.3.1, 12.3.2.3.2 12.5.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	 Void

[3]	Void.

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	Void.

[6]	3GPP TS 28.554: "Management and orchestration ; 5G end to end Key Performance Indicators (KPI)".

[7]	 Void

[8]	 Void

[9]	 Void

[10]	 Void

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	 Void

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	 Void

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[16]	 Void

[17]	Void

[18]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[19]	Void.

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[21]	Void.

[22]	Void.

[23]	Void.

[24]	Void.

[25]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects ".

[26]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[27]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[28]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[29]	W3C REC-xml-names-19990114: "Namespaces in XML".

[30]	Void.

[31]	Void.

[32]	IETF RFC 6241 "Network Configuration Protocol (NETCONF)".

[33]	3GPP TS 32.160 " Management and orchestration; Management service template ".

[34]	IETF RFC 7950 "The YANG 1.1 Data Modeling Language".

[35]	Void

[36]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[37]	IETF RFC 7396: "JSON Merge Patch".

[38]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[39]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[40]	IETF RFC 6455: "The WebSocket Protocol".

[41]	IETF RFC 793: "Transmission Control Protocol".

[42]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[43]	Void

[44]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[45]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://github.com/onap/vnfrqts-requirements/blob/05f26fac2b941513a7d0e856b99fd8c61d688299/docs/Chapter8/ves7_1spec.rst#resource-structure.

[46]	Void

[47]	3GPP TS 32.404: "Performance Management (PM); Performance measurements; Definitions and template".

[48]	Void

[49]	IETF RFC 8040: "RESTCONF protocol".

[50]	IETF RFC 7951: " JSON Encoding of Data Modeled with YANG".

[51]	IETF RFC 6243: "With-defaults Capability for NETCONF".

[52]	IETF RFC 3339: " Date and Time on the Internet: Timestamps".

[53]	3GPP SA5 FORGE OpenAPI definitions: https://forge.3gpp.org/rep/sa5

[54]	3GPP TS 28.111: "Fault management".

[55]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security"

[x]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire"

Next change

##### 11.1.1.1.1	Description

This operation is invoked by MnS consumers to request a MnS producer to create a (single) managed object instance on the MnS producer.

The "managedObjectClass" parameter in the request specifies the class name and the "managedObjectInstance" parameter the instance name of the object to be created. Both parameters shall be included in the request.

The MnS consumer shall generate the instance name by first assigning a value to the naming attribute of the new instance, and then constructing a DN according to TS 32.300 [25].

The MnS consumer shall provide in "attributeListIn" none, some or all values for the attributes specified by the managed object class definition of the class to be created. The MnS producer shall not update attribute values or remove attribute values, that are provided in the request, before creating the object and returning the "createMOI" response.

The properties of an attribute determine if attribute values shall, shall not or may be provided in the "createMOI" request. If no value is provided and a default value is specified for the attribute, the MnS producer shall set the attribute value to the default value. For further information on attribute properties and their impact on the presence or absence of attribute values in object creation requests and MnS producer behaviour, see TS 32.156 [x], clause 5.2.1 and annex B.

When the MnS producer assigns values, that are not known to the MnS consumer, to one or more attributes for which no value is included in the "createMOI" request, the MnS producer shall include "attributeListOut" in the "createMOI" response, otherwise "attributeListOut" may be omitted.

In case of a successful operation, the object shall be created immediately upon reception of the "createMOI" request, and the "createMOI" response shall be returned immediately after the creation of the object. The MnS producer shall not wait with the creation of the object or returning the response until some other potentially long-lasting process or activity, that might be triggered by the reception of the request or the creation of the object, has completed.

Only objects, whose parent exists, can be created (directly under that parent). The MnS producer shall consider an attempt to create an object whose parent object does not exist as an error.

The model state after applying the "createMOI" request shall fulfill all model constraints such as cardinality, multiplicity, allowed values, or data types, otherwise the operation shall fail.

Note that stage 3 protocols may represent missing values for attributes, that are defined by the object class, in the "createMOI" request in different ways. For some protocols just the attribute name may be present, without an attribute value. For other protocols, the complete attribute name/value pair may be absent.

Some stage 3 protocols do not support returning "attributeListOut". In this case, the MnS producer shall not modify the attribute list provided in the request before creating the object. As specified in TS 32.156 [x], clause 5.2.1 and annex B, the MnS producer shall assign default values to attributes only after returning the "createMOI" response. Attribute value change notifications may be used to notify MnS consumers about the changes. Only default values, that have a specific definitive value may be assigned upon object creation. This is because the MnS consumer knows the MnS producer will assign this value according to TS 32.156 [x], clause 5.2.1 and annex B. Default values that are determined by the MnS producer based on standardized or proprietary selection methods are typically not known to MnS consumers.

Next change

##### 11.1.1.2.3	Output Parameters

| Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| managedObjectClass | M | ManagedEntity class | For each returned MO: The class of the MO. |
| managedObjectInstance | M | ManagedEntity DN | For each returned MO: The name of the MO. This is a full DN according to TS 32.300 [25]. |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value > | For each returned MO: A list of name/value pairs for MO. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |

Next change

###### 12.1.1.4.1a.4	Type MoiChange

Table 12.1.1.4.1a.4 -1: Definition of type MoiChange

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| op | Operation | Operation associated to the reported change ("add", "remove , "replace"). | M |
| path | string | URI path component segments specifying when appended to "href" the created, deleted or updated resource or secondary resource | M |
| insert | Insert | Indicates whether the new attribute element was added before or after the attribute element specified by "path", only valid for attributes with the property isOrdered=True. It can take the values "before" and "after". If missing, it defaults to "before". The "insert" attribute shall be supported only when changes from YANG defined NRMs are reported. For JSON defined NRMs the attribute shall not be supported. | CM |
| value | any type | New value of the created or updated resource or secondary resource. Optional old value of the deleted resource or secondary resource | M |
| oldValue | any type | Old value of the updated secondary resource | O |

The properties "op", "path" and "value" shall use the 3GPP JSON Patch format (TS 32.158 [15]) for reporting NRM changes. The "merge" operation specified by 3GPP JSON Patch is not supported in "notifyMOIChanges". The "move", "copy" and "test" operations specified by JSON Patch are not supported either.

The "oldValue" is an optional extension for "notifyMOIChanges" allowing to report also the value that the attribute had before replacing the value with the new value, that is contained in "value".

The following example notification (where JSON is expressed in YAML notation) reports an object creation

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1

value:

id: 1,

objectClass: ClassA,

attributes:

attrA: 123

attrB:

subAttrB1: ABC

subAttrB2: 56

The following example reports the deletion of that object.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1

The following example reports the addition of a new attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrC

value: xyz

The following example reports the deletion of the attribute "attrC".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrC

The following example reports a value change for the simple attribute "attrA".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

When the old value is reported as well, the notification looks like.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

oldValue: 123

The following example reports a value change for the complex attribute "attrB".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: abc

subAttrB2: 78

The previous two notifications can be combined into a single notification as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA

value: 456

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: abc

subAttrB2: 78

Note the operation "replace" has replace semantics and not merge semantics. The following notification reports the value change of the attribute field "attrB:subAttrB1" to "def" and the deletion of the attribute field "attrB:subAttrB2".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrB

value:

subAttrB1: def

The value change of the attribute field "attrA:subAttrB1" is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrA/subAttrB1

value: def

Assume "attrD" is a JSON array with simple elements, then the creation of this multi-valued attribute is reported as follows.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 1

- 2

- 3

Its deletion is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: remove

path: /ClassA=1#/attributes/attrD

The complete replacement of the array is reported by the following notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: add

path: /ClassA=1#/attributes/attrD

value:

- 11

- 21

- 31

The following example reports the second item in the array changed to "22".

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrD/1

value: 22

Note the array index of the second item is "1".

Assume now "attrE" is a JSON array with complex array items, for example.

[{subItemE1: 11, subItemD2: abc}, {subItemE1: 21, subItemE2: def}, {subItemE1: 31, subItemE2": ghi}.

A value change to

[{subItemE1: 11, subItemE2: abc}, {subItemE1: 21, subItemE2: xyz}, {subItemE1: 31, subItemE2": ghi}.

is reported by

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/1/subItemE2

value: xyz

When "subItemE2" is defined as array item key at stage 2, then "attrE" should contain a JSON map.

attrE:

11:

subItemE2: abc

21:

subItemE2: def

31:

subItemE2: ghi

The same change as above is now reported by the notification.

href: https://example.com/3gpp

...

moiChanges

- notificationId: 123456789

op: replace

path: /ClassA=1#/attributes/attrE/21/subItemD2

value: xyz

When all attributes of an object have been updated with a new value, the MnS producer may use a compact format reporting that the "attributes" container was updated completely.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "newValueAttrB" } } ] }

Note that clause 4.3 of IETF RFC 6902 [36] does not consider it as an error if an attribute value is replaced with exactly the same value. For that reason, it would not be an error if in the example above an attribute value is included in the "value" property that did not change value. A MnS producer may consider this compact format hence also for the case that not all attributes of an object have been updated.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "oldValueAttrB" } } ] }

To allow the MnS consumer to understand which attributes have been updated, the MnS producer may decide to send the following notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/3gpp", "notificationId": 123456789, "notificationType": "notifyMOICreation", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "moiChanges": [ { "notificationId": 123, "op": "replace", "path": "/ClassA=1#/attributes", "value": { "attrA": "newValueAttrA", "attrB": "oldValueAttrB" }, "oldValue": { "attrA": "oldValueAttrA", "attrB": "oldValueAttrB" } ] }

Next change

##### 11.1.1.4.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| baseObjectInstance | M | DN | The MO instance that is to be used as the starting point for the selection of managed objects to which the filter (when supplied) is to be applied. This is a full DN according to TS 32.300 [25]. |
| scopeType | O | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| scopeLevel | O | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| filter | O | See comment. | See corresponding parameter in getMOIAttributes. |

Next change

##### 11.3.1.3.2	Notification information

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | Class of the managed object, where the threshold crossing occurred. |
| objectInstance | M | ManagedEntity.objectInstance | Instance of the managed object, where the threshold crossing occurred. |
| notificationId | M | -- |  |
| notificationType | M | "notifyThresholdCrossing" |  |
| eventTime | M | -- | Time when the threshold crossing occurred. |
| systemDN | M | MnSAgent.objectInstance |  |
| observedPerfMetricName | M | ThresholdMonitor.thresholdInfoList.\ performanceMetrics (see TS 28.622 [11]) | Name of the performance metric that has crossed the threshold. |
| observedPerfMetricValue | M | -- | Value of the performance metric, that has crossed the threshold, when the threshold crossing was observed |
| observedPerfMetricDirection | M | -- | Direction ("UP" or "DOWN") of the performance metric, when the threshold crossing was observed |
| thresholdValue | M | ThresholdMonitor.thresholdInfoList.\ Thresholdvalue (see TS 28.622 [11]) | Threshold value of the triggered threshold |
| hysteresis | O | ThresholdMonitor.thresholdInfoList.\ Hysteresis (see TS 28.622 [11]) | Hysteresis of the triggered threshold |
| monitorGranularityPeriod | M | ThresholdMonitor.monitorGranularityPeriod | Granularity period of the threshold monitor |
| additionalText | O | -- | Vendor specific information |

Next change

##### 11.3.2.1.2	Performance data file content description

Table 11.3.2.1.2-1 provides the content definition of a performance data file.

Table 11.3.2.1.2-1: Performance data file content description

| File content item | Description |
| --- | --- |
| measDataFile | Top-level tag indicating the file contains performance metrics. Each file includes a header ("measFileHeader"), a collection of information elements with produced performance metrics and associated meta data ("measData") and a footer ("measFileFooter"). |
| measFileHeader | File header including the file format version, information about the sending node (DN, type and vendor) and a time stamp indicating the begin of the first granularity period contained in the file ("collectionBeginTime"). |
| measData | Information element containing the DN of the common root of the measured object instances ("measObjRootDn ") included in that information element, followed by a list of information elements containing the produced performance metrics and associated meta data ("measInfo"). A "MeasDataFile" contains zero, one or more "measData" elements. |
| measFileFooter | File footer with a time stamp indicating the end of the last granularity period contained in the file ("collectionEndTime"). |
| fileFormatVersion | File format version applied by the sender as indicated by the specific format version identifier provided for each version. |
| senderName | DN of the entity, that generated and sent the file. The entity is either a managed element represented by a "ManagedElement" or a management node represented by a "ManagementNode" |
| senderType | Type of the entity, that generated and sent the file, as defined in TS 28.622 [11]. The type of a management node is "MANAGEMENT_NODE". |
| vendorName | Vendor of the the entity, that generated and sent the file. |
| collectionBeginTime | Time stamp indicating the begin of the first granularity period for which performance metrics are stored in the file. |
| measObjRootDn | DN of the measured object root. The measured object root is the first common object name-containing all objects that the metrics in one "measData" element are related to. When the metrics are produced by a managed element, the root object is the "ManagedElement" representing this managed element. When (aggregated) metrics are produced by a management node (based on input metrics from managed elements), such as metrics for sub-networks or network slices, the root object is the root "SubNetwork" of this management node. |
| measObjRootUserLabel | User label of the measured object root. |
| measObjRootSwVersion | Software version of the measured object root, allowing post-processing systems to take care of vendor specific performance metrics. It is either the software version of a managed element or of a management node. |
| measInfo | Information element added to "measData" for each expired granularity period, containing information on the produced performance metrics, starting with a time stamp ("measTimeStamp"), the granularity period ("granularityPeriod") and reporting period ("reportingPeriod") that are associated to the following performance metrics ("measValues"), for which is indicated the performance metric name, the measured or computed performance metric value and the object instance to which the performance metric is related to. |
| measInfoId | Identifier of a "measInfo". |
| jobId | Job identifier of the related "PerfMetricJob" in this "measInfo". |
| reportingPeriod | Period used for performance metric reporting in this "measInfo". Unit is seconds |
| granularityPeriod | Period used for performance metric production in a "measInfo". Unit is seconds. |
| measTimeStamp | End time of the granularity period in a "measInfo". |
| measTypes | Performance metric names in a "measInfo" |
| measValues | Performance metric values in a "measInfo". Each item in this list includes the LDN of the object the metrics are related to ("measObjLdn"), the measured or computed values of the metrics ("measResults") and a flag that indicates whether the metrics are reliable ("suspectFlag"). |
| measObjLdn | Local distinguished name (LDN) of the object the performance metrics are related to (measured object) within the scope defined by the "measObjRootDn". The concatenation of the "measObjRootDn" and the "measObjLdn" is the DN of the measured object. The "measObjLdn" is therefore empty if the "measObjRootDn" already specifies completely the DN of the measured object, which is the case for metrics associated to "ManagedElement" or the root "SubNetwork". For example, if the measured object is a "ManagedElement" representing RNC "RNC-Gbg-1", then the "measObjRootDn" may look like "DC=a1.operatorNN.com,SubNetwork=CountryNN,ManagedElement=RNC-Gbg-1" and the "measObjLdn" is empty. However, if the measured object is an "UtranCell" representing cell "Gbg-997" managed by that RNC, then the "measObjRootDn" is the same as above, i.e. "DC=a1.companyNN.com,SubNetwork=CountryNN,ManagedElement=RNC-Gbg-1" and the "measObjLdn" is "RncFunction=RF-1,UtranCell=Gbg-997". The class of the measured object is defined in item f) of measurement definitions (TS 32.404 [47], TS 28.552 [18]) and in item d) of KPI definitions (TS 28.554 [6]). |
| measResults | List of result values for the observed or computed performance metrics. The "measResults" sequence shall have the same number of elements and follow the same order as the "measTypes" sequence. The NULL value is reserved to indicate that the performance metric is not applicable or could not be produced for the object instance. |
| suspectFlag | Reliability of the performance metrics. FALSE means the metrics are reliable, TRUE means they are not reliable. The default value is "FALSE". |
| collectionEndTime | Time stamp indicating the end of the last granularity period for which performance metrics are stored in the file. |

The representation of all timestamps in PM files shall follow the representations allowed by the ISO 8601 [20]. 
The precise format for timestamp representation shall be determined by the technology used for encoding the PM file (e.g. ASN.1, XML DTD, and XML Schema). The choice of technology should ensure that this representation is derived from ISO 8601 [20]. Based on the representation used, the timestamp shall refer to either UTC time or local time or local time with offset from UTC.

Next change

##### 11.3.2.1.3	Void

Next change

##### 11.3.2.1.4	Performance data file naming convention

This clause defines a rule that shall be applied for constructing names for files containing performance data.

<Type><Startdate>.<Starttime>-[<Enddate>.]<Endtime>[_-<jobIdList>][_<UniqueIdList>][_-_<RC>]

1)	The "Type" field indicates if the file contains measurement results for single or multiple measured objects and/or granularity periods where:

-	"A" means single measured object, single granularity period (this is used when granularity period is equal to reporting period);

-	"B" indicates multiple measured objects, single granularity period (this is used when granularity period is equal to reporting period);

-	"C" signifies single measured object, multiple granularity periods (this is used when reporting period is multiples of the granularity period and will contain multiple measurement reports);

-	"D" stands for multiple measured objects, multiple granularity periods (this is used when reporting period is multiples of the granularity period and will contain multiple measurement reports).

2)	The "Startdate" field indicates the date when the granularity period began if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Startdate" contains the date when the first granularity period of the measurement results contained in the file started. The "Startdate" field is of the form YYYYMMDD, where:

-	YYYY is the year in four-digit notation;

-	MM is the month in two digit notation (01 - 12);

-	DD is the day in two-digit notation (01 - 31).

3)	The "Starttime" field indicates the time when the granularity period began if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Starttime" contains the time when the first granularity period of the measurement results contained in the file began. The "Starttime" field is of the form HHMMshhmm, where:

-	HH is the two-digit hour of the day (local time), based on 24-hour clock (00 - 23);

-	MM is the two digit minute of the hour (local time), based on 60-minutes clock (00 - 59);

-	s is the sign of the local time differential from UTC (+ or -), in case the time differential to UTC is 0 then the sign may be arbitrarily set to "+" or "-";

-	hh is the two-digit number of hours of the local time differential from UTC (00-23);

-	mm is the two digit number of minutes of the local time differential from UTC (00-59).

4)	The "Enddate" field shall only be included if the "Type" field is set to "C" or "D", i.e. measurement results for multiple granularity periods are contained in the file. It identifies the date when the last granularity period of these measurements ended, and its structure corresponds to the "Startdate" field.

5)	The "Endtime" field indicates the time when the granularity period ended if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Endtime" contains the time when the last granularity period of the measurement results contained in the file ended. Its structure corresponds to the "Starttime" field.

6)	The "UniqueIdList" field indicates the DNs of the measured objects.

7)	The "RC" field is a running count, starting with the value of "1", and shall be appended only if the filename is otherwise not unique, i.e. more than one file is generated and all other parameters of the file name are identical. Therefore it may only be used by the EM, since the described situation cannot occur with NE generated files. Note that the delimiter for this field, _-_, is an underscore character (_), followed by a minus character (-), followed by an underscore character (_).

8)	The "jobIdList" indicates the measurement job id(s) that the performance data file is associated with.

Some examples describing file-naming convention:

1)	file name:	A20000626.2315+0200-2330+0200_gNBId, 
meaning:	file produced for gNB <gNBId> on June 26, 2000, granularity period 15 minutes from 23:15 local to 23:30 local, with a time differential of +2 hours against UTC.

2)	file name:	B20021224.1700-1130-1705-1130_-job10_S-NSSAI,
meaning:	file containing results for multiple measured objects, generated for measurement job job10, produced for NSI <S-NSSAI> on December 24, 2002, granularity period 5 minutes from 17:00 local to 17:05 local, with a time differential of –11:30 hours against UTC.

3)	file name:	D20050907.1030+0000-20050909.1500+0000_SubnetworkId_-_2,
meaning:	file containing results subnetwork <SubnetworkId>, start of first granularity period 07 September 2005, 10:30 local, end of last granularity period 09 September 2005, 15:00 local, with a time differential of 0 against UTC. This is the second file for this subnetwork/granularity period combination.

4)	file name:	C20050907.1030+0000-20050909.1500+0000_gNBId,
meaning:	file produced for the gNB <gNBId>, start of first granularity period 07 September 2005, 10:30 local, end of last granularity period 09 September 2005, 15:00 local, with a time differential of 0 against UTC.

Next change

##### 12.1.3.1.1	Introduction

The YANG/Netconf based solution set is based on the TS 32.160 [33] clause 6.2 and the IETF RFC 6241 [32] including the Xpath capability.

NOTE: The clauses below omit namespaces for brevity.  In NETCONF operations namespaces are included following IETF RFC 7950 [34].

Next change

#### 12.3.2.3	Void

Next change

##### 12.5.1.1.4	Operation "reportStreamData"

The IS operation parameters are mapped to SS equivalents according to the tables 12.5.1.1.4-1 and 12.5.1.1.4-2.

Table 12.5.1.1.4-1: Mapping of IS operation input parameters to SS equivalents (WebSocket Data frame sent with Opcode of 0x2)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| connectionId | n/a | -- | n/a | n/a |
| -- | Opcode (see clause 5 of IETF RFC 6455 [40]) | -- | Constant value: 0x2 ("binary") | M |
| streamingData | Payload data | Streaming Trace Payload or streaming performance data payload or streaming analytics payload or proprietary data payload | See clause 5 of TS 32.423 [39] for detailed definition of the Streaming Trace Payload format and Annex G of TS 28.550 [42] for detailed definition of the streaming performance data payload format. | M |

The protocol stack with Streaming Trace Payloads formatted as per clause 5 of TS 32.423 [39] carried by WebSocket binary data frames (see clause 5.6 of IETF RFC 6455 [40]) is illustrated on Figure 12.5.1.1.4-1.

The protocol stack with streaming performance data payloads formatted as per Annex G of TS 28.550 [42] carried by WebSocket binary data frames (see clause 5.6 of IETF RFC 6455 [40]) is illustrated on Figure 12.5.1.1.4-2.

Table 12.5.1.1.4-2: Mapping of IS operation output parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| status | n/a | -- See Note 1. | n/a | n/a |
| NOTE 1: The delivery of WebSocket Data frame is taken care of by the underlying TCP (see IETF RFC 793 [41]) which provides reliable data transmission and ensures the data delivery. There is no mechanism at WebSocket protocol level to report the delivery status for WebSocket Data frame. |  |  |  |  |

Figure 12.5.1.1.4-1: Protocol stack for streaming trace data reporting

Figure 12.5.1.1.4-2: Protocol stack for streaming performance data reporting

End of changes