3GPP TSG SA WG5 Meeting #156	S5-244513

Maastricht, The Netherlands 19 - 23 August 2024

- Source:	Huawei

- Title:	Introduce the background of CAPIF charging

- Document for:	Approval

Agenda Item:	7.5.2

# 1 	Discussion

This is a pCR to TR 28.849.

# 2	References

[1]	3GPP TR 28.849: "Study on charging aspects of Common API Framework for Northbound APIs (CAPIF) phase 2".

# 3	Rationale

This pCR proposes to introduce the background of CAPIF Charging.

NOTE: The content of this pCR was endorsed during SA5#155 in S5-243013 as a discussion paper.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.849 [1].

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[4]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

End of changes

# 4	Concepts and background

## 4.1	General Description

The Common API framework (CAPIF) architecture is defined as service-based and interactions between the CAPIF functions are represented in two ways:

-	A service-based representation, where CAPIF functions enable other authorized CAPIF functions to access their services;

-	A reference point representation, where interactions between any two CAPIF functions (e.g. CCF, AEF) is shown by an appropriate point-to-point reference point (e.g. CAPIF-3).

The CAPIF functional architecture can be adopted by any 3GPP functionality providing 3GPP northbound service APIs.

Figure 4.0-1 shows the reference point based functional model for the CAPIF.

Figure 4.0-1: Functional model for the CAPIF

## 4.2	Background

### 4.2.1	API Exposure

According to the CAPIF relationship with network exposure aspects of 3GPP systems in the clause Annex B described in the TS 23.222 [2], the details of SCEF which is the role in exposing network capabilities of EPS to 3rd party applications and NEF which is the role in exposing network capabilities of EPS to 3rd party applications are present in the following clause:

-	B.1 	CAPIF relationship with 3GPP EPS network exposure

-	B.2	 	CAPIF relationship with 3GPP 5GS network exposure

According to the CAPIF role in charging in the clause Annex C described in the TS 23.222 [2], the common architecture for charging, including the offline charging and online charging, which is the role of CAPIF in charging service API invocations, is introduced in the figure Figure C.1-1 and following clauses in the TS 23.222 [2]:

-	C.2 	CAPIF role in online charging

-	C.3 	CAPIF role in offline charging

According to the high-level Service Exposure Function architecture described in the TS 32.254 [3], the offline, online and converged charging description for Northbound Application Program Interfaces (API), based on the transaction over T8 reference point between SCEF and SCS/AS and Network Exposure Function (NEF) are introduced:

-	Clause 4.2 and 4.3: the details of SCEF functioning as the CTF, may generate accounting metrics sets for Northbound Application Program Interfaces (APIs) CDRs in the figure 4.2.1, and the details of SCEF utilizing Ro interface and application towards the OCS in the figure 4.2.2 are described:

-	Clause 4.4: the details of NEF functioning as the CTF, using Nchf to perform converged charging for the northbound API access.

## 4.3	Business Roles related to CAPIF charging

### 4.3.1	Basic CAPIF business relationships

As per the TS 23.222 [2] clause 5.1, the basic business roles related to CAPIF charging include:

Figure 4.2.1-1: Business relationships in CAPIF

-	API invoker: typically provided by a 3rd party application provider, which is the charged party for CAPIF API invocation charging by CAPIF provider.

-	CAPIF provider: the charging party, which embeds the CTF to perform the charging to API invoker.

### 4.3.2	CAPIF business relationships for RNAA

As per the TS 23.222 [2] clause 5.2, the business roles related CAPIF for RNAA charging include:

Figure 4.2.2-1: CAPIF business relationships for RNAA

-	API invoker: the same as the basic CAPIF business relationships in clause 4.2.1.

-	CAPIF provider: the same as the basic CAPIF business relationships in clause 4.2.1.

## 4.4	Charging aspect requirements by SA6

As per the charging aspect requirements specified in TS  23.222 [2], the requirement related CAPIF Charging are present as following:

-	 [AR-4.4.2-a] The CAPIF shall support charging for service APIs usage.

-	 [AR-4.4.2-b] The CAPIF shall provide mechanisms to record the usage (e.g. invocation count) of the service APIs for charging purpose, on a per API invoker basis.

-	 [AR-4.4.2-c] The CAPIF shall provide mechanisms to record timestamp of the service API invocation.

-	 [AR-4.4.2-d] The CAPIF shall provide mechanisms to record the service API related information, e.g. API location.

-	 [AR-4.4.3-a] The CAPIF shall support online and offline charging for 3rd party API providers' service APIs -usage.

-	 [AR-4.4.3-b] The CAPIF shall provide mechanisms to query charging related information of the 3rd party service APIs by the authorized users.

The above charging requirements are defined by SA6, which need to be studied from SA5 charging aspect.

End of change