3GPP TSG SA WG5 Meeting #156	S5-244514

Maastricht, The Netherlands 19 - 23 August 2024

- Source:	Huawei, Nokia

- Title:	Addition of use cases for the API operation and management

- Document for:	Approval

Agenda Item:	7.5.2

# 1 	Discussion

This is a pCR to TR 28.849.

# 2	References

[1]	3GPP TR 28.849: " Study on Charging Aspects for CAPIF Phase2 ".

# 3	Rationale

This pCR proposes to introduce the Use Cases for the topic CAPIF Converged Charging support of Service APIs Operation and Management.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.849 [1].

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AEF	API Exposing Function

CAPIF	Common API Framework

CCF	CAPIF Core Function

Next change

## 5.1	Topic #1 CAPIF Converged Charging support of Service APIs Operation and Management

### 5.1.1	General description and assumptions

Informative Annex B.2.2 of 3GPP TS 23.222 [2] describes on how the NEF can implement the specific service aspect and how its aligned with the CAPIF Architecture. Therefore, it would be necessary to evaluate the possibility to support Converged charging in those deployment models.

### 5.1.2	Use Case

#### 5.1.2.1	Use Case #1.1: Service API Operation and management via CAPIF

The service API invokers (e.g. the 3rd party application provider) and the CAPIF providers (i.e. Operators) has the service agreement about the API invocations. The CAPIF providers and API providers has the Service API arrangement about the API publishing, API exposure and API management.

The API invoker can perform one or multiple service API invocation to CAPIF. The CAPIF Core Function acknowledges the API invocation request and provide the API invocation response to the API invoker. CAPIF can collect the charging information based on the following chargeable events, for example:

-	the service API publish, as specified in the TS 23.222 [2] clause 8.3

-	the service API unpublish, as specified in the TS 23.222 [2] clause 8.4

-	the service API retrieve, as specified in the TS 23.222 [2] clause 8.5

-	the service API update, as specified in the TS 23.222 [2] clause 8.6

The charging party: the CAPIF providers

The charged party: the API provider

The potential charging requirements for this UC are: REQ-3GPPCH-APIM-01.

5.1.3	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 23.222 [2].

REQ-3GPPCH-APIM-01: The CAPIF should support converged charging for APIs service management per API provider.

### 5.1.4	Key Issues

#### 5.1.4.1	Key issue #1.1: Chargeable events and charging information required

This key issue is for investigating how to support the charging considering REQ-3GPPCH-APIM -01. This investigation covers the following:

-	identification and classification of the charging information for APIs service Operation and Management;

-	identification and classification of the chargeable event for APIs service Operation and Management.

End of change