3GPP TSG SA WG5 Meeting #156	S5-244527

Maastricht, The Netherlands 19 - 23 August 2024

- Source:	Huawei, China Mobile

- Title:	New solution on the charging for Services Exposure to the USS

- Document for:	Approval

Agenda Item:	7.5.4

# 1 	Discussion

This is a pCR to TR 28.853.

# 2	References

[1]	3GPP TR 28.853: " Study on charging aspects of uncrewed aerial systems ".

# 3	Rationale

This pCR proposes to introduce the charging solution about the service exposure to the USS charging.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

### 5.X.4	Solutions

#### 5.X.4.1	Solution #X.1: API invocation via UAS NF/NEF

##### 5.X.4.1.1	General description

This solution#1.1 which relying on CHF/5G Converged Charging System for UAS NF/NEF Charging, addresses the Key Issue#1.1.

##### 5.X.4.1.2	Architecture description

Figure 5.X.4.1.2-1 shows the 5G System high level charging architecture for UAS NF/NEF Charging, in the Service-based interfaces for non-roaming:

Figure 5.1.4.1.2-1: UAS NF/NEF converged charging architecture non-roaming

##### 5.X.4.1.3	Procedures description

The figure 5.X.4.1.3-1 describes the high-level charging procedure for UAS NF/NEF charging for service invocation about UAV Tracking, based on the figure 5.3.2-1 UAV Location Reporting, Figure 5.3.3-1 UAV Presence Monitoring and Figure 5.3.4-1 List of Aerial UEs in a geographic area, specifed in the TS 23.256 [X]. The event-based charging. i.e. IEC, PEC and ECUR specified in TS 32.290 [X] are supported.

Figure 5.1.4.1.3-1: Charging Procedure for service invocation for the UAS-SP (PEC)

- USS to UAS NF/NEF: The USS/TPAE sends the service request to UAS NF.

-	For immediate location reporting request, USS (i.e. the UAS-SP) should include an indication of reliable UE location information required in the request.

-	For UAV presence monitoring request, USS subscribes the target UAV presence events from 3GPP network (e.g. moving in or out of the monitoring area) and should include the GPSI corresponding to the target UAV for the presence monitoring, the geographic area info (e.g. longitude/latitude, zip code, etc.), an indication of reliable UE location information required in the request.

-	For obtaining UAV list request, USS request the UAV identity (e.g. 3GPP UAV ID, CAA Level UAV ID). And should include geographic area info, an indication of reliable UE location information required and indication for immediate reporting in the request.

2.	UAS NF/NEF determines the relevant NF, e.g. AMF/MME or GMLC for location reporting based on the UAV's capability or network capability, location accuracy, geographic area info etc.

3. UAS NF/NEF receives the UE location, UAV presence monitoring report or UAV location from AMF/MME or GMLC.

4. UAS NF/NEF responds to USS and sends the notifications/responses about the UAV location UAV presence in the geographic area, or USS/TPAE with the list of filtered UAVs.

4cha.	UAS NF/NEF sends the Charging Data Request [Event] to CHF for the service response/service notification.

4chb.	The CHF performs the Account, Rating Control procedure and based on policies, the CHF creates a CDR related to the service response/notification.

4chc.	The CHF grants authorization to UAS NF/NEF for the response/notification.

End of change