3GPP TSG-SA5 Meeting #156	S5-244699 (rev of S5-244166)

- Maastricht, Netherlands, 19 - 23 August 2024

- Source:	NTT DOCOMO, China Mobile, Huawei

- Title:	pCR 28.869 Terminology alignment

- Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

The group is asked to discuss and approve.

# 2	References

[1]	3GPP TR 28.869 v0.3.0 Study on cloud aspects of management and orchestration

# 3	Rationale

This contribution adds definitions for the terms used related to Network Functions in cloud native environments and clarifies how the different terms are used in the context of SA5 specifications.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change - void

2nd Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

LCM	Life Cycle Management

CNF	As defined in ETSI GR NFV 003 [11]

NOTE:	This abbreviation is defined outside 3GPP and used throughout the present document due to its relevance to the present study.

3rd Change

# 4	Concepts and background

Editor's Note: This clause introduces the concepts and the background information including the relevant work done by other SDOs or industry organizations.

## 4.2	Terminology considerations

### 4.2.1	Terminology and concepts used in this report

In the present document, the acronym CNF is used for referring to a software implementation which realizes all or part of an NF, and that adheres to cloud-native design principles. Examples of cloud-native principles and potential impacts for 3GPP management system and OAM aspects are introduced in clause 4.X.

Table 4.2.1-1: Possible cloud native impacts to 3GPP management

A CNF is not limited by its form of deployment. For example, VM-based NFs (i.e., VNFs) or container-based NFs can both qualify as CNFs if cloud-native principles are adhered. In the present document the focus of the study is to investigate technical aspects related to the management and orchestration of CNFs which might result in potential impacts to 3GPP management systems and/or related management procedures when it comes to managing 3GPP-defined NFs deployed in cloudified environments.

End of Changes