3GPP TSG-SA5 Meeting #156 	S5-244703

Maastricht, Netherland, 19 - 23 August 2024

- Source:	Ericsson

- Title:	pCR TR 28.875 new use cases mobile IAB node needs new configuration in mobility scenarios

- Document for:	Approval

Agenda Item:	6.19.16

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.875 v0.2.0 Study on management of IAB nodes

# 3	Rationale

This contribution adds new use cases to TR 28.875 [1] about IAB node needs new configuration in mobility scenarios.

# 4	Detailed proposal

1st Change

- 3	Definitions of terms, symbols and abbreviations

- 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

- 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

- 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CM	Configuration management

IAB	Integrated access and backhaul

MBSR	Mobile Base Station relay

NE	Network element

PM	Performance management

PnC	Plug and Connect

HO	Handover

# 5 	Potential use cases and requirements to support IAB node management

## 5.x	Use case #x Mobile IAB node needs new configuration in IAB-DU inter-CU migration

### 5.x.1	Description

### Mobile IAB node may need to use new configuration in IAB-DU inter-CU migration. For example, mobile IAB node needs new NR cell configuration based on its location while moving.

This use case takes mobile IAB-DU inter-CU migration scenario to study the CM impact. In this scenario, both OAM connectivity and F1 link are changed. Before IAB-DU migration, IAB-node1 is in geographical area1 and IAB-node2 is in geographical area2 and each is configured with set of NR cells. When IAB-node1 moves to a new geographical area2, it connects to new OAM management system and set up a new F1 link towards target IAB-donor-CU, such procedure refers to TS 38.401 [3] clause 8.23.13. IAB-node1 needs new NR cell configuration when move to geographical area2. See Figure 5.x.1-1 below for the simplified view of this scenario:

Figure 5.x.1-1 Mobile IAB-DU inter-CU migration

5.x.2	Potential requirements

REQ_IAB_MOB_CM_1: 3GPP management system should have the capability to support configuration based on mobile IAB node location.

REQ_IAB_MOB_CM_2: 3GPP management system should have the capability to receive notification from IAB node about its location enable mobile IAB node to notify the OAM system about its location.

REQ_IAB_MOB_CM_3: 3GPP management system should have the capability to update mobile IAB node configuration based on mobile IAB node location.

## 5.y	Use case #y Mobile IAB node needs new configuration in IAB-MT HO F1 changed

### 5.y.1	Description

### Mobile IAB node may need to use new configuration in mobile IAB-MT HO between different IAB-donor-CU. For example, mobile IAB node needs new NR cell configuration based on its location while moving.

This use case takes mobile IAB-MT HO scenario to study the CM impact. In this scenario, mobile IAB-MT HO between different IAB-donor-CU, F1 link is changed. Before IAB-MT HO, IAB-node1 is in geographical area1 and IAB-node2 is in geographical area2 and each is configured with set of NR cells. When IAB-node1 moves to a new geographical area2, IAB-node1 OAM connectivity is unchanged, it sets up a new F1 link towards target IAB-donor-CU, such procedure refers to TS 38.401 [3] clause 8.23.1. IAB-node1 needs new NR cell configuration when move to geographical area2. See Figure 5.x.1-2 below for the simplified view of this scenario:

Figure 5.y.1-1 Mobile IAB-MT HO between different IAB-donor-CU, F1 changed

### 5.y.2	Potential requirements

The requirements are same as use case 5.x in 5.x.2.

## 5.z	Use case #y Mobile IAB node needs new configuration in IAB-MT HO F1 unchanged

### 5.z.1	Description

Mobile IAB node may need to use new configuration in mobile IAB-MT HO between different IAB-donor-CU. For example, mobile IAB node needs new NR cell configuration based on its location while moving.

This use case takes mobile IAB-MT HO scenario to study the CM impact. In this scenario, mobile IAB-MT HO between different IAB-donor-CU but IAB-DU remain same F1 link with source IAB-donor-CU. Before IAB-MT HO, IAB-node1 in geographical area1 and IAB-node2 is in geographical area2 and each is configured with set of NR cells. When IAB-node1 moves to a new geographical area2, IAB-node1 OAM connectivity is unchanged, F1 link with source IAB-donor-CU is unchanged, such procedure refers to TS 38.401 [3] clause 8.23.1. IAB-node1 needs new NR cell configuration when move to geographical area2. See Figure 5.x.1-1 below for the simplified view of this scenario:

Figure 5.z.1-1 Mobile IAB-MT HO between different IAB-donor-CU, F1 unchanged

### 5.z.2	Potential requirements

The requirements are same as use case 5.x in 5.x.2.

End of change