3GPP TSG-SA5 Meeting #156 	S5-244740d2

- Maastricht, NL, 19-23 August 2024

- Source:	Huawei, Deutsche Telekom, Verizon

- Title:	Potential solution for carbon emission information of non-split gNB

- Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TS 22.261: Service requirements for the 5G system; Stage 1

[2]		3GPP SP-231723: Study on energy efficiency and energy saving aspects of 5G networks and services

[3]		3GPP TR 28.880 0.3.0: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Rationale

TS 22.261 [1] clause 6.15a makes the following statements:

“

6.15a	Energy Efficiency as a Service Criteria

6.15a.1	Description

Climate change and the rising consumption of energy motivate increased energy efficiency. Energy efficiency is a strategic priority for telecom operators around the world.

Energy efficiency as a service criteria allows services to be delivered with diverse energy efficiency and energy consumption policies. Energy consumption and efficiency information and network energy states can be exposed to third parties and energy consumption can be constrained.

Energy related information can include ratio of renewable energy and carbon emission information when available. Calculation of energy related information as described in the following requirements is done by means of averaging or applying a statistical model. The requirements do not imply that some form of 'real time' monitoring is required.

”.

Amongst other study objectives in [2], WT-2.2 addresses the issue about which and how carbon emission information and renewable energy consumption information of the 5G system can be obtained:

“

- WT-2: New Rel-19 topics from other SA WGs

- WT-2.1 Consideration of energy consumption measurement/estimation at various granularities

- WT-2.2 Study whether and how mapping measured or estimated energy consumption measurements on to carbon emissions is possible. Study the estimation of carbon emissions efficiency and how to achieve carbon emissions saving in the 5G system. Additionally, whether and how renewable energy consumption information can be obtained will be studied

- WT-2.3 Study the support of different energy states of network elements and network functions. SA5 may have to provide OAM support

”.

This contribution proposes to introduce a potential solution to the use case #3 (Enable renewable energy consumption and carbon emission information reporting) in TR 28.880 [3]. This potential solution describes how to estimate the carbon emissions of non-split gNBs.

Note that this potential solution relies on how the carbon emission factor of operator’s sites is obtained. A potential solution for this is proposed in a companion contribution.

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [3].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GR NFV-IFA 029 V3.3.1 (2019-11): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[4]	ETSI GS NFV-IFA 040 V4.3.1 (2022-05): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[5]	ETSI GS NFV-IFA 027 V4.3.1 (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[6]	ETSI GS NFV-IFA 027 V5.1.1 (2024-4): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[7]		3GPP TS 22.261: Service requirements for the 5G system; Stage 1

[8]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[9]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[10]	ISO/IEC 30134-3:2016: Information technology - Data centres - Key performance indicators - Part 3: Renewable energy factor (REF)

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G"

[13]	3GPP TS 28.313: " Management and orchestration; Self-Organizing Networks (SON) for 5G networks ".

[14]                      3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[x]	ETSI EN 303 471 V1.1.1 (2019-01): "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for Network Function Virtualisation (NFV)".

Editor’s note: reference [6] is not published yet, will be published soon, the latest draft of DGS/NFV-IFA027 is available in the following location: https://docbox.etsi.org/ISG/NFV/Open/Drafts/IFA027ed451

Next Change

### 5.3.3	Potential solutions

#### 5.3.3.i	Potential solution #i: Estimation of carbon emissions of non-split gNB

##### 5.3.3.i.1	Introduction

This potential solution describes how the carbon emissions of non-split gNB can be estimated by the 3GPP management system.

##### 5.3.3.i.2	Description

###### 5.3.3.i.2.1		General

In this potential solution:

- The 3GPP management system collects Energy Consumption measurements for the non-split gNB, as those defined in TS 28.552 [d] clause 5.1.1.19.3;

- The Network Operator gathers information, from the energy supplier, about the energy consumption of the operator site (as defined in ETSI EN 303 471 [x]) where the non-split gNB resides, and provides the 3GPP management system with this information;

- The 3GPP management system can then calculate the Carbon Emission of the non-split gNB.

NOTE 1: all information collected above should correspond to the same observation period.

NOTE 2: how the Network Operator gets information about the energy consumption of the operator site from the energy supplier is out of scope of the present study.

NOTE 3: to enable the Network Operator to provision the 3GPP management system with this information requires NRM extension – this is FFS.

###### 5.3.3.i.2.2		Carbon Emission of non-split gNB

The Carbon Emission of a non-split gNB (CENonSplit gNB) over a time period can be defined as a proportion of the Carbon Emission of the operator site (CESite) where the non-split gNB resides, the proportion being given by the ratio of the Energy Consumption of the non-split gNB (ECNonSplit gNB) to the total Energy Consumption of the operator site during the same observation period. Its unit is kg CO2 / kWh.

Equation 1: Carbon Emission of a non-split gNB

, where:

- CESite is the Carbon Emission of the operator site where the non-split gNB resides. Its unit is kg CO2eq;

- ECNonSplit gNB is the Energy Consumption of the gNB; it corresponds to PEE.Energy as defined in TS 28.552 [d] clause 5.1.1.19.3. Its unit is kWh;

- ECSite represents the total Energy Consumption of the operator site where the non-split gNB resides. Its unit is kWh.

NOTE: in this potential solution, CESite and ECSite may need to be defined in TS 28.554 as new KPIs – this is FFS.

End of changes