3GPP TSG-SA5 Meeting #156 	S5-244855d1

Maastricht , NL, 19 - 23 Aug 2024

- Source:	China Unicom

- Title:	Add potential solution for S-RAN management of Indirect Network Sharing

- Document for:	Approval

Agenda Item:	6.19.19

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	S5-242035 Revised SID on Management of Network Sharing Phase 3.

[2]	S2-2405669, CR 23.501, Indirect network sharing.

[3]	3GPP TS 32.130: " Network sharing; Concepts and requirements".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

# 3	Rationale

As agreed in S5-242035[1], WT-3 is described as "Investigate potential enhancements for Indirect Network Sharing scenarios". Since S2-2405669[2] has been approved in SA2, this contribution is to add new use case and requirements on management supporting for Indirect Network Sharing.

In TS 32.130[3], 3GPP management system has already supported S-RAN management in two types of active RAN sharing scenarios, MOCN and GWCN. S-RAN of Indirect Network Sharing is supposed to be managed by 3GPP management system. This document is to propose a potential solution to support S-RAN management of Indirect Network Sharing.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

## 5.3	Use Case#3: S-RAN management of Indirect Network Sharing

### 5.3.1	Description

According to TS 22.261[2], Indirect Network Sharing is defined as follow:

"a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator’s core network is routed through the Hosting NG-RAN Operator’s core network ".

Therefore, Indirect Network Sharing is proposed to share NG-RAN indirectly through core network. Regarding 5G MOCN, NG-RAN is shared directly by the connection to each POP’s core network. According to the definition of S-RAN in TS 32.130[4], shared NG-RAN in the Indirect Network Sharing scenario might be identified as S-RAN. As a result, each POP in the Indirect Network Sharing scenario could have requirements to manage S-RAN. The hosting operator, as MOP in the Indirect Network Sharing scenario, should have capabilities to allow authorized consumers to manage S-RAN.

### 5.3.2	Potential requirements

REQ-NetShare_Req-1: The 3GPP management system of the MOP shall support a capability allowing its authorized consumer to manage S-RAN in the Indirect Network Sharing scenario.

### 5.3.3	Potential solutions

Following aspects needs to be added or enhanced in management workflows in TS 32.130[y]:

-	Add descriptions to support for management of Indirect Network Sharing as follows:

REQ-NS_GEN-CON-2		The 3GPP management system of the MOP shall support a capability allowing its authorized consumer to manage S-RAN according to any of the following scenarios:

i/ Multi-Operator Core Network

ii/ Gateway Core Network.

iii/ Indirect Network Sharing

- Add the management architecture diagram for Indirect Network Sharing

### 5.3.4 Evaluation of potential solutions

TBD

End of Changes