3GPP TSG-SA5 Meeting #156	S5-244897d1

Maastricht, Netherlands, 19 - 23 August 2024

- Source:	Huawei

- Title:	Potential solution of EE KPIs evaluated from network data traffic performance dimension

- Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.880 v0.3.0: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Rationale

This contribution proposes to add potential solution of EE KPIs evaluated from network data traffic performance dimension in TR 28.880 [1].

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

1st Change

### 5.6.3	Potential solutions

#### 5.6.3.i	Potential solution #<i>: <EE KPIs evaluated from network data traffic performance dimension>

##### 5.6.3.i.1	Introduction

In this potential solution, existing EE KPIs are considered from network data traffic performance dimension. It is no need to change the definition of the following existing EE KPIs.

##### 5.6.3.i.2	Description

Network data traffic, measured from the following performance measurements point of view

- The amount of data transferred as the Data Volume

- the number of connections including

  - the number of registered subscribers of MIoT network slice

  - the number of active UEs in MIoT the network slice

The existing EE KPIs evaluated from network data traffic performance dimension are captured in Table 5.6.3.i.2-1.

Table 5.6.3.i.2-1: EE KPIs evaluated from network data traffic performance dimension

| Performance dimension to evaluate EE KPIs | EE KPIs | Reference in TS 28.554 [2] | Note |
| --- | --- | --- | --- |
| Data Volume (DV) as the network data traffic performance indicator | NG-RAN data Energy Efficiency | 6.7.1 | A KPI that shows mobile network data energy efficiency in operational NG-RAN. |
|  | Energy efficiency of eMBB network slice | 6.7.2.2 | A KPI that shows the energy efficiency of network slices of type eMBB. |
|  | Energy efficiency of eMBB network slice – RAN-based | 6.7.2.2a | A KPI that shows the energy efficiency of network slices of type eMBB based on NR measurements. |
|  | Energy Efficiency of the 5GC (Based on the useful output of 5GC user plane) | 6.7.4.2 | A KPI that shows the energy efficiency of 5GC. This KPI is based on the useful output of 5GC user plane. |
| The number of connections supported as the network data traffic performance indicator | Energy efficiency of MIoT network slice (Based on the number of registered subscribers of the network slice) | 6.7.2.4.1 | A KPI that shows the energy efficiency of network slices of type MIoT. |
|  | Energy efficiency of MIoT network slice (Based on the number of active UEs in the network slice) | 6.7.2.4.2 | A KPI that shows the energy efficiency of network slices of type MIoT. |

End of change