3GPP TSG-SA5 Meeting #157	S5-246086

Hyderabad, India, 14 - 18 October 2024

- Source:	Nokia

- Title:	pCR 28.572 Add content for transactions

- Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.2	Planned configuration groups

### 6.2.1	Definition

A planned configuration group is a set of planned configurations that shall be processed (validated and activated) together.

For example, one plan may create a new cell, and another plan may start the collection of measurements for the new cell. Or one plan that configures one base station and another plan configures another base station, when both base stations need to be consistently configured to enable handovers between them.

### 6.2.2	Planned configuration group members

A planned configuration belonging to a planned configuration group is also called a member of that planned configuration group. A planned configuration can be a member of more than one planned configuration group.

### 6.2.3	Meta data for planned configuration groups

Each planned configuration group is described by meta data. This includes the name, the version, and a human readable textual description. The date and time at which information in the planned configuration group was modified the last time is provided as well.

### 6.2.4	Control data for planned configuration groups

Control data needs to be specified for the validation and activation of planned configuration groups. This can be done in the definition of the planned configuration group or when validating or activating the planned configuration group.

For a validation, the conflict mode is specified. For activation, the conflict mode, the desired service impact and if fallback shall be enabled are specified.

### 6.2.5	Planned configuration group descriptor

The planned configuration group members, its meta data and control parameters are specified in a planned configuration group descriptor. Each descriptor has a unique identifier.

Next modification

## 6.11	Managing planned configuration groups

Planned configuration group descriptors are created, updated and deleted by MnS consumers on MnS producers. Planned configurations referred to in a planned configuration group must exist.

Multiple planned configuration group descriptors can be maintained on a MnS producer. Each planned configuration group is identified by an identifier. The identifier is assigned by the MnS producer.

When control parameters are specified in the planned configuration group descriptor and in the validation or activation request, the latter takes precedence.

The following table specifies the information elements of the information model for planned configuration groups.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| plan-group-id | M | Identifier of the planned configuration group. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| name | M | The name of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: None |
| version | M | The version of the planned configuration group. Format is implementation specific. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| description | M | The textual human-readable description of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: None |
| plan-ids | M | The identifiers of the planned configurations that are members of the planned configuration group. | Type: String multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None |
| relationship-between-plans | M | The relationship between the planned configurations. allowedValues: - SEQUENTIAL_ORDERED - COMBINED Editor's note: The definition of the values is FFS | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| last-modified | M | The date and time at which the planned configuration group descriptor was modified the last time. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isWritable: False |
| conflict-mode | M | This attribute allows to specify, if conflicting changes in different plans shall result in a failure, or if conflicting changes shall be applied in order. allowedValues: - STOP_ON_CONFLICT - CONTINUE_ON_CONFLICT - ROLLBACK_ON_CONFLICT | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| enable-fallback | M | Specifies if the MnS producer shall create a fallback-plan o allow for a later fallback to undo the changes in the curent plan-group. If the element is missing it has the same effect as if it was False. The plan-id of the fallback-plan shall be reurned to the consumer. | Type: Boolean multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: False |
| activation-service-impact | M | The service impact mode allowedValues: - LEAST_SERVICE_IMPACT - SHORTEST_TIME | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |

End of modifications