3GPP TSG-SA WG6 Meeting #62	S6-243039

# Maastricht, The Netherlands, 19-23 August 2024

Source:	Kontron Transportation France, Nokia

Title:	Discussion paper on IP address of target data host or DNS server in MCData IPcon

Agenda item:	9.3

Document for:	DISCUSSION

- Discussion

From 3GPP TS 23.282:

## 5.11	IP connectivity (IPcon) capability

IP connectivity service enables the exchange of IP Data using MCData transport service and provides the transport of IP Data for e.g. data hosts, servers, etc. that do not have mission critical communication capabilities. The exchange of IP Data is not limited in a transaction.

The corresponding MCData client enables bidirectional IP Data communication with the support of the IP connectivity service and thus forms the gateway to data hosts or servers.

How is the tunnel defined in 3GPP in stage 3 (TS 24.282 and TS 24.582):

The tunnel is defined as a GRE-in-UDP tunnel using GRE-in-UDP encapsulation. That means that from the client and network perspective the tunnel looks like a communication over UDP between the two clients. The figures below illustrate in more detail the aspects on IP addressing. To transfer data through the tunnel, the endpoints must encapsulate/decapsulate the data coming from and going to the hosts behind the clients.

Figure 1 illustrates the end-to-end procedure for the use case to communicate to a single target server.

Figure 1: Establishment of point-to-point IP connectivity to a single target server

Simplified procedure:

1)	An application on the data host uses an API to trigger the establishment of an IPcon session from MCData client 1 to MCData client 2.

2)	MCData client 1 initiates a point-to-point IP connectivity session with MCData client 2 as defined in 3GPP TS 23.282 7 clause.7.14.2.2. The MCData ID of client 2 was provided in step 1 via the API.

3)	The IP connection between MCData client 1 and MCData client 2 is established.

4)	The application on the data host wants to initiate an IP connection to server 1 but does not know the IP address of server 1.

Proposed solution:

Enhance the MCData IPcon point-to-point response message as defined in 3GPP TS 23.282 with an optional additional information element containing the IP address of the target server.

Figure 2:illustrates the end-to-end procedure for the use case to communicate to multiple hosts in an IP network.

Figure 2: Establishment of point-to-point IP connectivity to a target network containing multiple hosts

Simplified procedure:

1)	An application on the data host uses an API to trigger the establishment of an IPcon session from MCData client 1 to MCData client 2.

2)	MCData client 1 initiates a point-to-point IP connectivity session with MCData client 2 as defined in 3GPP TS 23.282 7 clause.7.14.2.2. The MCData ID of client 2 was provided in step 1 via the API.

3)	The IP connection between MCData client 1 and MCData client 2 is established.

4)	The application on the data host wants to initiate an IP connection to a host in network 1 using the FQDN of the target host but does not know the IP address of the DNS server to query for the resolution of the FQDN of the target host.

Proposed solution:

Enhance the MCData IPcon point-to-point response message as defined in 3GPP TS 23.282 with an optional additional information element containing the IP address of the DNS server.

- Solution

Proposed change in 3GPP TS 23.282 clause 7.14.2.1.2 cover both use cases is to enhance information flow of MCData IPcon point-to-point response as follows:

Table 7.14.2.1.2-1: MCData IPcon point-to-point response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the targeted MCData user. |
| MCData ID | M | The MCData identity of the requesting MCData user. |
| SDP | O | Media parameters selected. This shall be present if the IP connectivity establishment result is successful. |
| Time Limit | O | Negotiated time (1 min – infinite) |
| IP connectivity status | M | IP connectivity establishment result |
| IP address (see NOTE) | O | IP address of a host that can be reached by this IPcon point-to-point connection |
| IP address (see NOTE) | O | IP address of the DNS server of the network that can be reached by this IPcon point-to-point connection |
| NOTE: How the MCData client gets the IP address is outside the scope of the present document. Only one of these information elements is present. |  |  |