3GPP TSG-SA WG6 Meeting #62	S6-243097

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Update key issue#1 in TR 23.700-22

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to update key issue#1 about recognizing the resource owner.

2. Reason for Change

Before resource owner consent management, the CAPIF core function is suggested to identify the resource owner and interact with it based on necessary information.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v 0.4.0

* * * First Change * * * *

5.1	Key issue #1: Managing resource owner consent

5.1.1	Description

With the introduction of support for Subscriber-aware Northbound API access in 3GPP Rel-18 a number of requirements were added at stage 1, one of which was motivated by the desire for a UE to be able to control whether or not to provide information considered private to a 3rd party entity. Specifically, the requirement is that the 5G system shall be able to allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party. At stage 2 the CAPIF-8 reference point was introduced to CAPIF where the aspect of consent was highlighted through the statement that the resource owner communicates with the authorization function in the CAPIF core function to manage resource owner consent, with such communication being expected to be performed over CAPIF-8. However, the mechanism for managing such consent was not specified with the functionalities over CAPIF-8 being is FFS and out of scope of the Rel-18 of the specification.

Besides, the mechanism for recognizing the resource owner and maintaining its information in CAPIF was not specified in Rel-18. This mechanism enables CAPIF core function to identify the resource owner based on the maintained information before the resource owner provides/revokes the resource owner consent.

5.1.2	Open issues

The CAPIF does not address management of resource owner consent in the context of supporting RNAA. The open issues are:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	How to enable retrieval of the resource owner consent parameters by an API exposure function from the authorization function.

3.	Whether (and how) “purpose of data processing” will be captured, where will it be stored and how will it be retrieved by the consent enforcement point, i.e., AEF (e.g., via CAPIF-3 in case it is available in the CCF).

4.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

5.	How to recognize a resource owner and maintain related information so that the CAPIF core function can use it to communicate with the resource owner via CAPIF-8.

NOTE:	Aspects pertaining to the definition of resource owner consent/authorization over CAPIF-8 are in the scope of SA3, noting that the R18 security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [3].

* * * Next Change * * * *