3GPP TSG-SA WG6 Meeting #62	S6-243228

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Convida Wireless LLC, KPN N.V.

Title:	AIMLE client registration

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, liu.lu@convidawireless.com

1. Introduction

The conclusion for key issue #3 in TR 23.700-82 stated that solutions #8 can be considered for normative work. The contribution introduces a procedure for AIMLE client registration.

2. Reason for Change

AI/ML capable clients need to first register with an AIMLE server in order to be able to participate in AI/ML operations such as data management, training, and inferencing. As part of the registration, the clients provide AIMLE client profile and optionally a list of supported services. This information will be used by the AIMLE server in discovering and selecting AIMLE clients for participation in AI/ML operation(s). An AIMLE client profile ID will be assigned for the AIMLE client to update information in the AIMLE client profile, e.g. availability of a new dataset and/or data capabilities. In addition, an AIMLE client identifier is assigned by the AIMLE server and returned in the response to the AIMLE client. The AIMLE client identifier serves to preserve the privacy of the AIMLE client when performing AI/ML operations.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change (all new text) * * * *

## 8.x	AIMLE client registration

### 8.x.1	General

Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. The AIMLE server assigns identifiers for the AIMLE client and the associated AIMLE client profile and store the context in a ML repository. The AIMLE client can update information in the AIMLE client profile using the assigned AIMLE client profile identifier. The AIMLE client can opt out of AI/ML participation by deregistering from the AIMLE server, at which point the AIMLE client identifier, the AIMLE client profile identifier, and the associated context are deleted.

The following clauses specify procedures, information flows, and APIs for AIMLE client registration.

### 8.x.2	Procedure

Pre-conditions:

1.	The AIMLE client has been pre-configured or has discovered the address (e.g. URI) of the AIMLE server.

2.  The AIMLE client has been pre-configured with an AIMLE client profile.

Figure 8.x.2-1: AIMLE client registration procedure

1.	The AIMLE client sends a registration request to the AIMLE server, the registration request includes information as described in Table 8.x.3.1-1.

2.	The AIMLE server validates the registration request and verifies the security credentials. The AIMLE server further performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. The AIMLE server assigns an identifier for the AIMLE client and an identifier for the AIMLE client profile. The AIMLE client profile identifier is associated with the AIMLE client. The AIMLE server saves the context of the AIMLE client registration in the ML repository.

3.	The AIMLE server returns an AIMLE registration response to the AIMLE client with the status of the registration request and the identifiers associated with the AIMLE client and AIMLE client profile.

### 8.x.3	Information flows

#### 8.x.3.1	AIMLE client registration request

Table 8.x.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration procedure.

Table 8.x.3.1-1: Request for AIMLE client registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.x.3.1-2. |
| List of supported services | O | Supported service information. |
| >List of VAL service ID(s) | O | List of VAL service ID(s) that the AIMLE client supports |
| >Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage) |

Table 8.x.3.1-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML models | O | AI/ML models supported by the AIMLE client |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as data collection, data preparation, training, and inferencing |
| Availability schedule | O | Availability schedule of the AIMLE client |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |

#### 8.x.3.2	AIMLE client registration response

Table 8.x.3.2-1 shows the response sent by the AIMLE server to the AIMLE client for the AIMLE client registration procedure.

Table 8.x.3.2-1: Response for AIMLE client registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIMLE client identifier | M | An identifier for the AIMLE client. |
| AIMLE client profile ID | M | An identifier for the AIMLE client profile. |

* * * End of Change * * * *