3GPP TSG-SA WG6 Meeting #62	S6-243291

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-242096)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0076 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interworking ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Nokia, Kontron |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ad hoc group call interworking, e.g., with GSM-R is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the ad hoc group call interworking procedures and information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ad hoc group call LMR interworking is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {10.x, 10.x.1, 10.x.2, 10.x.2.1, 10.x.2.2, 10.x.2.3, 10.x.2.4, 11.x.2.5, 10.x.3, 10.x.3.1, 10.x.3.2} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/************************ 1st changes *************************/

10.x	Ad hoc group call

10.x.1	General

The following clauses define information flows and signaling procedures for ad hoc group calls, where the ad hoc group call is established with a participants list provided by the call intitiator. The MCPTT server acts as the call control server and individually calls each LMR user within the participants list via the IWF.

10.x.2	Information flows for ad hoc group call

10.x.2.1	General

The following clauses define information flows for ad hoc group calls on the IWF-1 interface. Ad hoc group call related information flows on reference points other than IWF-1 are defined in 3GPP TS 23.379 [7].

10.x.2.2	IWF ad hoc group call request (MCPTT server – IWF)

Table 10.x.2.2-1 describes the information flow IWF ad hoc group call request from the MCPTT server to the IWF.

Table 10.x.2.2-1 IWF Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group communication |
| Preconfigured MCPTT group ID | M | Indicates pre-configured group configuration to be used for the ad hoc group call |
| MCPTT ID | M | MCPTT ID of the participant being invited for the ad hoc group call |
| SDP offer | M | Offered Media parameters |
| Implicit floor request (see NOTE 1) | O | When originating client requests the floor, this element shall be included |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| Location information | O | Location of the calling party. |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client requests the floor. NOTE 2: If used, only one of these information elements is present. |  |  |

10.x.2.3	IWF ad hoc group call response (IWF - MCPTT server)

Table 10.x.2.3-1 describes the information flow IWF ad hoc group call request from the IWF to the MCPTT server.

Table 10.x.2.3-1 IWF Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the group call request (success or failure) |

10.x.2.4	IWF ad hoc group call release request (MCPTT server – IWF)

Table 10.x.2.4-1 describes the information flow IWF ad hoc group call release request from the MCPTT server to the IWF.

Table 10.x.2.4-1 IWF ad hoc group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the user authorized to release the ad hoc group call |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call on which call is released |

10.x.2.5	IWF ad hoc group call release response (IWF - MCPTT server)

Table 10.x.2.5-1 describes the information flow IWF ad hoc group call release response from the IWF to the MCPTT server.

Table 10.x.2.5-1 IWF ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call that is released |

10.x.3	Procedures

10.x.3.1	IWF ad hoc group call initiated by an MCPTT user using a participants list

In this procedure, an MCPTT user is initiating an ad hoc group call with both MCPTT users and LMR users.

Pre-conditions:

1.	The MCPTT client 1 knows the MCPTT IDs of the participants, including both MCPTT users and LMR users, to be involved in this call.

2.	MCPTT client 1 and MCPTT client 2 are registered, and their respective users are authenticated and authorized to use the MCPTT service.

NOTE 1:	For all the signalling messages passing through the IWF between the MCPTT system and the LMR system, the IWF performs the identity conversion and protocol translation.

Figure 10.x.3.1-1: Ad hoc group call setup initiated by an MCPTT user in the MCPTT system

The procedure and information flows as defined in 3GPP TS 23.379 [7] clause 10.19.3.1.1 is applied with the following differences:

1-3.	Same as clause 10.19.3.1.1 in 3GPP TS 23.379 [7].

4a-4b.	For each of the LMR users, the MCPTT server sends the IWF ad hoc group call request to the IWF. The IWF returns the IWF ad hoc group call response to the MCPTT server.

NOTE 2:	IWF can handle the IWF ad hoc group call as a normal group call towards the LMR user in LMR system.

5.	Same as clause 10.19.3.1.1 in 3GPP TS 23.379 [7].

6.	If the initiating MCPTT user requires the acknowledgement from the invited MCPTT users, and the required MCPTT users do not acknowledge the call setup within a configured time (the "acknowledged call setup timeout"), then the MCPTT server may proceed with or abandon the call and then notify the initiating MCPTT user that the acknowledgements did not include all required members according to ad hoc group call policy from the user profile configuration. The MCPTT server may notify the initiating MCPTT user of all MCPTT users who did not acknowledge the ad hoc group call request within the configured time. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the MCPTT ad hoc group call.

NOTE 3:	For LMR users notification of not acknowledging the ad hoc group call is not possible.

7.	MCPTT client 1, MCPTT client 2, and the LMR users establish media plane and floor control resources.

10.x.3.2	IWF ad hoc group call released by the MC system

In this procedure, the ad hoc group call is released in the MC system due to the release conditions or the ad hoc group call is released by an authorized user.

Figure 10.x.3.2-1: Release IWF ad hoc group call by the MC system

The procedure and information flows as defined in 3GPP TS 23.379 [7] clause 10.19.3.1.2 and 10.19.3.1.2a are applied to the interactions between the MCPTT client and the MCPTT server. In addition, new step 2a and 2b are introduced.

1.	The IWF ad hoc group call is releasing at the MCPTT system either initiated by the server due to the release conditions are met as described in clause 10.19.3.1.2 in 3GPP TS 23.379 [7] or by an authorized user as described in clause 10.19.3.1.2a in 3GPP TS 23.379 [7].

2a-2b.	The MCPTT server sends the IWF ad hoc group call release request to the IWF (2a), and the IWF returns the IWF ad hoc group call release response (2b).

3.	MCPTT client 1, client 2 and LMR users have successfully released the floor control and media plane resources associated with the ad hoc group call that is terminated and the ad hoc group ceases to exist (i.e., further call is not possible over the same ad hoc group).