3GPP TSG-SA WG6 Meeting #62	S6-243296

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0575 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce traffic control at MC gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the MC gateway UE only provides the transparently traffic forwarding without any information of the “traffic”, several issues may occur due to its capability and the Uu resource is limited: - Unauthorized traffic may congest the Uu link of the MC gateway UE. The traffic may be rejected by the MC servers but the UL/DL Uu transmission resource of the gateway UE is wasted. - The MC gateway UE may be congested by big traffic volume (e.g., Video call) from fewer users. - One service from a single device may simultaneously use multiple MC gateway Ues, i.e., over occupied by a single UE. In order to solve the issues, it is propose to have the traffc control function at the MC gateway UE, which can: - Stop the unauthorized traffic at the MC gateway UE to protect the Uu resource from being wasted. - The MC gateway UE can be well managed to serve e.g., as many as users based on the MC organization’s different requirements. - Stop the second usage of MC gateway UE for a single MC service from a device. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - the functional model is updated with a new functional entity in the gateway UE and two reference points - New traffic control procedures are introduced accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Uu link of the MC gateway UE may be congested without well traffc control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1, 11.2.2, (new) 11.2.2.aa, (new) 11.2.2.bb, (new) 11.2.4, (new) 11.2.4.1, (new) 11.2.4.2, 11.5.2.2.1, 11.5.2.2.2, 11.5.2.2.3, 11.5.2.3, 11.5.2.3.1, 11.5.2.3.2, 11.5.2.3.3, 11.5.3.2.1, 11.5.3.2.2, 11.5.3.2.3, 11.5.3.2.4, 11.5.3.2.5, 11.5.3.2.6, 11.5.3.2.7, 11.5.3.2.8, 11.5.3.3, 11.5.3.3.1, 11.5.3.3.2, 11.5.3.3.3, 11.5.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ 1st changes *************************/

11.2.1	Functional model

The authentication and authorisation of the MC clients hosted on the MC gateway UE follows the procedures described in 3GPP TS 33.180 [25]. Upon successful authentication and authorisation the MC clients can access a MC server via an MC gateway UE.

The MC gateway UE supports the authentication and authorisation of the MC client residing at the non-3GPP device by forwarding the unmodified application layer signalling between the MC client and the corresponding MC server. The authentication and authorisation of the MC client in the non-3GPP device follows the procedures described in 3GPP TS 33.180 [25].

NOTE:	The authentication and authorisation between the MC gateway UE and the non-3GPP device is out of scope of 3GPP.

Figure 11.2.1-1 represents the functional model for the application plane for utilizing MC gateway UE to enable MC services to non-3GPP devices that can host an MC client.

Figure 11.2.1-2 represents the functional model for the signalling plane for utilizing MC gateway UE to enable MC services to non-3GPP devices that can host an MC client.

Figure 11.2.1-3 represents the functional model for the application plane for non-3GPP devices that cannot host an MC client.

Figure 11.2.1-1: Functional model of MC gateway UE application plane with MC client hosted in the non- 3GPP device

Figure 11.2.1-2: Functional model of MC gateway UE signalling plane with MC client hosted in the non- 3GPP device

Figure 11.2.1-3: Functional model of MC gateway UE application plane with MC client hosted in the MC gateway UE

The functional model for the signalling control plane as described in figure 7.3.1-2 is applicable to figure 11.2.1-3.

#### 11.2.2	Reference points

# /************************ Next changes *************************/

11.2.2.aa	Reference points GW-local-3 (between the MC service client and the MC traffic control function in the MC gateway UE)

The reference points GW-local-1 is between the MC service client hosted by the non-3GPP device and the MC traffic control function in the MC gateway UE. It supports the MC traffic control related signalling exchange.

11.2.2.bb	Reference points GW-core-1 (between the MC traffic control function in the MC gateway UE and the MC gateway UE control function in the MC service server)

The reference points GW-local-1 is between the MC traffic control function in the MC gateway UE and the MC gateway UE control function in the MC service server. It supports MC traffic control related signalling exchange.

# /************************ Next changes *************************/

11.2.4	Function entities description

11.2.4.1	Function entities description

Each subclause is a description of a functional entity and does not imply a physical entity.

11.2.4.2	MC gateway UE control function

The MC gateway UE control function is responsible for manage the traffic transmission through the MC gateway UE including authorization check the MC service user is authorized to use certain MC gateway UE to transport MC service data, informing the MC gateway UE to open the gate for certain traffic from the MC service client at the non-3GPP device.

The MC gateway UE control function is supported by the HTTP server functional entities of the signalling control plane.

11.2.4.3	MC traffic control function

The MC gateway UE control function is responsible for manage the traffic transmission through the MC gateway UE including sending the traffic connection request from the MC service client (the MC gateway UE not hosting the MC service client) or internally from the MC service client within the MC gateway UE (the MC gateway UE hosting the MC service client), receive the packet filter from the MC gateway UE control function to open the gate for such MC service traffic.

The MC traffic control function is supported by the HTTP server functional entities of the signalling control plane.

# /************************ Next changes *************************/

11.5.x	Traffic control management at the MC gateway UE

11.5.x.1	General

The MC service client on the non-3GPP client utilizes this procedure to activate transmission at the MC gateway UE.

11.5.x.2	information flows

The MC service client on the non-3GPP client utilizes this procedure to activate transmission at the MC gateway UE.

11.5.x.2.1	Connection request (MC service client - MC traffic control function)

Table 11.5.1.2a.1-1 describes the information flow connection request sent from the MC service client, which resides on a non-3GPP device, to MC traffic control function in the MC gateway UE.

Table 11.5.x.2.1-1: Connection request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |

11.5.x.2.2	Connection request (MC traffic control function - MC gateway UE control function)

Table 11.5.1.2.2-1 describes the information flow connection request sent from the MC traffic control function in the MC gateway UE to the MC gateway UE control function in the MC service server.

Table 11.5.x.2.2-1: Connection request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| MC service ID | M | The MC service ID of the MC gateway UE |

11.5.x.2.3	Connection response (MC gateway UE control function  - MC traffic control function)

Table 11.5.1.2a.3-1 describes the information flow connection response sent from the MC gateway UE control function in the MC service server to the MC traffic control function in the MC gateway UE.

Table 11.5.x.2.3-1: Connection response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| MC service ID | M | The MC service ID of the MC gateway UE |
| Response | M | Result of the connection from the requesting MC service user to use the MC gateway UE is allowed. |

11.5.x.2.4	Connection response (MC traffic control function - MC service client)

Table 11.5.1.2a.4-1 describes the information flow connection response sent from the MC traffic control function in MC gateway UE function to the MC service client.

Table 11.5.1. 2a.4-1: Connection response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Response | M | Result of the connection from the requesting MC service user to use the MC gateway UE is allowed. |

11.5.x.3	Traffic control management procedures (MC gateway UE not hosting MC service client)

Pre-conditions

-	The MC gateway UE already completes the identity management procedure with the identity management server and retrieves its MC service identity.

-	The MC user at the non-3GPP device completes the identity management procedure with the identity management server and retrieves its MC service identity.

Figure 11.5.x.3.1-1: traffic control at the MC gateway UE (MC gateway UE not hosting MC service client)

1.	The MC service client sends connection request with its MC service ID and the requested MC traffic (e.g., destination MC service server address) to request to use the MC gateway UE to transport requested MC traffic.

2.	Upon receive the request, the MC traffic control function in the MC gateway UE determines the MC service server based on the MC service ID or the destination MC service server information. The MC traffic control function sends a connection request to the MC gateway UE control function in the MC service server. The MC service ID of the MC gateway UE, the MC service ID of the requesting MC service user are included.

3. 	The MC gateway UE control function in the MC service server, performs the authorization check and determines to allow the requesting MC service user to use this MC gateway UE to transport the requested MC traffic.

4. 	The MC gateway UE control function in the MC service server returns a connection response with the result including the allowed indication and the allowed MC traffic description (e.g., 5-IP tuples).

5.	The MC traffic control function in the MC gateway UE, after receiving the response indicating allowing the requesting MC service user to use the MC gateway UE for certain MC traffic. The MC gateway UE marks the allowed MC traffic description.

NOTE:	Before receiving such allowed MC traffic description, the MC gateway UE only forwards the identity management signallings and the connection request from the non-3GPP device to the server side.

6.	The MC traffic control function in the MC gateway UE, sends the connection response to the MC service client. After this, the MC service client uses this MC gateway UE to transmit the MC traffic towards the MC service server including the service authorization, affiliation request,  group call signallings, media packets.

11.5.x.4	Traffic control management procedures (MC gateway UE hosting MC service client)

Pre-conditions

-	The MC gateway UE already completes the identity management procedure with the identity management server and retrieves its MC service identity.

-	The MC user at MC gateway UE completes the identity management procedure with the identity management server and retrieves its MC service identity.

Figure 11.5.x.3.1-2: traffic control at the MC gateway UE (MC gateway UE hosting MC service client)

1.	The MC gateway UE control function in the MC service server, performs the authorization check and determines to allow the requesting MC service user to use this MC gateway UE to transport the requested MC traffic.

2. 	The MC gateway UE control function in the MC service server returns a connection response with the result including the allowed indication and the allowed MC traffic description (e.g., 5-IP tuples).

3.	The MC traffic control function in the MC gateway UE, after receiving the response indicating allowing the requesting MC service user to use the MC gateway UE for certain MC traffic. The MC gateway UE marks the allowed MC traffic description.

NOTE:	Before receiving such allowed MC traffic description, the MC gateway UE only forwards the identity management signallings and the connection request from the non-3GPP device to the server side.

4.	The MC traffic control function in the MC gateway UE, sends the connection response to the MC service client. After this, the MC service client uses this MC gateway UE to transmit the MC traffic towards the MC service server including the service authorization, affiliation request,  group call signallings, media packets.