3GPP TSG-SA WG6 Meeting #64	S6-245069

Orlando, United States, 18th – 22nd November 2024

Source:	TNO

Title:	AI/ML model lifecycle management

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	nassima.toumi@tno.nl

1. Introduction

The conclusion for Key Issue #2 in TR 23.700-82 indicates that solution #12 is a candidate for normative work. This contribution introduces support for AI/ML model lifecycle management into TS 23.482.

2. Reason for Change

The introduced clauses specify procedures, information flows, and APIs to add support for AI/ML model lifecycle management into TS 23.482.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change (all new text) * * * *

## 6.x	Support for AI/ML model lifecycle management

This functionality enables the AIMLE server to manage the lifecycle of trained and deployed AI/ML models by detecting model performance degradation, and triggering AI/ML model re-training and update to fix the observed degradation.

* * * Next Change (all new text) * * * *

## 8.x	AI/ML model lifecycle management

### 8.x.1	General

This clause provides the procedures to support AI/ML model lifecycle management for AI/ML model re-training and update when model performance degradation is observed by the AI/ML Enablement layer. The model update procedure also supports using an existing model to re-train the model using Transfer Learning. Additionally, if the degraded model is related to other models due to e.g., Transfer Learning, the AI/ML Enablement server may trigger the update of those related models as well.

### 8.x.2	Procedure

#### 8.x.2.1	ML Model Update

Figure 8.x.2.1-1 depicts the procedure where the AI/ML Enablement capability can trigger model update upon detecting model performance degradation.

Pre-conditions:

1.	The AI/ML Enablement Server has provided a ML model to the AI/ML Enablement Consumer.

2.	The AI/ML consumer may indicate to the AI/ML Enablement Server that the performance degradation will be detected by the consumer based on its local configuration.

Figure 8.x.2.1-1: Support for AI/ML model lifecycle management

0.	The AI/ML Enablement Consumer detects a performance degradation of the AI/ML model.

1.  The AI/ML Enablement Consumer sends an AI/ML model update request to the AI/ML Enablement Server that includes the ID of the model

2.	The AI/ML Enablement Server determines whether an update of the AI/ML model is required.

3.	The AI/ML Enablement Server retrieves the ML model information from the ML Model Repository as described in clause 8.11.3. The AI/ML Enablement Server may also perform ML model discovery to determine whether an existing ML model stored by the ML Model and Data Repository can be used to train the new model (e.g., using Transfer Learning).

The AI/ML Enablement Server can also discover models that are related due to Transfer Learning or the use of the same training data, to identify additional models that may require an update.

4.	The AI/ML Enablement Server performs ML model re-training, which corresponds to the ML model training procedure as described in other solutions in this TR.

If the degraded model is linked to other models (e.g., due to Transfer Learning, or the same training data has been used), the AI/ML Enablement Server may trigger the re-training and update of those related models as well.

5.	The AI/ML Enablement Server provides the updated ML model to the AI/ML Enablement Consumer either by sending it directly, or by providing information to retrieve it from the ML Model and Data Repository.

#### 8.x.2.2	Consumer-based ML Model Performance Degradation Detection

Figure 8.x.2.2-1: Consumer-based ML Model Performance Degradation Detection

This procedure corresponds to the step 0b in clause 8.x.2.1.

0.	An ADAE Server, as an AI/ML Enablement Consumer, receives a trained ML model (or ML model information) from the AI/ML Enablement Server.

1.	The ADAE Server receives a request from the consumer for analytics, generates analytics by using the ML model provided by AI/ML Enablement Server, and responds/notifies to the consumer with the required analytics.

2.	The ADAE Server requests the consumer to provide feedback on the usage of the analytics.

3.	The consumer uses the analytics for its operations and collects operation results. Performance degradation may be found from the operation results. The performance degradation may be caused by e.g., insufficient analytics accuracy, or the current analytics cannot fulfill the changed conditions at the consumer.

4.	The consumer feedbacks the usage result of the analytics to the ADAE Server.

5.	The ADAE Server updates the analytics accuracy based on feedback information from the consumer, and checks the performance of the ML model, which is used to generate the analytics, e.g., performance degradation of the ML model.

If the AI/ML Enablement Consumer is e.g. a VAL Server, a similar procedure as shown in Figure 8.x.2.2-1 can be performed for detecting of ML Model performance, by replacing "analytics" with e.g., "operation instructions".

### 8.x.3	Information flows

#### 8.x.3.1	AI/ML model update request

Table 8.x.3.1-1 details the ML model update request IEs.

Table 8.x.3.1-1: AI/ML model update request

| Information element | Status | Description |
| --- | --- | --- |
| AI/ML Enablement Consumer Identity | M | The identity of the AI/ML Enablement Consumer sending the notification. |
| AI/ML model ID | M | Provides the ID of AI/ML model for which the performance degradation has been detected. |
| Performance degradation information | O | Provides details about the detected performance degradation, such as the time, instances, or information on the degraded metrics (e.g. accuracy, recall, F1score). |
| AI/ML model retrieval information | O | Information on where the AI/ML is stored, such as the repository endpoint or ID. |

#### 8.x.3.1	AI/ML model update response

Table 8.x.3.2-1 details the ML model update response IEs.

Table 8.x.3.2-1: AI/ML model update response

| Information element | Status | Description |
| --- | --- | --- |
| Model update indication | M | Indicates whether the model has been updated |
| AI/ML model ID | M | Provides the ID of the updated AI/ML model |
| AI/ML model | O (NOTE) | Provides the updated AI/ML model |
| AI/ML model retrieval information | O (NOTE) | Provides the information of the ML model and data repository, from which the AI/ML model can be retrieved, such as the repository endpoint or ID. |
| NOTE: At least one of these information elements shall be provided. |  |  |

* * * Next Change (all new text) * * * *

## 9.x	AI/ML model update API

### 9.x.1	General

Table 9.x.1-1 illustrates the API for AI/ML model update.

Table 9.x.1-1: Aimles_MLModelRetrieval API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_MLModelUpdate | Request | Request/Response | AIMLE Consumer |

### 9.x.2	Aimles_MLModelUpdate_Request operation

API operation name: Aimles_MLModelUpdate_Request

Description: The consumer requests AI/ ML Model update.

Inputs: See clause 8.x.3.1.

Outputs: See clause 8.x.3.2.

See clause 8.x.2.1 for details of usage of this operation.

* * * End of Changes * * * *