3GPP TSG-SA WG6 Meeting #63	S6-245215

Orlando (FL), USA, 18th – 22nd November 2024	(revision of S6-24xxxx)

Source:	Huawei, HiSilicon

Title:	pCR on new gap analysis on supporting of IoT service

Spec:	3GPP TR 23.700-35 v0.1.0

Agenda item:	8.9

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to a new SEAL service API gap analysis of supporting IoT service at NRM server.

2. Reason for Change

1. The network API is not well designed for dedicated applicaion scenarios, e.g., pover saving, anti-theft, real-time snapshot of UE footprint information as described in TR 22.853.

Several network APIs as defined in TS 29.122 can be used by the IoT service. However, they are defined to cover all the verticals and all the scenarios. The input paramters contain both common parameters and vertical/scenario dedicated parameters, which makes it hard for the IoT appliction develper/service provider to figure out how to invoke the network API in a esay-to-use way.

| Network API in TS 29.122 | Description | Gaps |
| --- | --- | --- |
| MonitoringEvent API | allows the SCS/AS to subscribe to notifications about specific events in 3GPP networks when the corresponding event is detected. Support multiple event (as defined in table 5.3.2.4.3-1), but only one “MonitoringType” per subscription. | - Normally, one application may have multiple events to monitor. It has to maintain multiple subscription for each event. - Events serve different purposes and no aggregation - Some events is not for IoT, NUM_OF_REGD_UES, NUM_OF_ESTD_PDU_SESSIONS, GROUP_MEMBER_LIST_CHANGE - Some events are not per UE, e.g., NUMBER_OF_UES_IN_AN_AREA |
| CpProvisioning API | allows the SCS/AS to add, change or delete the communication pattern parameter sets of the UE. | This is related with pover saving for communication model with a regular interval. |
| NpConfiguration API | allows the SCS/AS to send suggested network parameters to influence certain aspects of UE/network behaviour such as the UE's PSM, extended idle mode DRX, and extended buffering configurations. | This is related power saving. |

In sumamry,

(2) NRM is defined to simplify the utilization of network resource by 3rd party, however, there are some gaps to well support IoT service:

-	SS_NetworkResourceAdaptation covers all the verticals and use cases e.g., unicast, multicast/broadcast, TSC. The supporting of IoT is defined as several API operations including BDT Configuration, Unified_Traffic_Pattern_and_Monitoring_Management operation, BDT_Configuration, Device_Triggering. The whole input parameters may confuse the IoT appliction develper/service provider how to invoke such API for IoT service.

-	Similar like the MonitoringEvent API from SA2, NRM Event monitoring API has the same issue.

-	The NRM service API granularity is not matched with the application use case, i.e., anti-theft, pover saving. the For a single use case, several NRM service APIs has to be invoked. For example, for pover saving, the IoT appliction develper/service provider has to invoke the SS_NetworkResourceAdaptation with Unified_Traffic_Pattern_and_Monitoring_Management operation, EventMonitoring APi with pover saving related input parameters.

Therefor, how to design IoT specific NRM service with proper the parameters and API granularity needs to be studied.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-35 v0.1.0.

* * * First Change * * * *

4.x.	Existing SEAL service #3: IoT service

4.x.1	Analysis

1. The network API is not well designed for dedicated applicaion scenarios, e.g., pover saving, anti-theft, real-time snapshot of UE footprint information as described in TR 22.853.

Several network APIs as defined in TS 29.122 can be used by the IoT service. However, they are defined to cover all the verticals and all the scenarios. The input paramters contain both common parameters and vertical/scenario dedicated parameters, which makes it hard for the IoT appliction develper/service provider to figure out how to invoke the network API in a esay-to-use way.

| Network API in TS 29.122 | Description | Gaps |
| --- | --- | --- |
| MonitoringEvent API | allows the SCS/AS to subscribe to notifications about specific events in 3GPP networks when the corresponding event is detected. Support multiple event (as defined in table 5.3.2.4.3-1), but only one “MonitoringType” per subscription. | - Normally, one application may have multiple events to monitor. It has to maintain multiple subscription for each event. - Events serve different purposes and no aggregation - Some events is not for IoT, NUM_OF_REGD_UES, NUM_OF_ESTD_PDU_SESSIONS, GROUP_MEMBER_LIST_CHANGE - Some events are not per UE, e.g., NUMBER_OF_UES_IN_AN_AREA |
| CpProvisioning API | allows the SCS/AS to add, change or delete the communication pattern parameter sets of the UE. | This is related with pover saving for communication model with a regular interval. |
| NpConfiguration API | allows the SCS/AS to send suggested network parameters to influence certain aspects of UE/network behaviour such as the UE's PSM, extended idle mode DRX, and extended buffering configurations. | This is related power saving. |

(2) NRM is defined to simplify the utilization of network resource by 3rd party, however, there are some gaps to well support IoT application:

-	SS_NetworkResourceAdaptation covers all the verticals and use cases e.g., unicast, multicast/broadcast, TSC. The supporting of IoT is defined as several API operations including BDT Configuration, Unified_Traffic_Pattern_and_Monitoring_Management operation, BDT_Configuration, Device_Triggering. The whole input parameters may confuse the IoT appliction develper/service provider how to invoke such API for IoT service.

-	Similar like the MonitoringEvent API from SA2, NRM Event monitoring API has the same issue.

-	The NRM service API granularity is not matched with the application scenarios, i.e., anti-theft, pover saving. For a single use case, several NRM service APIs has to be invoked. For example, for pover saving, the IoT appliction develper/service provider has to invoke the SS_NetworkResourceAdaptation with Unified_Traffic_Pattern_and_Monitoring_Management operation, EventMonitoring APi with pover saving related input parameters.

In summary, current NRM service and API are not designed in an application - oriented and friendly manner that the input parameters should be dedicated to IoT application, simple and application familiar, and the granularity of API should be well mapped to application scenarios.

4.x.2	Identified gaps

how to enhance NRM services service with proper the parameters and API granularity to well support IoT application needs to be studied.