3GPP TSG-SA WG6 Meeting #64	S6-245223

Orlando, USA, 18th – 122nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0354 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area handling at NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eSEAL2 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 has agreed several CRs related to MBS service area handling in case of the MBS service area may be across multiple MB-SMFs. In this case the NEF/MBSF groups the MBS service area based on the supported MB-SMF and returns the grouped MBS service area to the AF (i.e., the NRM in SEAL). So the SEAL needs to handle the grouped MBS service area as follows: - If TMGI allocation is rejected with returned MBS service areas, the NRM server should initiate a new TMGI allocaiton for each grouped MBS serivce area. - If MBS session creation is rejected with returned MBS service areas, the NRM server should initiate a new MBS session creation procedure for each grouped MBS serivce area. - If MBS session update is rejected with returned MBS service areas, the NRM server should initiate a new MBS session creation procedure for each grouped MBS serivce area. In addition,the NRM server needs to update the VAL server the new DL MBS data ingress address. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new descriptions about how the NRM server handle MBS service area as described above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NRM is not able to resolve the MBS service area failure issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.42, 14.3.2.43, 14.3.4A.2.2, 14.3.4A.2.3, (new)14.3.4A.3.x, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /*********************************** First changes ******************************/

14.3.4A.2.2	Procedure for pre-created MBS session and MBS session announcement

Pre-conditions:

-	The NRM server has decided to use an MBS session for VAL service group communications associated to a certain VAL servive group based on transport only mode.

-	The NRM server has performed MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	NRM clients 1 to n are attached to the 5GS, registered and belong to the same VAL service group X.

-	The NRM server is aware whether to request the creation of the MBS service server with or without dynamic PCC rule.

Figure 14.3.4A.1.2-1: Use of pre-created MBS session.

1.	The VAL server sends a multicast/broadcast resource request towards the NRM server including the VAL server identity, service description(s), multicast resource type (e.g,. multicast type or broadcast type), service announcement mode (i.e., service announcement is sent by NRM or the VAL itself), Endpoint of the VAL server.

2.	The NRM server initiates an MBS session creation procedure towards the 5GC as described in 3GPP TS 23.247 [39]. The procedure starts once the NRM server initiates a TMGI allocation request (either directly to MB-SMF or indirectly via NEF). Upon the reception of the TMGI allocation response, the NRM server sends an MBS session creation request, including further information related to the MBS session, e.g., MBS session ID, MBS session mode and the QoS requirements if dynamic PCC rule is not considered. However, if dynamic PCC rule is considered, the NRM server defines these requirements at a later step, namely it sends an MBS authorization/policy create request towards PCF (either directly or to the NEF) indicating the QoS requirements.

In the case of an untrusted NRM server, when the requested MBS service area crosses several MB-SMF service areas, the 5GC rejects the TMGI allocation or MBS session creation request and returns a group of MBS service areas which are devided based on the requested MBS service area as described in 3GPP TS 23.247 [15]. If the TMGI allocation request is rejected as above, NRM server should initiate a new TMGI allocation request procedure for each grouped MBS service area. If the MBS session creation is rejected as above, the NRM server may utilize a unicast session if any MBS service area is not supported by any MB-SMF.

NOTE 1:	In case of LTE eMBMS and 5G MBS co-existence, the NRM server may trigger the establishment of eMBMS bearers as described in clause 14.3.4 (or it may establish a unicast bearer) based on the RAT capabilities supported by the VAL service group members in the VAL service group X. If MBSF and BM-SC are co-located, TMGI used by 4G eMBMS can be the same as the MBS session ID.

NOTE 2:	For the case of multi carrier support for broadcast MBS sessions, the NRM server may indicate the frequencies within a broadcast MBS service area by providing the MBS frequency selection area ID(s) (MBS FSA ID(s)) to the MB-SMF or indirectly via NEF.

3.	The NRM server provides the NRM clients of the VAL service group X with the information related to the created MBS session via the MBS session announcement. The MBS session announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type) and SDP information related to the MBS session under consideration.

NOTE 3:	The NRM server may send an MBS session announcement at an earlier step during the MBS session creation procedure towards the NRM clients once the VAL service group associated to the MBS session is known.

Optionally, the NRM server includes the information elements related to the established eMBMS bearer, as indicated in table 7.3.2.2-1. The NRM clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].

4.	NRM clients store and process the received MBS session information.

5.	NRM clients may provide an MBS session announcement acknowledgment to the NRM server to indicate the reception of the corresponding MBS session announcement.

6.	Based on the MBS session mode (either multicast or broadcast), the following actions take place;

6a.	For multicast MBS sessions, NRM clients initiate a UE session join request towards the 5GC using the information provided via the MBS session announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved, if the session is in an active state. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [39]. The NRM clients sends a UE session join notification towards the server as indicated in the MBS session announcement. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server; or

6b.	For broadcast MBS sessions, if the NRM client is accessing over 5G, the session is established as part of the session creation procedures as described in 3GPP TS 23.247 [39], and the network resources are reserved both in 5GC and NG-RAN. The NRM clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server.

NOTE 4:		It is implementation specific whether the MBS session reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signals, such as the reference signals from the NG-RAN node(s), or packet loss.

7.	The NRM clients provide a listening status notification related to the announced session (multicast or broadcast session) in the form of an MBS listening status report.

8.	The NRM server provides a multicast/broadcast resource response to the VAL server.

9.	An VAL service group communication setup takes place and uses the pre-created MBS session for this group communication packet DL delivery.

14.3.4A.2.3	Procedure for dynamic MBS sessions

In this scenario, the VAL service group communication is already taken place and a unicast PDU session is utilized for  DL transmission. When the NRM server decides to use an MBS session for the transmission under consideration, the NRM server interacts with 5GC to reserve the necessary network resources.

NOTE 1:	The NRM server logic for determining when to create a dynamic MBS session is implementation specific.

The procedure in figure 14.3.4A.2.3-1 shows one NRM client receiving the DL media. There might also be NRM clients in the same VAL service group communication session that receive the communication on a PDU session.

Pre-conditions:

-	NRM client is attached to the 5GS, registered and affiliated to a certain VAL service group X.

-	The NRM server is aware whether to request the creation of the MBS session with or without dynamic PCC rule.

-	The NRM server has performed MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	No MBS session exists, or the existing multicast MBS session fails to satisfy the QoS requirements.

Figure 14.3.4A.2.3-1: Use of dynamic MBS session.

1.	An VAL service group communication session is established and the DL media packet is delivered to the VAL client via unicast.

2.	The VAL server sends a multicast/broadcast resource request towards the NRM server including the VAL server identity, service description(s), multicast resource type (e.g,. multicast type or broadcast type), service announcement mode (i.e., service announcement is sent by NRM or the VAL itself), Endpoint of the VAL server.

3.	The NRM server decides to create an MBS session. The MBS session creation procedure takes place as described in clause 14.3.4A.2.2.

NOTE 2:	In case of LTE eMBMS and 5G MBS co-existence, the NRM server may trigger the establishment of eMBMS bearers as described in 3GPP TS 23.280 [3] (or it may establish a unicast bearer) based on the RAT capabilities supported by the affiliated members in the VAL service group X. If MBSF and BM-SC are co-located, TMGI used by 4G eMBMS can be the same as the MBS session ID.

NOTE 3:	For the case of multi carrier support for broadcast MBS sessions, the NRM server may indicate the frequencies within a broadcast MBS service area by providing the MBS frequency selection area ID(s) (MBS FSA ID(s)) to the MB-SMF or indirectly via NEF.

4.	The NRM server provides the NRM client with the information related to the created MBS session via an MBS session announcement. As described in table 7.3.2.2-1, the session announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type), and SDP information related to the MBS session.

Optionally, the NRM server includes the information elements related to the established eMBMS bearer once the NRM server has determined the need, as indicated in table 7.3.2.2-1. The NRM clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].

5.	The NRM client stores the MBS session ID and other associated information.

6.	The NRM client may send an MBS session announcement ack back to the NRM server.

7.	Based on the MBS session mode (either multicast or broadcast), the following actions take place:

7a.	For multicast MBS sessions, NRM client initiates a UE session join request towards the 5GC using the information provided via the MBS session announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved, if the session is in active state. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [39]. The NRM client sends a UE session join notification towards the server as indicated in the MBS session announcement. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server; or

7b.	For broadcast MBS sessions, if the NRM client is accessing over 5G, the session is established as part of the session creation procedures as described in 3GPP TS 23.247 [39], and the network resources are reserved both in 5GC and NG-RAN. The NRM clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server.

NOTE 4:		It is implementation specific whether the MBS session reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signaling such as the reference signals from the NG-RAN node(s), packet loss.

8.	The NRM clients provide a listening status notification related to the announced session (multicast or broadcast session) in the form of an MBS listening status report.

9.	The NRM server provides a multicast/broadcast resource response to the VAL server.

10.	An VAL service group communication via dynamic MBS session is established. The VAL server sends the downlink packet for the VAL service group communication session over the MBS session.

# /******************** Next Changes ********************/

14.3.4A.3.x	MBS service area update failure handling

The MBS service area requested by the MC service server if it is across multiple MB-SMFs, may be rejected in the TMGI allocation procedure and the MBS session creation procedure. It may be partially accepted in the MBS session update procedure. In such case, the 5GC  the NRM server may take different actions as follows:

-	If the MBS service area requested in the TMGI allocation or MBS Session Creation without TMGI procedure as specified in 3GPP TS 23.247 [15] is rejected and a group of MBS service areas are returned , the NRM server should initiate a new TMGI allocation or MBS session creation request procedure for each grouped MBS service area. The NRM server may also determine to use the unicast delivery for the group communication in that area.

-	If the MBS service area requested in MBS Session Update procedure as specified 3GPP TS 23.247 [15] is partially accepted due to the requested MBS service area is beyond the service area of the serving MB-SMF and a group of MBS service areas are returned, the NRM server should initiate a new MBS session creation request procedure for each grouped MBS service area.

The figure 14.3.4A.3.x-1 provides a high level procedure of the NRM server determines to create an extra MBS Session for each grouped MBS service area indicated by the 5GC due to the partially accept of MBS service area update of the ongoing MBS session.

NRM client

3GPP

NRM server

VAL server

VAL client

UE

Created MBS session with initial MBS service area

1a. MBS session update request with new service area

1b. New service area based on UE location report

2. MBS session update with new service area is partially accepted

4. MBS session creation with new service area, and obtain new MBS session ID and ingress address

3. Based on the return information, try to create a new MBS session

5. Annouce the new MBS session, UE join the MBS session if multicast type, UE report listening status.

6. MBS session update response

7. User plane delivery mode

Figure 14.3.4A.3.x-1: MBS service area update failure handling

1.	The NRM server determines to update service area of the existing MBS session. This may be triggered by the MBS session update request with a new service area from the VAL server (step 1a), or the NRM server monitors the UE’s mobility and generates the new service area (step 1b) based on e.g., SEAL LMS mechanism.

2. The NRM servers performs the MBS session update with updated MBS service area towards the 5GC as described in 3GPP TS 23.247 [39]. The MBS session update is partically accepted and a group of  MBS service areas is returned as described in 3GPP TS 23.247 [39].

3.	Upon receiving the partially accept response, the NRM determines to create extra MBS sessions for each grouped MBS service area.

4.	 For each grouped MBS service area, the NRM server performs as described in 3GPP 23.247 [39]. The NRM server obtains the new MBS session ID and new ingress address of the new MBS session.

5.	The NRM server announces the new MBS session information towards the UEs. The UE joins the MBS session if it is multicast type. The UE may report the MBS listening status to the NRM server.

6.	 The NRM server returns the MBS session update response to the VAL server. The new MBS session ID are included. The ingress address of the new MBS session may be also included.

7.	The NRM server also indicates the VAL server to deliver the data over the new MBS session (i.e., deliver data over both the existing MBS session and the new MBS session). If the MBS session creation is failed in step 4, the NRM server indicates the VAL server to use unicast delivery for that MBS service area.

# /******************** Next Changes ********************/

14.3.2.42	Multicast/broadcast resource update request

Table 14.3.2.42-1 describes the information flow for the multicast/broadcast resource update request from VAL server to NRM server.

Table 14.3.2.42-1: Multicast/broadcast resource update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Identity of the multicast/broadcast resource | M | Identity of the multicast/broadcast resource to be updated. |
| Service requirement description | O (NOTE 3) | Indicates the service characteristics, e.g., Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator, emergency indicator Application service provider. |
| Multicast/Broadcast area information | O(NOTE 3) | Indicate the area where the multicast/broadcast resource is requested for |
| Local MBMS information (NOTE 1, NOTE 2) | O(NOTE 3) |  |
| > MB2-U information | O | IP address, UDP port number of the MB2-U interface |
| > xMB-U information | O | IP address, UDP port number of the xMB-U interface |
| > M1 interface information | O | M1 interface information for local MBMS |
| Local MBMS activation indication (NOTE 1, NOTE 2) | O | Indicates whether to request the NRM server to use Local MBMS information. |
| NOTE 1: The VAL server may provide either the Local MBMS information or the Local MBMS activation indication. This IE is present when the local MBMS is required for VAL services like V2X service. NOTE 2: This information element is only applicable when EPS is specified in the network system indicator IE. NOTE 3: At least one of those IEs shall be present. |  |  |

14.3.2.43	Multicast/broadcast resource update response

Table 14.3.2.43-1 describes the information flow for the Multicast/broadcast resource update response from NRM server to VAL server.

Table 14.3.2.43-1: Multicast/broadcast resource update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the multicast/broadcast resource update request operation. |
| Identity of the multicast/broadcast resource | M (NOTE 1) | The identity of the multicast/broadcast MBS session, or/and the identity of the MBMS bearer to be updated. |
| New Identity of the multicast/broadcast resource | O | The identity of the new multicast/broadcast MBS session created during update. |
| User plane address | O | Additional user plane IP address(es) and port for VAL to deliver the DL packet |
| Session description information | O (NOTE 2) | Indicates Multicast/broadcast related configuration information as defined in 3GPP TS 26.346 [28] (e.g. radio frequency and MBMS Service Area Identities) or/and 3GPP TS 23.247 [39] (e.g. MBS FSA ID(s), Area Session ID, service requirements) |
| NOTE 1: This IE may not be required if the session announcement mode indicates that the request is sent by the NRM server. NOTE 2: If the Result Information element indicates failure then the values of the other information elements shall not present. |  |  |