3GPP TSG-SA WG6 Meeting #64	S6-245224

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0355 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Upcase correction of location management client and location management server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several format of location management server and location management client used in TS 23.434, e.g., Location Management Server, Location Management Client, Location management server, Location management client, LM server, LM client, LM server, LM Client. Such term should be aligned in order to avoid confusion that they may be different entities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the term of location management server and management client |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bad TS quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.3, 9.3.2.7, 9.3.2.9, 9.3.2.11, 9.3.2.12, 9.3.2.13, 9.3.2.16, 9.3.2.26, 9.3.2.27, 9.3.2.28, 9.3.2.29, 9.3.2.30, 9.3.2.31, 9.3.2.34, 9.3.2.35, 9.3.2.40, 9.3.2.42, 9.3.2.55, 9.3.5, 9.3.6, 9.3.7, 9.3.8, 9.3.9, 9.3.11.1, 9.3.11.2, 9.3.12, 9.3.12.1, 9.3.12.2, 9.3.12.3, 9.3.12.4, 9.3.14.2, 9.3.15, 9.3.18, 9.3.19, 9.3.20, 9.3.21, 9.3.21.1, 9.3.21.2, 9.3.22, 9.4.8.1, 9.5, 15.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and/or from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |
| Requested location information | O | Identifies what location information is requested |
| Requested location access type (NOTE 4) | O (NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Requested positioning method | O (NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. If the requested VAL user/UE is not the receiving VAL user/UE hosting the location management clientg, the UE positioning method includes PC5 SEAL LM, BT, WiFi. |
| Location QoS | O (NOTE 3) | Definition of the location Quality of Service for which the location information is requested (see NOTE 2). |
| NOTE 1: This e element is only applicable for request sent from the location management server to the location management clientg. NOTE 2: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 3: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 4: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. |  |  |

Editor's Note: It's FFS the security aspects for LM-Uu and LM-S in relation to the VAL service ID that need to be coordinated with SA3.

# /**************************** Next changes **********************/

9.3.2.7	Location information notification

Table 9.3.2.7-1 describes the information flow from the location management server to the VAL server or the location management client.

Table 9.3.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of the VAL users or VAL UEs whose location information needs to be notified |
| Identity (see NOTE 2) | O | Identity of the VAL user or VAL UE subscribed to location of another VAL user or VAL UE (see NOTE 1) |
| Subscription ID (see NOTE 2) | O | Subscription identity related to VAL server subscription with location management server for location information notification. |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information | M | Location information |
| Velocity information | O | Indicates the velocity of the target UE for which the location information is requested. |
| Statistic of location data | O | Indicates the statistic result of target UE location data per temporal/spatial granularity as requested. |
| Velocity information | O | Velocity information |
| Timestamp | O | Timestamp of the location report |
| NOTE 1: This is only used for location management server sends location information notification to the VAL user or VAL UE who has subscribed the location. NOTE 2: Either Identity or Subscription ID shall be included. |  |  |

# /**************************** Next changes **********************/

9.3.2.9	Get UE(s) information request

Table 9.3.2.9-1 describes the information flow for a VAL server to get UE(s) information at the location management server.

Table 9.3.2.9-1: Get UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| Location information | M | Location information around which the UE(s) information is requested (e.g. geographic area or VAL service area ID). The information is also used for Geofencing service. |
| > Shape type | O | It includes the Geofencing location area represented by different types of shape (e.g., polygon or circle) as defined in 3GPP TS 29.572 [51]. |
| > Area Coordinates | O | Geographical coordinates of shape as defined in 3GPP TS 29.572 [51]. |
| Application defined proximity range information | M | Description of the range information over which the UE(s) information is required. |

# /**************************** Next changes **********************/

9.3.2.11	Monitor Location Subscription Request

Table 9.3.2.11-1 describes the information flow from the VAL server to the location management server for monitoring the location deviation of the VAL UE(s) or for updating an existing monitor location subscription.

Table 9.3.2.11-1: Monitor Location Subscription Request

| Information element | Status | Description |
| --- | --- | --- |
| Target Identity | M | Identifier of the target VAL users or VAL UEs whose location monitoring is requested to be monitored in a given location (see NOTE). |
| Area of Interest | M | Geographic area location information where the VAL server wishes to monitor the VAL UE's location adherence. This could be geographical co-ordinates or VAL service area identified by the VAL service area ID. Alternatively, it includes identifier of the reference VAL user or VAL UE and its proximity range. |
| Notify Interval | M | Periodic time interval in which the location management server needs to notify the target VAL UE's location information to the VAL server. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about target VAL UE's location. |
| Timeout | O | A timeout period when subscription response is not received. |
| Interested notification information | O | One or more interested notification events (e.g. Notify Absence, Notify Presence, Notify Mismatch Location and Notify Unknown) the VAL server can subscribe to. |
| Dynamic geofencing conditions | O | Temporal, connectivity and/or spatial conditions for the reference VAL UE. |
| NOTE: This information element shall not be updated via monitor location subscription update procedure in clause 9.3.2.12a. |  |  |

9.3.2.12	Monitor Location Subscription Response

Table 9.3.2.12-1 describes the information flow from location management server to the VAL server for Monitor Location Subscription Response or monitor location subscription update response.

Table 9.3.2.12-1: Monitor Location Subscription Response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |

9.3.2.13	Notify Location Monitoring Event

Table 9.3.2.13-1 describes the information flow from location management server to the VAL server for notification of location monitoring events.

Table 9.3.2.13-1: Notify Location Monitoring Event message

| Information element | Status | Description |
| --- | --- | --- |
| Event | M | Information of the event to be reported. The event shall be one of the following: "Notify_Mismatch_Location" – When the location information of the target VAL UE, from the location management client and the core network do not match, or when the location management server determined area of interest result based on location management client location and the core network reported area of interest result do not match. "Notify_Absence" – When the target VAL UE's current location is deviating from the VAL server's area of interest information. "Notify_Presence" – When the target VAL UE's current location is within the VAL server's area of interest information. "Notify_Unknown" – When the location management server cannot determine whether the target VAL UE's current location is within the VAL server's dynamic area of interest information or not. |
| Identity | M | Identifier of the VAL UE whose location information is reported. |
| Location | M | Current location of the target VAL UE. |

# /**************************** Next changes **********************/

9.3.2.16	Location area monitoring notification

Table 9.3.2.16-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.16-1: Location area monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription |
| UEs currently present (see NOTE) | O | List of the identities of all VAL UEs who are currently present in the given location area |
| UEs moved in/out (see NOTE) | O | List of UEs either moved in to the location area or moved out of the location area |
| > UEs moved in | O | List of the identities of the VAL UEs who moved in to the given location area since previous notification. |
| > UEs moved out | O | List of the identities of the VAL UEs who moved out of the given location area since previous notification. |
| Number of UEs moved in | O | Number of the VAL UE(s) who moved in/out the given location area in a given time period. |
| > Number of UEs moved in | O | Number of the VAL UE(s) who moved in the given location area in a given time period. |
| > Number of UEs moved out | O | Number of the VAL UE(s) who moved out the given location area in a given time period. |
| Time of UEs moved in/out | O | The timestamp and time period of UEs either moved in the location area or moved out of the location area. |
| >Time stamp and time duration of UE moved in | O | The timestamp and time period of UEs who moved in the given location area since previous notification. |
| >Time stamp and time duration of UE moved out | O | The timestamp and time period of UEs who moved out the given location area since previous notification. |
| Triggering event | O | Identity of the event that triggered the sending of the notification |
| Reason | O | It indicates the failure reason for the notification report (e.g., "UE cannot be connected") when the location management server can’t obtain the UE location data. |
| NOTE: Any one of these shall be present. |  |  |

# /**************************** Next changes **********************/

9.3.2.26	VAL service area obtain request

Table 9.3.2.26-1 describes the information flow from a VAL server to a location management server for obtain VAL service area identifiers at the location management server.

Table 9.3.2.26-1: Obtain VAL service area request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| VAL service area IDs | M | Indicates the list of the requested VAL service area identifiers. |

9.3.2.27	VAL service area obtain response

Table 9.3.2.27-1 describes the information flow from a location management server to the VAL server for obtain VAL service area identifiers at the location management server.

Table 9.3.2.27-1: Obtain VAL service area response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |
| List of VAL service area information | O (see NOTE) | List of one or more VAL service area identifiers. |
| > VAL service area ID | M | The VAL service area identifier. |
| > Geographical locations list | M | List of one more geographical locations (e.g., co-ordinates) corresponding to the VAL service area ID. |
| NOTE: This information element shall be present only if the obtain VAL service area identifier(s) is successful. |  |  |

9.3.2.28	VAL service area update request

Table 9.3.2.28-1 describes the information flow from a VAL server to a location management for update VAL service area identifiers at the location management server.

Table 9.3.2.28-1: Update VAL service area request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| List of VAL service area information | M | List of one or more VAL service area identifiers that shall be updated. |
| > VAL service area ID | M | Indicates the VAL service area identifier. |
| > Geographical locations | M | Indicates the list of the updated geographical locations (e.g., the geographical coordinates) corresponding to the VAL service area ID. |

9.3.2.29	VAL service area update response

Table 9.3.2.29-1 describes the information flow from a location management server to the VAL server for update VAL service area identifiers at the location management server.

Table 9.3.2.29-1: Update VAL service area response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |
| VAL service area IDs | O (see NOTE) | Indicates the list of the updated VAL service area identifiers. |
| NOTE: This information element shall be present only if the update VAL service area identifier(s) is successful. |  |  |

9.3.2.30	VAL service area delete request

Table 9.3.2.30-1 describes the information flow from a VAL server to a location management for delete VAL service area identifiers at the location management server.

Table 9.3.2.30-1: Update VAL service area request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the VAL server identity. |
| List of VAL service IDs | M | List of one or more VAL service area identifiers that shall be deleted. |

9.3.2.31	VAL service area delete response

Table 9.3.2.31-1 describes the information flow from a location management server to the VAL server for delete VAL service area identifiers at the location management server.

Table 9.3.2.31-1: Update VAL service area response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |
| VAL service area IDs | O (see NOTE) | Indicates the list of the deleted VAL service area identifiers. |
| NOTE: This information element shall be present only if the delete VAL service area identifier(s) is successful. |  |  |

# /**************************** Next changes **********************/

9.3.2.34	Monitor location unsubscribe request

Table 9.3.2.34-1 describes the information flow from the VAL server to the location management server for monitor location unsubscribe request.

Table 9.3.2.34-1: Monitor location unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the VAL server’s requesting ID. |

9.3.2.35	Monitor location unsubscribe response

Table 9.3.2.35-1 describes the information flow from location management server to the VAL server for monitor location unsubscribe response.

Table 9.3.2.35-1: Monitor location unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the unsubscribe request result |

# /**************************** Next changes **********************/

9.3.2.40	Location reporting configuration update request

Table 9.3.2.40-1 describes the information flow from the location management server to the location management client for the location reporting configuration update when location management server needs to deliver the new or update location reporting configuration to location management client if the location management server received the new requests/parameters from other entities (e.g. the VAL server or other location management client). This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.40-1: Location reporting configuration update request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Requested location access type (see NOTE 2) | O (see NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Requested positioning method | O (see NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report as specified in table 9.3.2.4-2. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE 1: If none of the information element is present, this represents a cancellation for location reporting. NOTE 2: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. |  |  |

# /**************************** Next changes **********************/

9.3.2.42	VAL service area subscription request

Table 9.3.2.42-1 describes the information flow from a SEAL server to a location management server to subscribe VAL service area identifier update events or update an existing subscription at the location management server.

Table 9.3.2.42-1: VAL service area subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the SEAL server identity (see NOTE). |
| List of the subscribed events | M | Indicates the list of the requested events. |
| > Event type | M | Indicates the subscribed event (e.g., update/deletion of the VAL service area ID) |
| > VAL service area IDs | O | Indicates the list of the VAL service area identifiers. This information element shall be provided when the requested event type is update or delete VAL service area ID. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL service area update events. |
| Subscription duration | O | It indicates the subscription duration. |
| NOTE: This information element shall not be updated via the update VAL service area subscription request. |  |  |

9.3.2.55	Surrounding UE retrieval request

Table 9.3.2.55-1 describes the information flow from the location management server to the location management client for obtaining surrounding UE information.

Table 9.3.2.55-1: Surrounding UE retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requested VAL user or VAL UE |
| Surrounding UE retrieval method | O | Identify how to retrieve surrounding UE(s), e.g. ProSe, BT, WiFi |

# /**************************** Next changes **********************/

9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. The location reporting event triggers as specified in table 9.3.2.4-1, e.g. minimum time between consecutive reports, SAI changes, access RAT changes, or ECGI changes for reporting the location of the VAL UE, are included.

NOTE:	Step 1 can be performed when location management client 2 or VAL server require to update the location reporting trigger corresponding to location management client 1.

2.	Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. If the immediate report indicator is included in step 1 and there is valid location report of the target UE (i.e. the timer for the stored location report is not expired), the location management server reuses the stored UE location report and sends the report to the location management client 2 or VAL server.

a.	If the adaptive reporting is requested in step 1, the location management server interacts with NWDAF (e.g., UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [16]) to obtain the location management client 1 moving statistic and prediction

i.	If "DIRECT UPDATE" is requested, the location management server determines the reporting configuration based on VAL service ID, VAL server ID, and the retrieved location management client 1 moving statistic and prediction. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or update location change condition (e.g., to change the location accuracy).

ii.	If "SUGGESTIVE UPDATE" is requested, the location management server initially uses the configuration provided by VAL server and later dynamically adjusts the reporting configuration to determine adaptive location configuration based on VAL service ID, VAL server ID, and by analysing the retrieved location management client 1 moving statistic and prediction and received location information. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or update the location change condition (e.g., to change the location accuracy).

b.	If the valid location report is not available, depending on the information specified by the location reporting trigger or determined reporting configuration (if "DIRECT UPDATE" is requested), location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server.

Further, if adaptive reporting is enabled by the VAL server with "SUGGESTIVE UPDATE", the location management server dynamically adjusts the reporting configuration as specified above (in step 2.a.ii).

Once location configuration is adjusted and if it is required to update the triggers to the SEAL client, the SEAL LMS may suggest the adaptive location configuration to the VAL server (or authorized SEAL location management client) as specified in clause 9.3.19. If VAL server accepts the suggested adaptive location configuration, then the location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1. If VAL server rejects the suggested adaptive location configuration, then the location management server will discard the updated configuration.

9.3.6	Location reporting triggers configuration cancel

Figure 9.3.6-1 illustrates the procedure used for cancelling the location reporting triggers configuration at the target location management client.

Pre-conditions:

1.	The location management server has subscribed the location management client 2 location with the location reporting event triggers.

2.	If multicast delivery mode is used, the MBMS bearer being used is activated by the location management server.

Figure 9.3.6-1: Location reporting triggers configuration cancel

1.	The location management client 1 (authorized VAL user or VAL UE) or VAL server sends a location reporting configuration cancel request to the location management server (1a). The location management server sends the location reporting configuration cancel request to the location management client 2 to stop receiving the UE location information (1b). This message can be sent via unicast or multicast.

NOTE:	Step 1b can be initiated without step 1a.

2.	The location management client invalidates the location reporting triggers configuration and no longer reports its location to the location management server.

3.	The location management client 2 sends the location reporting configuration cancel response to the location management server (3a) as an acknowledgement. The location management server sends the location reporting configuration cancel response to the location management client 1 (3b) as an acknowledgement.

9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for initiating tracking a UE's location. The procedure considers target UE location information provided by its surrounding UEs, the procedure can improve location accuracy and can be used also for target UE temporarily out of 3GPP radio coverage. location management client 1 is used as example in the figure as target UE that the VAL server is interested to know its location information.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information, the location QoS which contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50], the velocity information of target UEs, and an indication whether the statistic of target UE location is needed per e.g., temporal/spatial granularity. If the indication whether the statistic of target UE location is needed, the corresponding time/location information should also be included. In addition, the VAL server may request velocity information of the VAL UEs/Users in step 1.

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request and may retrieve the available and proper location access type and positioning methods (e.g. as described in TS 23.273 [50] and TS 29.572 [51]) for the target VAL UE based on the received location QoS. Further, the location management server may initiate location reporting configuration optionally including the retrieved location access type and positioning methods with the location management client of the UE for immediate reporting.

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network and/or the 3rd party location management server.

If the velocity information of target UE is requested in step 1, the location management server asks the location management client or 3GPP core network to report the target UE location data with the requested velocity information.

NOTE 1:	How the location management server obtains the UE location from the 3rd party location management server is out of scope of this specification.

4.	The location management server determines the UE location information of the UE as received in steps 2 and 3 and checks if it meets the location QoS requirements (if any) or not.

If the indication whether the statistic of target UE location is included in step 1, the location management server performs the following actions based on different granularity.

If the indication whether the statistic of target UE location is needed per temporal granularity is included in step 1, the location management server calculates the UE location data received in step 3 per the temporal granularity with the requested time condition.

If the indication whether the statistic of target UE location is needed per spatial granularity is included in step 1, the location management server calculates the UE location data received in step 3 per the spatial granularity with the requested location information.

5.	The location management server may optionally request the UE location from its surrounding UEs (as described in step 5 to 9). The location management server decides how to retrieve surrounding UEs which may be based on location management client 1 registered positioning method in location service registration. If the location management server decides to use UE-assisted surrounding UE retrieval, step 6 is executed; otherwise, NW-assisted surrounding UE retrieval is used as described in step 7.

6.	The location management server requests the location management client 1 with an optional surrounding UE retrieval method (e.g. ProSe, BT, WiFi) to provide its surrounding UEs and the location management client 1 responds the location management server with VAL UE/User ID of discovered UEs as described in clause 9.3.x;

NOTE 2:	For ProSe capable UE, the User Info ID defined in 3GPP TS 23.586 [57] can represent VAL user ID. For Bluetooth capable UE, the Bluetooth MAC address can represent VAL UE ID. For WiFi Direct capable UE, the WiFi MAC address can represent VAL UE ID.

7a.	The location management server derives an appropriate location area (where UE to UE communication is possible) internally using the location information of UE 1 (determined in step 4) as reference location. If the UE 1 location is last known UE location, the location management server uses UE mobility analytics service from 3GPP CN to derive UE location estimation for UE 1.

7b.	The location management server uses 3GPP CN service (NEF service of Number of UEs present in a geographical area) to obtain all UEs within the derived location area in step 6.

8.	The location management server selects a set of UEs (location management client 2..n) from UEs obtained in step 5 or step 7 based on registered location service information (e.g. positioning method) from location management client 2..n. Then location management server sends location request to location management client 2..n using On-demand location reporting procedure (as defined in clause 9.3.4) to obtain UE 1 location including an optional UE positioning method (e.g. PC5 SEAL LM, BT, WiFi) to be used to retrieve location.

NOTE 3:	The location management server can, based on local policy, select an UE without LM registration and trigger location request to the UE. If so, the request can be ignored by the UE (e.g., if SEAL LM-UU service is not supported) or the UE can reject the request (if the required positioning method is not supported).

9.	The location management server takes the location information of UE 1 reported by location management client 2..n into consideration to calibrate UE 1 location determined in step 4.

NOTE 4:	It is assumed that different location information collected over different positioning method may have differences in accuracy. How location management server processes calibration is implementation specific.

10.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

NOTE 5:	The VAL server can use obtained UE location and velocity in application specific ways (e.g. traffic monitoring in V2X).

9.3.8	Event-trigger location information notification procedure

Figure 9.3.8-1 illustrates the high level procedure of event-trigger usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE. This procedure is also used for obtaining latest UE's location for tracking purpose.

Figure 9.3.8-1: Event-trigger usage of location information procedure

1.	The location management server receives the latest location information of the UE as per the location report procedure described in clause 9.3.3.3.

2.	The location management server may optionally receive the location information of the UE from 3GPP core and/or the 3rd party location management server network. If the indication for supplementary location information is included in the subscription, then UE location information is obtained from the 3GPP core network and/or the 3rd party location management server. If the velocity information of target UE is requested, the location management server asks the location management client or 3GPP core network to report the target UE location data with the requested velocity information.

3.	Based on the configurations, e.g., subscription, periodical location information timer, location management server is triggered to report the latest user location information to VAL server. The location management server determines the location information of UE as received in steps 1 and 2, including the supplementary location information (if indicated). The location management server may report the location to the VAL server considering the location information received via non-3GPP positioning technologies (e.g. GNSS, Bluetooth), for instance, to improve the location accuracy. The location management server may reuse the stored and valid UE location information (if any) to report to the VAL server/location management client to reduce the response time when the trigger condition is time-based.

If the indication whether the statistic of target UE location is included in the request from the VAL server/location management client, the location management server calculates the received UE location data per temporal/spatial granularity as requested and then expose them to the VAL server/location management client for the valued-added location information.

4.	Same as step 5-9 of Figure 9.3.7-1.

4.	The location management server sends the location information report including the latest location information of one or more VAL users or VAL UEs to the VAL server or to the location management client that has previously configured. In addition, velocity of the requested VAL UEs may be included as part of the location information report.

5.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 5 can be skipped if not needed.

9.3.9	On-demand usage of location information procedure

The VAL server can request UE location information at any time by sending a location information request to the location management server, which may trigger location management server to immediately send the location report.

Figure 9.3.9-1 illustrates the high level procedure of on demand usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE.

Figure 9.3.9-1: On-demand usage of location information procedure

1.	VAL server sends a location information request to the location management server which may include the location QoS that contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50].

2.	The location management server checks if there is stored and valid UE location report (e.g. the timer for the stored UE location report is not expired) for the target UE. If available, it reuses the stored UE location information and reports it to the VAL server. If there isn’t, it acquires the latest location of the UEs being requested, by triggering an on-demand location report procedure as described in subclause 9.3.4, or from PLMN operator, and checks if the obtained location of the VAL UE could meet the location QoS requirements (if any).

3.	Then, location management server immediately sends the location information report including the latest location information acquired of one or more VAL users or VAL UEs. The location management server may report the location to the VAL server considering the location information received via non-3GPP positioning technologies (e.g. GNSS, Bluetooth), for instance, to improve the location accuracy.

4.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 4 can be skipped if not needed.

# /**************************** Next changes **********************/

9.3.11.1	General

The VAL server requests the location management server to monitor the location of the VAL UE in relation to an area of interest. The LMS fetches the VAL UE's location information periodically from 3GPP core network as specified in 3GPP TS 23.502 [11] and also, using the Location Information procedures specified in clause 9.3.7 and clause 9.3.10. With the periodic location information of the UE from 3GPP core network and SEAL procedures, the location management server evaluates the current location of the VAL UE in relation to the area of interest configured by the VAL server. If subscribed, the VAL server is notified by the location management server when the VAL UE relationship (e.g. inside or outside) to the area of interest changes along with current location information of the VAL UE. The area of interest can be predefined or mobile where the latter one is used to support dynamic geofencing function.

9.3.11.2	Monitoring Location Deviation procedure

Figure 9.3.11.2-1 describes the procedure for monitoring the VAL UE's location in a given area of interest.

Pre-condition:

-	The location management server authorized to consume the 3GPP core network service (Monitoring events provided by NEF as specified in 3GPP TS 23.502 [11]).

Figure 9.3.11.2-1: Monitoring VAL UE's location at a given location

1.	The VAL server sends Monitor Location Subscription Request to location management server including target VAL UE Identifier(s), predetermined or dynamic area of interest information, notification interval and notification URI where the VAL server intends to receive the notifications from LM server regarding target VAL UE's presence in a given area, interested notification information, dynamic geofencing conditions.

-	"Area of interest" is the location information, which the VAL server wishes to monitor the target VAL UE's location adherence. This parameter can include an area of interest information and other relevant parameters. In the case of dynamic area of interest, it includes a reference VAL UE Identifier and its proximity range.

-	"Notify_Interval" represents the periodic interval in which the location management server needs to notify target VAL UE's location information to the VAL server. When the target VAL UE moves away from the "Area of interest", then the location management server ignores the "Notify_Interval" and sends the location notification to the VAL server immediately.

-	"dynamic geofencing conditions" includes a temporal condition(s) (e.g. time of day and day of week), connectivity condition(s) (e.g. operator information, roaming status, access type) and/or a spatial condition(s) (e.g. location area(s)) for the reference VAL UE. For instance, the monitoring is valid when a reference UE is served by operator A without using satellite in location area X from 7:00am to 9:00am on working days.

2.	If dynamic area of interest information and dynamic geofencing conditions are received, the location management server requests 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11]) to monitor the reference UE location change, access type change and roaming status change, in order to determine spatial dynamicity in area of interest related to the target UE(s). If spatial condition is received, the location management server also requests 3GPP CN (i.e. NEF service for area of interest monitoring as specified in 3GPP TS 23.502 [11]) to know whether the reference UE is in the VAL server's predetermined area (e.g. PLMN x). The location management server will process the monitoring considering the received dynamic geofencing conditions for the reference UE.

3.	If dynamic area of interest information is received, the location management server requests 3GPP CN (i.e. NEF service for loss of connectivity as specified in 3GPP TS 23.502 [11]) for both target UE(s) and reference UE.

4a.	location management server processes the Area of interest information in the request, and then subscribes to target UE location monitoring as specified in 3GPP TS 23.502 [11] with appropriate parameters mapping. Based on the subscription, the location management server receives the target VAL UE location information periodically from the 3GPP core network. The location management server also invokes NEF service for Area of Interest monitoring as specified in clause 5.3.3 of 3GPP TS 23.256 [52] to know whether the target VAL UE is in the VAL server's predetermined or dynamic area of interest.

4b.	location management server shall use the Location information procedures as specified in clause 9.3.7 and clause 9.3.10, to periodically obtain the target VAL UE location information. Based on the geographic information from the VAL server, the location management server may determine to additionally include the positioning methods in SEAL LMS procedures to obtain location information.

4c.	LMS may periodically obtain the target VAL UE location information from the 3rd party location management server based on the geographic information from the VAL server.

NOTE 1:	How the LMS obtains the target UE location from the 3rd party location management server is out of scope of this specification.

NOTE 2:	Step 4a to 4c are performed for each target VAL UE if multiple target VAL UE Identifiers are provided by the VAL server.

5.	Location management server, after successful subscription according to steps 2~4, sends Monitor Location Subscription response, indicating that the location management server accepts VAL server's request and will monitor the location of the target VAL UE(s) to verify if the target VAL UE(s) is in the area of interest.

6a and 6b. Location management server processes the location information received from SEAL Location Information procedures, the 3GPP core network and/or the 3rd party location management server, and validates the information. If the location information is matching, then the location management server shall check if the VAL UE's current location is within the predetermined or dynamic area of interest received in step 1.

NOTE 3:	The location information received from the 3GPP core network and from location management clientg does not necessarily be exactly the same, the location management server can aggregate the location information if there is no big deviation.

6c.	Location management server verifies its determined target VAL UE location status (i.e. presence or absence with regard to the predetermined area of interest) with the retrieved 3GPP core network result from the NEF monitoring service for Area of Interest. The location management server will continue with step 7, step 8 and step 9 as applicable.

7.	If the location information received from location management client, the 3GPP core network and/or the 3rd party location management server do not match or the NEF reported Area of Interest monitoring result does not match with the determined target VAL UE location status (present/absence) in location management server, then the location management server shall consider the target VAL UE as outside from its specified area of interest and shall notify ("Notify Mismatch Location" message) the VAL server of the same, including target VAL UE ID and the location information from location management server, the 3GPP core network and/or the 3rd party location management server in the notification message.

8.	If the target VAL UE's current location is from location management client, the 3GPP core network and/or the 3rd party location management server matches, and either not in the predetermined area of interest received from VAL server in Monitor Location Subscription Request message or not within the proximity range of the reference UE, and such out of area of interest result is also aligned with the NEF reported area of interest monitoring result, then the location management server considers the target VAL UE as outside from its specified area of interest and shall notify the VAL server that the target VAL UE's current location is outside of area of interest and target VAL UE ID in "Notify Absence" message.

9.	When the target VAL UE's current location is either in the predetermined area of interest or in the proximity range of the reference UE, and such in area of interest result is also aligned with the NEF reported area of interest monitoring result, then the location management server shall notify ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the target VAL UE is within the area of interest, along with target VAL UE's current location information.

10.	If the location management server is notified by the 3GPP CN about loss of connectivity for the reference UE or target UE(s), then the location management server notifies ("Notify Unknown" message with detailed reason) the VAL server indicating that the location management server does not know whether the target UE is "Presence" or “Absence", therefore the dynamic geofencing monitoring for the impacted target UE(s) is suspended.

NOTE 4:	Upon "Notify Unknown", the VAL server can remind end user to check status for the reference/target UE, which is out of 3GPP scope.

Editor's note:	Detailed handling for “loss of connectity” in LM server is FFS. For instance, whether to address the case when reference UE and target UE(s) both lose connectivity (e.g., using UE-only operation for SL/Ranging).

Editor's note:	It is FFS whether to use SL/Ranging mechanism to further figure out the target UE(s) are present or absent in the proximity range of the reference UE when the reference UE (or target UE(s)) lost the connectivity with CN but the target UE(s) (or the reference UE) can still be reached via ProSe.

NOTE 5:	Notifications in step 7-10 can be subject to explicit notification indication from the VAL server, for instance, the VAL server can be interested to know only "Notify Absence" and "Notify Unknown".

9.3.12	Location area monitoring information procedure

9.3.12.1	Location area monitoring subscribe procedure

Figure 9.3.12.1-1 illustrates the high level procedure of location area monitoring subscription request. The same procedure can be applied for location management client and other SEAL servers that would like to subscribe to the list of UEs moving in or moving out of the specific location area. The subscribe request can be for a reference UE for which the subscriber is authorized to monitor location information.

The procedure can also be applied for Geofencing service which initiated via VAL server to subscribe to the location management server to report the Geofencing monitoring events.

Figure 9.3.12.1-1: Location area monitoring subscription procedure

1.	The VAL server sends a location area monitoring subscription request to the location management server to subscribe to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.14-1. The location information criteria may include the geographic location information or the VAL service area identified by the VAL service area ID where the UEs moving in or moving out to be monitored, or it may include reference UE information where in the UEs moving in or moving out of given application defined proximity range from the reference UE (target UE) to be monitored. The reference UE information may include VAL UE ID.

If the VAL server initiates the subscription for the Geofencing service, the Geofencing monitoring events (i.e. the UEs moving in/out of the pre-defined location area, the number of UEs moving in/out of the pre-defined location area, the time period of UEs enter/leave the pre-defined location area) can be included in the request message as specified in Table 9.3.2.14-1.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription request.

3.	The location management server replies with a location area monitoring subscription response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.15-1.

9.3.12.2	Location area monitoring subscribe modify procedure

Figure 9.3.12.2-1 illustrates the high level procedure of location area monitoring subscribe modify request. The same procedure can be applied for location management client and other SEAL servers that would like to modify the subscription to the list of UEs moving in or moving out of the specific location area. The subscribe modify request can be for a reference UE for which the subscriber is authorized to monitor location information.

The procedure can also be applied for Geofencing service that the VAL server would like to modify the subscription to the location management server to report the Geofencing monitoring events.

Figure 9.3.12.2-1: Location area monitoring subscription modify procedure

1.	The VAL server sends a location area monitoring subscription modify request to the location management server to modify the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.17-1.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription modification request.

3.	The location management server replies with a location area monitoring subscription modify response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.18-1.

9.3.12.3	Location area monitoring unsubscribe procedure

Figure 9.3.12.3-1 illustrates the high level procedure of location area monitoring unsubscribe request. The same procedure can be applied for location management client and other SEAL servers that would like to unsubscribe to the list of UEs moving in or moving out of the specific location area. The unsubscribe request can be for a reference UE for which the subscriber is authorized to monitor location information.

The procedure can also be applied for Geofencing service that the VAL server would like to unsubscribe to the location management server to report the Geofencing monitoring events.

Figure 9.3.12.3-1: Location area monitoring unsubscribe procedure

1.	The VAL server sends a location area monitoring unsubscrbe request to the location management server to unsubscribe to the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.19-1.

2.	The location management server replies with a location area monitoring unsubscrbe response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.20-1.

9.3.12.4	Location area monitoring notification procedure

Figure 9.3.12.4-1 illustrates the high level procedure of location area monitoring notification. The same procedure can be applied for location management client and other SEAL servers who have subscribe to the list of the UEs moving in or moving out of the specific location area.

The procedure can also be applied for Geofencing service which requests the location management server to report the Geofencing monitoring events.

Figure 9.3.12.4-1: Location are monitoring notification procedure

1.	One of the events occurs at the location management server as specified in the subscribe request. The location management server identifies the UEs which are moved into the area or moved out of the area based on their location data and time stamp of the location. The LMS may report the list of all UEs in the given location or UEs moved in and moved out.

When the Geofencing service is triggered, the location management server also checks whether the UE is inside/outside the area based on the location information criteria as defined in Table 9.3.2.14-1 and the received UE location data. Further, the LM Server statistics and reports the Geofencing monitoring events (e.g. the list of UEs moving in/out of the location area, the number of UEs moving in/out of the location area, the time period of UEs enter/leave the location area, etc.) to the VAL server.

2.	The location management server sends a location area monitoring notification to the VAL server. In the notification message, the location management server includes the information as specified in Table 9.3.2.16-1.

If the location management server didn't obtain any UE location information due to UE issues or network issues in step 1, it informs the VAL server with the proper reason (e.g., "UE cannot be connected") in the notification report.

3.	After received the Geofencing information report from the location management server, the VAL server may take some actions to inform the VAL UE (e.g. send the alarms or warnings to the UE).

NOTE:	The step 3 is out of 3GPP scope.

# /**************************** Next changes **********************/

9.3.14.2	Procedure of Location profiling for location service

The procedure includes the translation of the vertical request to a location profile and the derivation of the requested location information report.

Pre-condition: The location management server has configured a set of location service profiles.

Figure 9.3.14.2-1: Location profiling for location derivation and exposure

1.	The location management server receives a location reporting trigger from VAL server and activates a location reporting procedure as defined in clause 9.3.5.

2.	The fused location function determines a mapping of the vertical location services to a location service profile based on the location request information, the location profiles and the location capability of VAL UE which registered to the location management server before.

3a. The location management server requests from the LMC the location information of the target VAL UE optionally with the requested location information (the access type, positioning method) based on the determined location profiles in location management server obtained in step 2.

3b. The location management client responds to the location management server the location report based on the request.

4.	The location management server performs a location information request to one or more of the following (based on the location profile):

-	to GMLC directly or via NEF (as defined in TS 23.273 [50]), acting as AF.

-	to 3rd party location servers.

5.	The fused location function calculates the location information based on combined location reports from step 3~4 and checks whether the location reports fulfil the location profile requirements.

6-7.	If the requirements are not fulfilled, the location management server will repeat the step 3~5 iteratively to request location information and re-check whether the requirements are met or not.

8.	If the requirements are fulfilled, the location management server sends the location information report to the VAL server.

9.3.15	Location service registration procedure

Before the location management server requesting the location information for the target UE, the location management client may register the available location services to the location management server to report the UE's location capabilities.

Figure 9.3.15-1 illustrates the procedure of client-triggered location service registration.

Figure 9.3.15-1: Location service registration procedure

1.	The location management client of a VAL UE sends location service registration request to the location management server, with the identifier of the UE (e.g. GPSI) and UE-based location capabilities.

2.	The location management server checks authorization for the VAL UE's registration request.

3.	After successful authorization, location management server sends location service registration response to the location management client and stores the UE identifier information and location capabilities.

# /**************************** Next changes **********************/

9.3.18	Location service registration update procedure

Figure 9.3.18-1 illustrates the procedure of client-triggered location service registration  update. The location management client may update its supported location capabilities (e.g. location access type, position methods) which has registered to the location management server before.

Pre-condition:

The location management client has registered to the location management server.

Figure 9.3.18-1: Location service registration update procedure

1.	The location management client decides to trigger the registration  update request to location management server to update the location capabilities which have registered to the location management server before.

2.	The location management client sends location service registration update request to the location management server with the identifier of the VAL UE, the VAL service ID and updated location capabilities.

3.	The location management server checks the authorization for the VAL UE's registration  update request. If the authorization is successful, the location management server will update the UE's location capabilities received in step 2.

4.	The location management server sends location service registration  update response to the location management client.

9.3.19	Location service deregistration procedure

Figure 9.3.19-1 illustrates the procedure of client-triggered location service deregistration. By deresgistration, the location management client may deregister the available location services which have registered to the location management server before.

Pre-condition:

The location management client has registered to the location management server.

Figure 9.3.19-1: Location service deregistration procedure

1.	The location management client decides to trigger the deresgistration request to location management server to deregister the location services which have registered to the location management server before.

2.	The location management client sends location service deregistration request to the location management server with the identifier of the UE.

3.	The location management server sends location service deregistration response to the location management client and removes all registration information of the UE.

9.3.20	SEAL location management server provides adaptive configuration

Figure 9.3.20-1 illustrates procedure for SEAL LMS to provide adaptive location configuration suggestion to the VAL server for the VAL UE for which VAL server has requested for adaptive reporting (as specified in clause 9.3.5).

NOTE:	The procedure is applicable to SEAL location management clientg-2 (as specified in above clause 9.3.5) if SEAL location management clientg-2 has requested adaptive reporting.

Pre-conditions:

1.	The VAL server has requested to SEAL LMS to suggest adaptive configuration for VAL UE;

2.	The VAL server has provided endpoint information in the Location reporting trigger to the SEAL LMS.

3.	The SEAL LMS has received location reports from SEAL client and based on the different information the SEAL LMS as determined the new/adaptive location configuration.

Figure 9.3.20-1: Adaptive location reporting configuration provisioning

1.	The SEAL LMS sends a message to the VAL server (or authorized SEAL location management clientg). The message includes UE identity and suggested updates for location reporting trigger configuration.

2.	Upon receiving the request from the SEAL LMS, the VAL server (or authorized SEAL location management clientg) sends a response message. The response message includes whether the VAL server accepts or rejects the updated location reporting configuration suggestion or not. If the VAL server accepts the location reporting configuration as suggested by the SEAL LMS, then the SEAL LMS updates the location reporting configuration to the SEAL location management clientg of the target UE.

9.3.21	Location history querying procedure

9.3.21.1	Location tracing configuration create procedure

Figure 9.3.21.1-1 illustrates the high level procedure of creating location tracing configuration from VAL server to the location management server.

Figure 9.3.21.1-1: Location tracing configuration procedure

1.	The VAL server sends a location tracing configuration request to the location management server to configure the location tracing service in advance. The location tracing configuration request may contain the geographical area for history location tracing, events/conditions for triggering the location history report, location QoS, the identities for the UE and service, the time till when the SEAL LM needs to store the location history data and the required positioning method(s) , etc.

NOTE:	Step 1 can be also performed when the VAL server requires updating the location tracing configuration corresponding to location management server.

2.	The location management server checks whether the VAL server is authorized to request the location tracing configuration service. If the request is approved, the location management server stores the configuration received in step 1. If the request includes multiple UEs, the location management server creates and stores the contexts for each UE.

3.	The location management server sends the location tracing configuration responses to the VAL server.

9.3.21.2	Location history request procedure

Figure 9.3.21.2-1 illustrates the high level procedure of location history request which the VAL server triggers to query the history location data for some specific UEs in some specific conditions.

Pre-conditions:

1.	The location management server has obtained the UE location data from multiple sources as defined in clause 9.3.3 and clause 9.3.5 periodically based on the event triggers or conditions received in the request of step 1 in clause 9.3.21.1.

Figure 9.3.21.2-1: Location history request procedure

1.	The VAL server sends a location history request to the location management server with the specific conditions (e.g. the given location area, time/period, etc.) to query the history location data for a single UE or multiple UEs.

2.	The location management server checks the authorization for the request. If the request is authorized, the location management server matches the obtained UE location data with the location history request. Before the match, the location management server may transform the obtained UE location data into the same format of the location area information received in step 1 if needed.

3.	The location management server sends a location history response to the VAL server with the matched UE location information. If there is no matched UE location data, the failure reason can be included.9.3.21.3	Location tracing configuration cancel procedure

Figure 9.3.21.3-1 illustrates the high level procedure of location tracing configuration cancel request which the VAL server triggers to cancel the location tracing configuration for a single UE or multiple UEs.

Figure 9.3.21.3-1: Location tracing configuration cancel procedure

1.	If the VAL server decides to stop the location tracing services for the UE(s), it sends a location tracing configuration cancel request to the location management server with the UE identities.

2.	The location management server stops getting the UE’s location and deletes the stored UE location data and the location tracing configuration for the service.

3.	The location management server sends location tracing configuration cancel response to the VAL server.

9.3.22	Surrounding UEs retrieval procedure

Figure 9.3.22-1 illustrates the procedure of surrounding UE(s) retrieval. The location management server can use this procedure to know how many UE(s) are close to the requested UE hosting the location management clientg, and their identities.

Figure 9.3.22-1: Surrounding UE(s) retrieval procedure

1.	The location management server decides to Surrounding UE retrieval request to the location management clientg. The request may include surrounding UE retrieval method.

2.	The location management clientg obtains UE(s) close to itself. The location management clientg considers the surrounding UE retrieval method (if received) during the obtaining procedure.

3.	The location management clientg sends its surrounding UE(s) to the location management server in Surrouding UE retrieval response.

# /**************************** Next changes **********************/

9.4.8.1	General

API description: This API enables the VAL server to configure the list of VAL service area identifiers and the corresponding geographical co-ordinates with SEAL location management server.

9.5	Procedures and information flows for location management (Off-network)

# /**************************** Next changes **********************/

15.3	Service-based interfaces

Table 15.3-1 specifies the service-based interfaces supported by SEAL.

Table 15.3-1: Service-based interfaces supported by SEAL

| Service-based interface | Application functionEntity | Mapping server entity | APIs offered |
| --- | --- | --- | --- |
| Slm | Location management function | Location management server | Specified in subclause 9.4 |
| Sgm | Group management function | Group management server | Specified in subclause 10.4 |
| Scm | Configuration management function | Configuration management server | Specified in subclause 11.4 |
| Sim | Identity management function | Identity management server | Specified in subclause 12.4 |
| Skm | Key management function | Key management server | Specified in subclause 13.4 |
| Snrm | Network resource management function | Network resource management server | Specified in subclause 14.4 |
| Snsce | Network slice capability enablement function | Network slice capability enablement server | Specified in 3GPP TS 23.435 [40] |
| Sdd | Data delivery function | Data delivery server | Specified in 3GPP TS 23.433 [48] |
| Cccf | CAPIF core function | Not applicable | Specified in subclause 10 of 3GPP TS 23.222 [8] |
| Snm | Notification management function | Notification management server | Specified in subclause 17.4 |
| Sadae | Application data analytics enablement function | Application data analytics enablement server | Specified in 3GPP TS 23.436 [49] |