3GPP TSG-SA WG6 Meeting #64	S6-245240

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Huawei, Hisilicon

Title:	clarification on AI/ML member selection for AI/ML task transfer assist

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yajie Hu (huyajie1@huawei.com)

1. Introduction

This paper is introduced for AI/ML member selection for AI/ML task transfer assist

2. Reason for Change

Currently, the detail of select target AI/ML member for AI/ML task tranfer is missing in the specification, then the procedure is not clear.

Thus, this paper is introduced for providing detaile of AI/ML member selection for AI/ML task transfer assist.

3. Proposal

It is proposed to agree the following pCR to TS 23.482.

* * * First Change * * * *

8.6.2.1	Request AIMLE Server to assist AI/ML task transfer

Pre-conditions:

1.	The information of target AI/ML member (e.g., another AIMLE Client or VAL Client different from the source AI/ML member) is unknown at the source AI/ML member. The source AI/ML member decides that assistance from the AIMLE Server is needed.

2. 	The AIML E server obtains the target AI/ML member’ capability information as defined in clause 8.7.2

Figure 8.6.2.1-1: Procedure for requesting AIMLE Server assist AI/ML task transfer

Figure 8.6.2.1-1 illustrates the procedure for requesting AIMLE Server to assist the AI/ML task transfer. The detailed corresponding procedure is detailed as follows:

1.	The source AI/ML member (e.g., an AIMLE Client) sends an AI/ML task transfer assist request to AIMLE Server for assisting the AI/ML task transfer. The request message contains the information as specified in Table 8.6.3.2-1. The intermediate AI/ML model are obtained by source AI/ML member performing AI/ML model training. then the AIML E server determines the remaining AI/ML model training requirement based on the intermediate AI/ML operation results and AI/ML model traning requirement.

Editor’s Note:	The procedure can also be used for transfer AI/ML services (e.g., ML model performance evaluation, ML model update) between AI/ML members. The details are FFS.

2.	The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the request from the source AI/ML member. The AI/ML E server determines the target AI/ML member(s) bases on AI/ML member(s)’s capability and remaining AI/ML model training requirement. The remaining AI/ML model training requirement indicates the requirement for continuing to execute the AI/ML training task for the AI/ML model, including required AI/ML model remaining traing time, remaining resource, remaining number of iterations.

The AIMLE Server sends request to the selected target AI/ML member(s) for intermediate AI/ML operation information transfer, as defined in clause 8.6.2.2. Then, the AIMLE Server determines the transfer mode, i.e., transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member.

If target AI/ML member(s) is discovered, the AIMLE Server generates assistance information for the AI/ML task transfer from the source AI/ML member to the target AI/ML member(s) (e.g., time window for the transfer).

3.	The AIMLE Server sends an AI/ML task transfer assist response to the source AI/ML member with the information of the target AI/ML member(s), and assistance information generated in step 5. The response message contains the information as specified in Table 8.6.3.3-1.

8.6.3.2	AI/ML task transfer assist request

Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server.

Table 8.6.3.2-1: AI/ML task transfer assist request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE ID (the source AI/ML member). |
| VAL service ID | O | The identifier of the VAL service for which the assistance information is requested. |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| AI/ML model training requirement | O | Indicate the requirement for AI/ML model training including required training time, required training resource, required training number of iterations, |
| AI/ML intermediate model information | O | Indicate the AI/ML intermediate model information, including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used traning number of iterations. |
| Time validity | O | The time validity of the request. |

8.7.3	Information flows

8.7.3.1	AIMLE client registration request

Table 8.7.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration procedure.

Table 8.7.3.1-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.1-2. |
| List of supported services | O | Supported service information. |
| > List of service ID(s) | O | List of service ID(s) that the AIMLE client supports |
| > Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage). |

Editor’s Note: The addition of AIMLE client profile ID is FFS.

Table 8.7.3.1-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split). |
| AIMLE client schedule | O | Indicates the availability schedule of the AIMLE client for the AIML service. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g. ML application type, allowed resource usage level, available time for training, available resource for training, supported training number of iterations, etc,). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIML client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

* * * END of Change * * * *