3GPP TSG-SA WG6 Meeting #64	S6-245243

Orlando, USA, 18th – 22nd November 2024	(revision of S6-24xxxx)

Source:	Huawei, Hisilicon

Title:	SEALDD enabled reliable guarantee data transmission connection establishment

Spec:	3GPP TS 23.700-35 v0.1.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yajie Hu (huyajie1@huawei.com)

1. Introduction

This contribution is proposed to corresponding solution corresponding Existing SEAL service #2.

2. Reason for Change

Currently, the API defined in SEALDD can be improved to expose a better service to the 3rd party. Thus, as for the data transmission connection establishment API should be decoupled to difference performance level, and the SEALDD enabled reliable guarantee data transmission connection establishment procedure is provided in this paper to provide a a reliable data transmission service to application.

3. Proposal

It is proposed to agree the following pCR to TR 23.700-35.

* * * First Change * * * *

8.3	Solution#3

8.3.1	Solution description

The 3GPP TS 23.433 [2] should capture following enhancement as this solution proposed.

8.3.1.1 SEALDD regular connection management

8.3.1.1.1	General

The following clauses specify procedures, information flow and APIs for establishing an SEALDD enabled end-to-end connection between VAL client and VAL server. The end-to-end connection is uniquely identified in the SEALDD layer by the SEALDD-UU flow ID. The specific procedures detailed in the subsequent clauses are for cases in which the SEALDD regular connection is used respectively for application signalling, application data delivery initiated by VAL server, and application data delivery initiated based on DD policy.

As for the signalling transmission connection establishment, the SEALDD enabler layer can determine to establish different signalling transmission connection (i.e. best effort signalling transmission connection, reliable signalling transmission connection) for the VAL server based on the signalling transmission requirement.

8.3.1.1.2	Procedure

8.3.1.1.2.1	SEALDD enabled best effort signalling transmission connection establishment procedure

Figure 8.3.1.1.2.1-1 illustrates the procedure for best effort signalling transmission connection establishment. In this clause, considering VAL server’s requirement on signalling transmission is not strict, then the VAL server can invoke best effort signalling transmission connection establishment API to enable the connection establishment.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 8.3.1.1.2.1-1: SEALDD best effort signalling transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for application signalling transfer and allocates address as SEALDD-S connection information for receiving the application signalling packets from SEALDD server. The VAL server sends SEALDD enabled best effort signalling transmission connection establishment request to the SEALDD server. The SEALDD enabled best effort signalling transmission connection establishment request includes the VAL server ID, VAL service ID to identify the VAL application traffic, the SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address used for SEALDD traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD enabled best effort signalling transmission connection establishment response.

Step 3 - step 8 are same as 3GPP TS 23.433[2] clause 9.2.2.1 step 2 - step 8.

8.3.1.1.2.Y	SEALDD enabled reliable guarantee data transmission connection establishment procedure

Figure 8.3.1.1.2.Y -1 illustrate the procedure for establishing reliable guarantee data transmission connection. The reliable guarantee data transmission connection establishment feature can provide VAL server with low latency and high reliability data transmission.

This procedure is similar to the SEALDD enabled Bandwidth guarantee data transmission connection establishment procedure, the differences are as follows:

Figure 8.3.1.1.2.Y -1: SEALDD enabled reliable guarantee data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for reliable guarantee application data traffic transmission, then the VAL server send SEALDD enabled Reliable guarantee data transmission connection establishment request to the SEALDD server. The request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S connection information of the VAL server side, and optionally, the Reliable guarantee data transmission requirement for the application data traffic.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address of the SEALDD server to receive the application data traffic from the VAL server for application data transfer. The SEALDD server responds VAL server with a SEALDD enabled Bandwidth guarantee response.

The SEALDD server determines the QoS requirement based on the application data transmission requirement provided by VAL server. Then SEALDD server may invoke 5GC function as described in clause 9.2.2.2 step2 in order to establish reliable guarantee data transmission connection.

Step 3-9 are same as the step3-9 in clause 9.2.2.2, the only difference is that athe SEALDD server and SEALDD client establish reliable guarantee data transmission connection to transmit application data based on the Data transmission requirement provided by the VAL server.

8.3.1.1.3	Signalling flow

8.3.1.1.3.1	SEALDD enabled best effort signalling transmission connection establishment request

Table 8.3.1.1.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the best effort application signalling traffic transmission service.

Table 8.3.1.1.3.1-1: SEALDD enabled Best effort transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL) of the VAL server to receive the signalling traffic from the SEALDD server |
| Requested 5GS Delay | O | The Requested 5GS Delay provided by the VAL server |

8.3.1.1.3.2	SEALDD enabled best effort transmission connection establishment response

Table 8.3.1.1.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the best effort application signalling traffic transmission.

Table 8.3.1.1.3.2-1: SEALDD enabled best effort transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S connection information | O | Address information (e.g., IP address and/or port, URL) of the SEALDD server to receive the signalling traffic from the VAL server |
| Cause | O See NOTE | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE: The IE is only present if the Result is failure. |  |  |

8.3.1.1.3.3	SEALDD signalling transmission connection establishment initiated by SEALDD client request

Table 8.3.1.1.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the SEALDD signalling transmission connection establishment.

Table 8.3.1.1.3.3-1: SEALDD signalling transmission connection establishment initiated by SEALDD client request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable for SEALDD server side initiated request. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. |  |  |

8.3.1.1.3.4	SEALDD regular signalling transmission connection establishment initiated by SEALDD client response

Table 8.3.1.1.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the SEALDD signalling transmission connection establishment initiated by SEALDD client.

Table 8.3.1.1.3.4-1: SEALDD signalling transmission connection establishment initiated by SEALDD client response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure |  |  |

8.3.1.1.3.B1	SEALDD enabled reliable guarantee data transmission connection establishment request

Table 8.3.1.1.3.B1-1 describes the information flow from the VAL server to the SEALDD server for requesting the reliable guarantee data transmission service.

Table 8.3.1.1.3.B1-1: SEALDD enabled Reliable guarantee data transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| UEID/ VAL user ID | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g. IP address and/or port, URL) of VAL server to receive the signalling traffic from the VAL server |
| Data transmission requirement | M | The data transmission requirement |
| > VAL server’s total bandwidth limit | M (See NOTE) | The total bandwidth limit of VAL server, including UL/DL |
| > VAL users’ bandwidth limit | M (See NOTE) | The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL |
| > Requested Priority | O | The priorigy requested by the VAL server |
| > Maximum Burst Size | O | The Maximum Burst Size provided by the VAL server |
| > Requested 5GS Delay | M | The Requested 5GS Delay provided by the VAL server |
| > Requested Maximum Bitrate | O | The Requested Maximum Bitrate provided by the VAL server |
| > Requested Guaranteed Bitrate | M | The Requested Guaranteed Bitrate provided by the VAL server |
| > Requested Packet Error Rate | O | The Requested Packet Error Rate provided by the VAL server |
| NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users. |  |  |

8.3.1.1.3.B1	SEALDD enabled Reliable guarantee data transmission connection establishment response

Table 8.3.1.1.3.B1-1 describes the information flow from the SEALDD server to the VAL server for responding to reliable guarantee application data traffic transmission.

Table 8.3.1.1.3.B1-1: SEALDD enabled reliable guarantee data transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicate the bandwidth guarantee data transmission connection establish success or failure. |
| SEALDD-S connection information | O | Address information (e.g. IP address and/or port, URL) of SEALDD server to receive the signalling traffic from the SEALDD server |
| Cause | O See NOTE | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * END of Change * * * *