3GPP TSG-SA WG6 Meeting #63	S6-245279

Hyderabad, India, 14th – 18th Oct 2024	(revision of S6-245xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0358 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE unavailability information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Study in TR 23.700-01 v19.0 has KI#3 for Supporting Satellite access with discontinuous coverage and has been concluded as follows: Solution #AE3 resolves open issue about application enabler (SEAL Configuration management server) obtaining the satellite coverage availability information and to retrieve unavailability period for UE (either from UE or from the 5GS), for supporting satellite discontinuous coverage. Further the solution also resolves open issue about leveraging UE unavailability period information for minimizing the service impact or adjusting the application behaviour by the SEALDD server. There is dependency with SA2 for obtaining exposure of loss of coverage event information. Solution #AE3 can be considered for normative work. So the proposal is to address the KI#3 according to Sol#AE3 in the TS for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding procedure for getting and notifying UE unavailability information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service may be impacted when delivering via satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New{21.x, 21.x.1, 21.x.2, 21.y, 21.y.1, 21.y.2, 21.y.3} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 21.x	UE unavailability period information

### 21.x.1	General

The following clauses describe the procedure for obtaining UE unavailability period information and consuming by the VAL server or SEALDD server.

### 21.x.2	Procedures for AS handling UE unavailability period information

The procedure for SEAL NRM server obtaining the UE unavailability period information and notifying the VAL server or SEALDD server, for optimal service under a discontinuous coverage pattern is illustrated in figure 21.x.2-1.

Pre-conditions:

-	The VAL UE has registered to the 3GPP network via satellite access as defined in clause 4.2.2.2.2 of 3GPP TS 23.502 [11].

-	5GC is aware of Satellite coverage availability information (SCAI) and VAL UE location.

-	VAL UE does not support SCAI provisioning or VAL UE has not received SCAI configuration from the 5GC.

-	VAL server (or SEALDD server) has requested NRM server to provide UE unavailability information.

Figure 21.x.2-1: Procedures for AS handling UE unavailability period information

1.	Upon receiving request to provide UE unavailability information, the NRM server sends get UE unavailability request to the 5GC, including the UE/User identity and service identity for which UE has satellite access. SEAL NRM server can subscribe with 5GC to loss of coverage event subscription to get notified about the unavailability period or unavailability duration when certain events occur e.g. start of unavailability period or before certain duration in a particular location, as described in clause 5.4.1.4 of 3GPP TS 23.501 [5].

2.	5GC checks if UE supports satellite access and then determines UE unavailability e.g. based on SCAI information and UE location.

3.	5GC sends get UE unavailability response including the UE unavailability information to the NRM server. Or when an event is triggered in the 5GC corresponding to the subscription in step 2, 5GC notifies to NRM server the UE unavailability period information e.g. duration/start time, as described in clause 5.4.1.4 of 3GPP TS 23.501 [5].

4.	NRM server sends the UE unavailability period information to the requesting entity (e.g. VAL server or SEALDD server).

5.	VAL server (or SEALDD server) uses the UE unavailability information for adjusting the application behaviour e.g. to trigger the downlink data delivery, bulky data exchanges can be deferred and planned during longer connectivity intervals, giving precedence to data suiting short connectivity intervals.

* * * Next Change * * * *

## 21.y	Information flows

#### 21.y.1	Get UE unavailability request

Table 21.y.1-1 describes the information flow for NRM server to get the UE unavailability information from 5GC.

Table 21.y.1-1: Get UE unavailability request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | M | List of VAL UEs whose unavailability is requested. |
| VAL service ID | M | Identity of the VAL service for which UE's unavailability is requested. |

* * * Next Change * * * *

#### 21.y.2	Get UE unavailability response

Table 21.y.2-1 describes the information flow for the response from 5GC to the NRM server with UE unavailability information.

Table 21.y.2-1: Get UE unavailability response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the request result. |
| List of UE unavailability information (NOTE) | O | List of UE unavailability information |
| > UE ID (NOTE) | O | Indicates the UE ID for which the unavailability period information is being provided. |
| > UE unavailability information (NOTE) | O | Indicates the UE unavailability period information for a service and UE location. |
| NOTE: These IEs are present only when the result is successful. |  |  |

* * * Next Change * * * *

#### 21.y.3	Notify UE unavailability information

Table 21.y.3-1 describes the information flow for the NRM server notifying UE unavailability information to the requesting entity (VAL server or SEALDD server).

Table 21.y.3-1: Notify UE unavailability information

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | O | Indicates the UE ID for which the unavailability period information is being provided. |
| UE unavailability information | O | Indicates the UE unavailability period information for a service and UE location. |

* * * End of Changes * * * *