3GPP TSG-SA WG6 Meeting #64	S6-245289

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Samsung

Title:	Clarification on resource owner consent information in CAPIF

Agenda Item:	4.2

Contact:	Basavaraj (Basu) Pattan, (basavarajjp@samsung.com)

Abstract: Clarification on resource owner consent information in CAPIF

With reference to the GSMA LS (S6-242012) titled “LS on clarifications on consent management”, question 2, requested  clarification on relationship between CAPIF RNAA and UDM’s user consent information GSMA and plan/roadmap for unified approach.

3GPP SA3 WG have shared their inputs (in S6-243008) to SA on GSMA LS. In their response, for question 2 (see below), SA3 clarifies that user consent information stored in UDM/UDR and CCF to be independent from each other under different context of resource usage.

Q2. What is the relationship between CAPIF RNAA and UDM’s user consent information? Is there any plan/roadmap for a unified approach? Answer: User Consent Subscription Data (UcSubscriptionData) is specified in 3GPP TS 29.503 as permanent subscription data stored in the UDR. This data can be retrieved from UDR by UDM and from UDM by any other authorized NF (e.g. NEF). Permanent subscription data can be modified only by provisioning/administration means locally at the UDR. This framework describes therefore a static handling of user consent in UDM. Hence consent framework in Annex V is not suitable for consent control in runtime at the CAPIF RNAA layer. The CAPIF RNAA is about subscriber’s authorization/permission to a requesting AF enabling the AF to access subscriber resources. The consent mechanism enabled by RNAA, has different granularity in support of different use cases for exposure of subscriber related network resources via APIs. It is important to understand that the same data can be processed for different purposes and under different legal bases, but data controllers cannot swap to another legal basis to continue the consent-governed processing, when consent is revoked; if consent is revoked for a purpose, the data controller must honour the revocation. Hence, keeping information stored in UDM/UDR and CCF to be independent from each other is important.

Observation: SA3 response clarifies user consent information stored in UDM/UDR and CCF to be independent for different context of resource usage. However, such clarification is not present explcitly in 3GPP specifications. From 3GPP TS 23.222 readers perspective, it is better to explictly clarify the relation between the two storages of user consent. Otherwise, the same question might arise again later and for different readers.

Conclusion: Clarify the relation between user consent at CCF and UDM, as an informative note in 3GPP TS 23.222 (shown below). Send inputs to SA for GSMA LS response, aligning to the informative note agreed for CAPIF.

NOTE: 	The resource owner consent information stored in CCF can be independent of the user consent information stored in user subscription data at UDM/UDR, under different context of resource usage. Any synergy needed between CCF and UDM for the same context of resource usage while processing service APIs requests is not specified in the current release of the specification.