3GPP TSG-SA WG6 Meeting #62	S6-243473

Maastricht, Netherlands 19th – 23rd August 2024	(revision of S6-243335)

Source:	Nokia

Title:	Updates to clause 8 on business relationships

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Updates to clause 8 on business relationships.

2. Reason for Change

In the TS 23.222, the following description is specified for RNAA business relationship.

## 5.2 CAPIF business relationships for RNAA Figure 5.2-1 shows the CAPIF business relationships for the resource owner-aware northbound API access (RNAA). Figure 5.2-1: CAPIF business relationships for RNAA The business relationships the API invoker, the CAPIF provider, and the API provider follow the description in the clause 5.1. In addition to them, the resource owner is an entity capable of granting access to a protected resource related to the resource exposed by the API provider. The API invoker and the resource owner can be the same entity or separate entities. In the current release, the resource owner is a user of a UE and can provide authorization information using the UE. NOTE: In the current release, both the CAPIF provider and the API provider should belong to the same organization (e.g., PLMN operator) and the service API arrangement is not required explicitly.

From Rel.19, the API provider can be a 3rd party CAPIF provider is to be introduced.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0

* * * First Change * * * *

# 8	Business Relationships

The following updates (in bold) are proposed to the RNAA business relationships specified in clause 5.2 of 3GPP TS 23.222 [2].

## 5.2 CAPIF business relationships for RNAA Figure 5.2-1 shows the CAPIF business relationships for the resource owner-aware northbound API access (RNAA). Figure 5.2-1: CAPIF business relationships for RNAA The business relationships the API invoker, the CAPIF provider, and the API provider follow the description in the clause 5.1. In addition to them, the resource owner is an entity capable of granting access to a protected resource related to the resource exposed by the API provider. The API invoker and the resource owner can be the same entity or separate entities. In the current release, the resource owner is a user of a UE and can provide authorization information using the UE. The CAPIF provider and the API provider can belong to the same organization (e.g. PLMN operator), in which case the service API arrangement is not required explicitly. The CAPIF provider and the API provider (e.g. 3rd party CAPIF provider or 3rd party API provider) can belong to different organizations, in which case the service API arrangement is required. The specification of service API arrangement is out of scope of 3GPP. NOTE: In the current release, both the CAPIF provider and the API provider should belong to the same organization (e.g., PLMN operator) and the service API arrangement is not required explicitly.