3GPP TSG-SA WG6 Meeting #62	S6-243594

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243215)

Source:	Samsung

Title:	Pseudo-CR on model distributon for federated learning

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

TR 23.700-82 studied the procedure (Sol#24) for application servers initiating the AIML model distribution incase of federated learning. Model ditribution request for federated learning is initiated by the application server. The AIMLE server discovers the list of AIMLE clients which can particiapte in the FL and distributes the model to those client. Also the AIMLE server continuously monitor for the AIML clients which can particapate in the FL and distributes the model to those clients.

2. Reason for Change

This pCR proposes the solution for the normative work.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	ML model training for federated learning

### 8.x.1	General

This clause describes the procedure for supporting the ML model training for federated/distributed learning. The VAL server requests the AIMLE server for the federated/distributed learning. AIMLE consumer requests the AIMLE server to train a ML model by specifiying the type of learning, details about the ML model to be trained and criteria for selecting the FL members who can participate in the federated learning.

### 8.x.2	Procedure for ML model training for federated learning

Figure 8.x.2-1 illustrates the procedure for AIMLE server to support federated/distributed learning based on the request from the VAL server.

Figure 8.x.2-1: ML model training for federated learning

1.	The VAL server sends ML model federated learning request to AIML enablement server, requesting to assist in its model training.This request consists of the type of learning to be performed, criteria to select the members, ML model information, indication of whether to receive notification whenever the list of FL/ML members changes, time period of when to train the model etc.

2.	The AIML enablement server checks whether the VAL server is authorized to perform the ML model federated learning request.

3.	If the VAL server is authorized, AIME server returns the success response, otherwise a failure response indication the reason for failure.

* * * Next Change * * * *

### 8.x.3	Information flows

#### 8.x.3.1	ML model federated learning request

Table 8.x.3.1-1 describes the information flow from the VAL server to the AIMLE server for requesting the ML model training.

Table 8.x.3.1-1: ML model federated learning request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identifty of the VAL server performing the request |
| Training type | M | Idenifies whether the VFL or HFL training to be performed |
| Member selection criteria | M | Identifies the criteria that needs to be continuously monitored for selecting the FL/ML clients (e.g AIMLE clients in a particular location, member availability duration, new FL/ML clients registering with the required capabilities). |
| ML model information | M | Identifes the ML model that has to be distributed to the selected FL/ML clients for training. This information consists of model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| Time of distribution | O | Indicates the time or time period when the model can be distributed to FL/ML clients for training (e.g. date and time or day of week or combination of date, time and day of week |
| Members update notification | O | Indicates whether requestor needs to be notified whenever there is update related to new FL/ML clients selected or de-selected.. |
| List of FL/ML clients | O | List of FL/ML member clients which are currently participating in federated/distributed learning. |
| Notification Target URI | O (NOTE) | Target URI where the VAL server wishes to receive the notifications whenever the list of FL/ML clients selected changes. |
| NOTE: This IE shall be present if the Members update notification IE is present and is set to true. |  |  |

Editor's Note: Additional information elements that are required specifically for HFL and VFL training type are FFS.

#### 8.x.3.2	ML model federated learning response

Table 8.x.3.2-1 describes the information flow of ML model federated learning response from the AIMLE server to the VAL server.

Table 8.x.3.3-1: Model distribution response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the model distribution request was successful |
| Failure response | O (NOTE) | Indicates that the model distribution request was failure |
| > Cause | M | Reason for the failure |
| NOTE : At least one of these information elements shall be present |  |  |

* * * End Change * * * *