3GPP TSG-SA WG6 Meeting #62	S6-2433368

Maastricht, Netherlands 19th – 23rd August 2024	(revision of S6-243114)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0125 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of exposing network congestion information for MC services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Network congestion is an important aspect in MC services for meeting MC’s KPIs. Therefore, this CR introduces the support of exposing 5GS congestion information using L4S. To enable this capability, the MC service server requests the 5G system to enable the exposure of network congestion using L4S, which is done by marking the ECN bits in the IP header at the IP packets sent between the MC service server and MC service UE. As a result, the application layer bit rate is adjusted accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added to enable the support the exposure of 5GS congestion information using L4S. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No actions are taken due to network congestion, and MC’s KPIs may not be met during network congestion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3 and 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 7.x mentioned in this CR is available in CR number 0127 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ECN	Explicit Congestion Notification

L4S	Low Latency, Low Loss and Scalable Throughput

NPN	Non-Public Network

PNI-NPN	Public Network Integrated Non-Public Network

SNPN	Stand-alone Non-Public Network

# * * * * Second change * * * *

## 4.x	Support of the 5GS congestion information exposure using L4S

Based on the request from the MC system, the 5G system can expose network congestion information on the media path between the MC service server and MC service UE to enable reacting accordingly, e.g., adjust the application layer bit rate at the media sender side.

The 5G System can support Explicit Congestion Notification (ECN) marking for Low Latency, Low Loss and Scalable Throughput (L4S) support as defined in 3GPP TS 23.501 [7], clause 5.37.3. When supported and enabled, the 5G System is applying ECN marking for L4S on a per QoS flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS flows.

To support this feature on a service data flow, the MC service client and MC service server are capable of supporting the L4S handling including e.g., parsing the L4S ECN mark in the IP header, reporting the L4S feedback about downlink and/or uplink direction. The MC service server enables this feature by initiating the request for L4S marking towards the network (either directly towards PCF or indirectly via NEF).

Related procedures are specified in clause 7.x.

NOTE 1:	The activation of ECN marking can also be triggered via the 5G core (SMF or PCF) based on either a dynamic or a predefined PCC rule, or dynamically upon the detection of L4S traffic in the IP header.

NOTE 2:	The details related to MC service server and MC service UE upon reception of congestion information, e.g., including media handling, codec and bit rate negotiation are outside the scope of the current document.

# * * * * End of changes * * * *