3GPP TSG-SA WG6 Meeting #62	S6-243466

Maastricht, Netherlands, 19th – 23rd August 2024	revision of S6-243090

Source:	ChinaTelecom

Title:	Add new solution for KI#1 about obtaining resource owner consent

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solution for KI#1 about obtaining resource owner consent.

2. Reason for Change

This contribution provides solution for the CAPIF core function to obtain resource owner consent.

From the description of authorization mechanism in TS 33.122, CCF will check and store the resource owner consent information. However, there is no relevant statements about how to get the authorization from resource owner. Thus, a procedure for obtaining resource owner consent is proposed in this paper.

6.5.3.2 Authorization using oauth client credential flow If client credential flow is used for authorization of the API invoker by the AEF, the procedures in RFC 6749 [4] shall be followed with the following profile: - The access token request message may include the resource owner ID. NOTE 1: If the API invoker is on a UE, the CCF obtains its GPSI during authentication. Editor’s note: the mapping of API Invoker ID and GPSI is left for stage 3. - The CCF shall check whether the API invoker is entitled to consume the API and allowed to access the resources of the resource owner, by using authorization information available in the CCF. - If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. If the API invoker is an AF not on a UE, the check is omitted. NOTE 2: How to get the authorization from the resource owner and store it in the CCF is out of scope of the present document.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #X | X |  |  |  |  |  |  |

* * * Second Change * * * *

6.X	Solution #X: Obtaining Resource Owner Consent

6.X.1	Solution description

6.X.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

From the description of authorization mechanism in TS 33.122, CCF will check and store the resource owner consent information. However, there is no relevant statements about how to get the authorization from resource owner. This solution enables the CCF to obtain resource owner consents which are used to authorize subsequent service API invocation.

6.X.1.2		Procedure for obtaining resource owner consent

Pre-condition:

1.	The service API access requires obtaining consent from resource owner.

Figure 6.X.1.2-1: Procedure for obtaining resource owner consent

1. The CAPIF core function sends a resource owner consent request to the resource owner client with its CAPIF core function ID when it determines to obtain the resource owner consent. For example, in order to reduce real-time interruptions to the resource owner and reduce waiting time for API invokers to obtain authorization, the CCF is configured to obtain consents in advance.

2. The resource owner client responds to the CAPIF core function with its resource owner ID and consents.

3. The CAPIF core function stores the resource owner consents with the resource owner ID.

6.X.2	Architecture Impacts

This solution is based on the existing architecture of CAPIF as described in 3GPP TS 23.222 [2] and relates to the CAPIF-8 reference point.

6.X.3	Corresponding APIs

This solution impacts the CAPIF-8 interface with new APIs. The APIs will be specified in normative work.

### 6.X.4	Solution evaluation

This solution addresses open issue 1 of KI#1 and provides a mechanism for the CCF to obtain resource owner consents which are used to authorize subsequent service API invocation. This solution can reduce waiting time for API invokers to obtain authorization. Besides, this solution support obtaining bulk resource owner consent information.