3GPP TSG-SA WG6 Meeting #62	S6-243516

Maastricht, Netherlands, 19th – 23rd August 2024	revision of S6-243475

Source:	ChinaTelecom

Title:	Update to Solution #9

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution propose to update solution #9 to address the 4th open issue of KI #1.

2. Reason for Change

This contribution propose to update solution #9 to address the 4th open issue of KI #1, i.e. How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.5.0.

* * * First Change * * * *

6.10	Solution #9: Support more API invoker info in RNAA

6.10.1	Solution description

6.10.1.1	General

This solution provides more API invoker info related to RNAA so that the resource owner can know better about which user and/or which service (and its provider) is asking for sensitive information (e.g. location) of the resource owner.

6.10.1.3	Procedures

6.10.1.3.1	Authorization

Figure 6.10.1.3.1-1: Procedure for the API invoker obtaining authorization for service API access

The procedure is similar to clause 8.11.3 of 3GPP TS 23.222 [2] with the following difference:

-	In step 1, more information about API invoker (application service info like ASP id, application id) and purpose of data processing (e.g. location resource to be used for tracking on the map) can be sent from API invoker to CCF during service API authorization procedure.

NOTE 1:	Application service info are applicable for both AF side API invoker and UE side API invoker.

-	In step 2, if received, the CCF additionally verifies application service info

NOTE 2:	To support RNAA, the CCF checks whether the API invoker is permitted to access the requested service API based on the API invoker's subscription information and resource owner consent.

3516

6.10.2	Architecture Impacts

Existing CAPIF architecture in 3GPP TS 23.222 [2] is used to accommodate new impact to CAPIF-1/1e reference point.

The CAPIF architecture shall support CAPIF-8 reference point between CCF and resource owner.

6.10.3	Corresponding APIs

Existing CAPIF service API authorization API is enhanced.

6.10.4	Solution evaluation

This solution addresses KI#1 with more granular information about API invoker in the context of RNAA. It also addresses the purpose of data processing in KI#1. The impacted CAPIF procedure includes service API authorization.

* * * Next Change * * * *