3GPP TSG-SA WG6 Meeting #62	S6-242662

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243049, S6-243593)

Source:	InterDigital Inc.

Title:	Pseudo-CR on AIML model distribution

Spec:	3GPP TS 23.482 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This CR brings normative text on AIML model distribution.

2. Reason for Change

The following agreement has been approved in TR 23.700-82 on transfer of AI/ML models.

The following principles will be considered in the normative phase for KI #5.

Transfer of AI/ML models:

-	Solution #24 will be considered for distribution of AIML models (request/response).

-	Solution #25 will be considered for distribution of AIML models (subscribe/notify).

NOTE 2: Possible alignment of the request/response and subscribe/notify models in Solution #24 and Solution #25 will be considered in normative phase.

Solution #24 and Solution #25 have a complementary relationship.

3. Conclusions

This pCR provides normative text for KI#5 Solution #24 and solution #25.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.1.0.

* * * First Change * * * *

## 8.x	Model Retrieval

### 8.x.1	General

Model retrieval enable applications consuming services from the AI/ML application enablement layer to retrieve AI/ML models available with the enablement layer.

Model retrieval enables an AIMLE client or VAL server to obtain AI/ML models of interest. The retrieval of AI/ML models is based on matching model retrieval filters provided in the request.

Model retrieval subscription enables an AIMLE client or VAL server to receive notifications about AI/ML models of interest. The subscription is based on matching model retrieval filters provided in the subscription request.

### 8.x.2	Procedure

#### 8.x.2.1	General

Following procedures are supported for model retrieval:

-	Model retrieval request-response procedure;

-	Model subscribe-notify procedures for retrieval of models, comprising:

-	Subscription procedure;

-	Subscription update procedure; and

-	Unsubscribe procedure;

#### 8.x.2.2	Model retrieval

Figure 8.x.2.2-1 illustrates the procedure for an AIMLE client or VAL server to discover and obtain AI/ML models from the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has  received information (e.g. URI, IP address) related to the AIMLE Server;

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.x.2.2-1: Model retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends a model retrieval request to the AIMLE server. The request includes the requestor identifier along with security credentials and may include model retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

If the requestor is authorized, the AIMLE server may determine if the requested model is available with the AIMLE server based on model retrieval filters.

Editor’s Note: Details of AIMLE server interactions with the ML model repository are FFS.

3.	The AIMLE server sends a model retrieval response to the requestor. If the AIMLE server has determined AI/ML models (e.g., locally, or from the ML repository), the response includes an indication of success and may include the AI/ML models. Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including AI/ML models, the requestor can store the AI/ML models and may provide the AI/ML model to the VAL client(s) if the requestor is the AIMLE client.

#### 8.x.2.3	Model retrieval subscription

##### 8.x.2.3.1	General

Clause 8.x.2.3.2 and clause 8.x.2.3.3 together illustrate the model retrieval subscribe/notify model.

Clause 8.x.2.3.4 illustrates the model retrieval subscription update procedure.

Clause 8.x.2.3.5 illustrates the model retrieval unsubscribe procedure.

##### 8.x.2.3.2	Subscribe

Figure 8.x.2.3.2-1 illustrates the procedure for an AIMLE client or VAL server to subscribe with the AIMLE server to be notified of retrieval of AI/ML models.

Pre-conditions:

- The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.x.2.3.2-1: Model retrieval subscription

1.	The requestor (e.g., AIMLE client or VAL server) sends a model retrieval subscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and may include model retrieval filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server creates the subscription and stores the subscription information.

3.	The AIMLE server sends a model retrieval subscribe response to the requestor. If the AIMLE server has created the subscription, the response includes an indication of success, the subscription identity and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the model retrieval subscription expires. If the AIMLE server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.3.3	Notify

Figure 8.x.2.3.3-1 illustrates the model retrieval  notify operation between the AIMLE server and an AIMLE client or VAL server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for model retrieval with the AIMLE Server;

Figure 8.x.2.3.3-1: Model retrieval notification

1.	An event occurs at the AIMLE server that satisfies trigger conditions for notifying a subscriber (e.g., AIMLE client or VAL server). The event may be the detection of availability of new AI/ML model(s) (e.g., new model or new revision of a model) that satisfies the model retrieval filters of the subscription.

Editor’s Note: Details of AIMLE server interactions with the ML model repository to detect trigger conditions are FFS.

2.	The AIMLE server sends a model retrieval  notification to the requestor indicating newly available AI/ML models. The notification includes a subscription identity and newly available AI/ML model(s).

##### 8.x.2.3.4	Subscription update

Figure 8.x.2.3.4-1 illustrates the procedure for an AIMLE client or VAL server to update a subscription with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for model retrieval with the AIMLE Server;

Figure 8.x.2.3.4-1: Model retrieval subscription update

1.	The requestor (e.g., AIMLE client or VAL server) sends a model retrieval update request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier and may include model retrieval filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the subscription information.

3.	The AIMLE server sends a model retrieval subscription update response to the requestor. If the AIMLE server has updated the subscription, the response includes an indication of success and may include an expiration time. To maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the model retrieval subscription expires. If the AIMLE server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.3.5	Unsubscribe

Figure 8.x.2.3.5-1 illustrates the procedure for an AIMLE client or VAL server to unsubscribe with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for model retrieval with the AIMLE Server;

Figure 8.x.2.3.5-1: Model retrieval unsubscribe

1.	The requestor (e.g., AIMLE client or VAL server) sends a model retrieval unsubscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server cancels the subscription.

3.	The AIMLE server sends a model retrieval unsubscribe response to the requestor. If the AIMLE server has canceled the subscription, the response includes an indication of success. If the AIMLE server has not canceled the subscription, the response includes an indication of failure and may include a reason for failure.

### 8.x.3	Information flows

##### 8.x.3.1	Model retrieval request

Table 8.x.3.1-1 describes information elements for the model retrieval request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.1-1: Model retrieval request

| Information element |  | Status |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Requestor identifier |  | M |  | The identifier of the requestor (e.g., AIMLE client or VAL server). |  |
| VAL client ID |  | O |  | The identifier of the VAL client when the requestor is an AIMLE client. |  |
| Security credentials |  | M |  | The security credentials of the requestor. |  |
| Retrieval filters |  | O |  | The filters used for determining the model to retrieve. |  |
| > Model ID |  | O |  | The identifier of the model. |  |
| > Provider identifier |  | O |  | The identifier of the model provider. |  |
| > Type |  | O |  | The type of model; the type provides information about the model purposes. |  |
| > Input requirements |  | O |  | The description of model input requirements. |  |
| > Output requirements |  | O |  | The description of model output requirements. |  |
| > Size |  | O |  | The model size. |  |

Editor’s Note: The retrieval filters can be refined during the normative phase.

##### 8.x.3.2	Model retrieval response

Table 8.x.3.2-1 describes information elements for the model retrieval response from the AIMLE server.

Table 8.x.3.2-1: Model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the model retrieval request was successful. |
| List of models | O | List of retrieved models. |
| > Model ID | O | The model identifier. |
| > Model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the model file can be retrieved. |
| > Model file (NOTE) | O | The model file containing a retrieved model. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: At least one of these elements is needed. |  |  |

##### 8.x.3.3	Model retrieval subscribe request

Table 8.x.3.3-1 describes information elements for the model retrieval subscription request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.3-1: Model retrieval subscription request

| Information element |  | Status |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Requestor identifier |  | M |  | The identifier of the requestor (e.g., AIMLE client or VAL server). |  |
| VAL client ID |  | O |  | The identifier of the VAL client when the requestor is an AIMLE client. |  |
| Security credentials |  | M |  | The security credentials of the requestor. |  |
| Notification Target Address |  | M |  | Notification target address (e.g. URL) where the notifications should be sent. |  |
| Proposed expiration time |  | O |  | Proposed expiration time for the subscription. |  |
| Retrieval filters |  | O |  | The filters used for determining the model to retrieve. |  |
| > Model ID |  | O |  | The identifier of the model. |  |
| > Provider identifier |  | O |  | The identifier of the model provider. |  |
| > Type |  | O |  | The type of model; the type provides information about the model purposes. |  |
| > Input requirements |  | O |  | The description of model input requirements. |  |
| > Output requirements |  | O |  | The description of model output requirements. |  |
| > Size |  | O |  | The model size. |  |

Editor’s Note: The retrieval filters can be refined during the normative phase.

##### 8.x.3.4	Model retrieval subscription response

Table 8.x.3.4-1 describes information elements for the model retrieval subscription response from the AIMLE server.

Table 8.x.3.4-1: Model retrieval subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the model retrieval request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.x.3.5	Model retrieval notification

Table 8.x.3.5-1 describes information elements for the model retrieval subscription response from the AIMLE server.

Table 8.x.3.5-1: Model retrieval notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| List of models | O | List of retrieved models. |
| > Model ID | O | The model identifier. |
| > Model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the model file can be retrieved. |
| > Model file (NOTE) | O | The model file containing a retrieved model. |

##### 8.x.3.6	Model retrieval subscription update request

Table 8.x.3.6-1 describes information elements for the model retrieval subscription update request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.6-1: Model retrieval subscription update request

| Information element |  | Status |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Requestor identifier |  | M |  | The identifier of the requestor (e.g., AIMLE client or VAL server). |  |
| Security credentials |  | M |  | The security credentials of the requestor. |  |
| Subscription ID |  | M |  | Subscription identifier corresponding to the subscription to be updated. |  |
| Notification Target Address |  | O |  | Notification target address (e.g. URL) where the notifications should be sent. |  |
| Proposed expiration time |  | O |  | Proposed expiration time for the subscription. |  |
| Retrieval filters |  | O |  | The filters used for determining the model to retrieve. |  |
| > Model ID |  | O |  | The identifier of the model. |  |
| > Provider identifier |  | O |  | The identifier of the model provider. |  |
| > Type |  | O |  | The type of model; the type provides information about the model purposes. |  |
| > Input requirements |  | O |  | The description of model input requirements. |  |
| > Output requirements |  | O |  | The description of model output requirements. |  |
| > Size |  | O |  | The model size. |  |

Editor’s Note: The retrieval filters can be refined during the normative phase.

##### 8.x.3.7	Model retrieval subscription update response

Table 8.x.3.7-1 describes information elements for the model retrieval subscription update response from the AIMLE server.

Table 8.x.3.7-1: Model retrieval subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the model retrieval subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.x.3.8	Model retrieval unsubscribe request

Table 8.x.3.8-1 describes information elements for the model retrieval unsubscribe request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.8-1: Model retrieval unsubscribe request

| Information element |  | Status |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Requestor identifier |  | M |  | The identifier of the requestor (e.g., AIMLE client or VAL server). |  |
| Security credentials |  | M |  | The security credentials of the requestor. |  |
| Subscription ID |  | M |  | Subscription identifier corresponding to the subscription to be unsubscribed. |  |

##### 8.x.3.9	Model retrieval unsubscribe response

Table 8.x.3.9-1 describes information elements for the model retrieval unsubscribe response from the AIMLE server.

Table 8.x.3.9-1: Model retrieval unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the model retrieval unsubscribe request was successful. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

* * * End of Changes * * * *